3GPP TSG-SA3 Meeting #119	S3-244790

Orlando, US  11 - 15 November 2024

# Source:	Nokia

# Title:	Updates to Overall Summary of solutions

# Document for:	Approval

Agenda Item:	5.7

# 1	Decision/action requested

Approve the updates to overall summary of solutions added to TR 33.700-29

# 2	References

[1]	3GPP TR 33.700-29 Study on Security Aspects of 5G Satellite Access in the 5G Architecture phase 3

[2]          3GPP TR 23.700-29 Study on integration of satellite components in the 5G architecture

# 3	Rationale

The contribution proposes an update to overall summary of all solutions including new solutions included in SA3#118. This update includes EN removal and other editorial changes. Also, the last column is proposed to be deleted, and whether any LS is required or not can be discussed and decided during the meeting.

# 4	Detailed proposals

# * * * * First change* * * *

# 7	Conclusions

## 7.1	Overall Summary of Solutions

| Legend |  |
| --- | --- |
| X | Key Issue #1 Split MME S&F |
| Y | Key Issue #1 Full CN onboard S&F |
| XX | Key Issue #2 Split MME S&F |

| Solution number | No / Minimal Security Changes | Security Architecture Change | Procedural Adaptations | Novel solutions | DDoS Solutions | 5G Only solutions | Summary of key concept | Group of solutions to derive common principles for conclusions | (COLUMN TO BE DELETED) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | X |  | X |  |  | Inverse AKA | Solutions 1, 7, 8, 9, 11, 15, 16, 17, 21, 25 |  |
| 2 | Y |  | Y |  |  |  | IOPS Security for S&F | Solutions 2, 3, 5, 12, 13 |  |
| 3 | Y |  | Y |  |  |  | IOPS Security for S&F | Solutions 2, 3, 5, 12, 13 |  |
| 4 |  |  |  | X |  | X | Bundling the authentication round trip with the NAS SMC |  |  |
| 5 | Y |  |  |  |  | Y | UDM onboard satellite | Solutions 2, 3, 5, 12, 13 |  |
| 6 | X |  | X |  |  |  | NAS Timer updates | Solutions 6, 17 |  |
| 7 |  | X |  | X |  | X | Inverse AKA for subsequent authentication | Solutions 1, 7, 8, 9, 11, 15, 16, 17, 21, 25 |  |
| 8 |  |  | X | X | X |  | Addresses DDoS attack on UE by False Satellite. eNB and UE generate and verify digital signatures for this. Requires a set of credentials for ECCSI is provisioned to the UE and Satellite. | Solutions 1, 7, 8, 9, 11, 15, 16, 17, 21, 25 |  |
| 9 |  |  | X |  | X |  | Temporarily storing the UE's IMSI and issuing an Attach Reject message with a Cause value and a Timer. | Solutions 1, 7, 8, 9, 11, 15, 16, 17, 21, 25 |  |
| 10 |  |  | X | X |  |  | UE subscription information is stored in a UE authentication token. UE provides this when it connects to satellite. Token protection keys need to be pre-configured in the satellites. | Solutions 10, 11 |  |
| 11 |  |  | X | X |  |  | Similar to solution #10. After UE successfully accesses the satellite for the first time, it can perform authentication and data transmission processes in any subsequent connection. How can the UE access satellite in S&F mode for the first time is unclear. | Solutions 1, 7, 8, 9, 11, 15, 16, 17, 21, 25 |  |
| 12 | Y |  | Y |  |  |  | IOPS based solution. The HSS-NT needs to fetch the unstored user security credentials from the ground HSS when feeder link is available. | Solutions 2, 3, 5, 12, 13 |  |
| 13 | Y |  | Y |  |  |  | With HSS onboard, re-use legacy security procedures. Disadvantage of Subscriber credentials are provisioned in several satellites HSSs. Also, the key used to generate authentication vectors should be stored on the satellite, which may not be the resources of the home operator. | Solutions 2, 3, 5, 12, 13 |  |
| 14 |  | X |  | X |  |  | S&F Operation function and OTP based security solution |  |  |
| 15 | X |  |  | X | X |  | Protection against unauthorized UEs launching DDoS attacks. MME-T checks UE authorization based on subscription information from HSS before requesting authentication vectors. This solution does not address DoS attack when only service link is available and NAS/AS security context is not established. | Solutions 1, 7, 8, 9, 11, 15, 16, 17, 21, 25 |  |
| 16 | X |  |  | X | X |  | UE authorization during S&F MO transmission. Authorization information included in UE context. How can the UE access satellite in S&F mode for the first time (i.e., unprovisioned with authorization credentials) is unclear.. This solution only addresses DoS attack after NAS/AS security context is established. | Solutions 1, 7, 8, 9, 11, 15, 16, 17, 21, 25 |  |
| 17 | X |  | X |  |  |  | MME leverages pre-provisioned orbit information to determine the back-off timer and provide the back-off timer to the UE. Only compliant UEs will heed the backoff timer. | Solutions 1, 7, 8, 9, 11, 15, 16, 17, 21, 25 |  |
| 18 | X |  | X |  |  |  | Re-use legacy UP Integrity procedures and discard fake data from potential attackers. |  |  |
| 19 | XX |  | XX |  |  |  | Temporary UE IDs used to generate interim GUTI |  |  |
| 20 | XX |  | XX |  |  |  | Alt1: Short Validity Period for interim GUTI Alt2: Not to use interim GUTI | Solutions 20, 30 |  |
| 21 |  |  |  | X | X |  | Backoff timer and throttling are effective means to remediate (D)DOS attacks post-authentication. However, the backooff timer is applicable to compliant UEs only and throttling can work only for authenticated UEs. Solution proposes puzzle solving to slow down the rate of Attach Requests from un-authenticated UEs. This solution does not require pre-provisioned specific satellite credentials into UEs. This solution does not address DoS attack on UE by false information in Attach Reject message. | Solutions 1, 7, 8, 9, 11, 15, 16, 17, 21, 25 |  |
| 22 | X |  | X |  |  |  | Based on satellite constellation to allow UEs to connect via one of the satellites from the group. Relies on ISL connections, which is assume to be excluded in Rel-19 |  |  |
| 23 |  |  | X | X |  |  | Protect the RRC connection procedure via using NAS parameters from core network to ensure RAN is talking to genuine UE. |  |  |
| 24 | X |  | X |  |  | X | Pre-fetch authentication information into next satellite that will fly over UE's location. pre-fetch authentication information into next satellite that will fly over UE's location. |  |  |
| 25 |  |  | X | X | X |  | before the AKA procedure between the UE and the network is completed, the satellite and the UE check the authenticity of each other using the symmetric cryptosystem, which prevents the threats of DoS attack and false satellite attack. This solution requires satellite-specific pre-provisioned credentials. It is unclear how UEs without such credentials can attach. | Solutions 1, 7, 8, 9, 11, 15, 16, 17, 21, 25 |  |
| 26 | XX |  | XX |  |  |  | UE pre-configured with private key and certificate for partial accept message before authentication |  |  |
| 27 | XX |  | XX |  | X |  | Preconfigured group keys to secure communication related to attach request. Once the group key from one UE is leaked, all UEs that have the key are affected. |  |  |
| 28 |  | X |  | X |  |  | The purpose of this solution is FFS. |  |  |
| 29 | Y |  | Y |  |  |  | Reuses the existing security procedures for full CN onboard. |  |  |
| 30 | XX |  | XX |  |  |  | Dynamic interim GUTI with short validity period. |  |  |
| 31 | X, Y |  | X, Y |  | X, Y |  | An upper limit of wait timer OR a valid wait timer is sent to UE. Also, a list of valid Satellite IDs is sent to UE. This can protect UEs from DDoS attacks which intend to provide a very large wait timer. However, the UE is still vulnerable to DoS attack caused by forged information in the Reject message at initial attach procedure due to the unprotected Reject message. |  |  |
| 32 |  |  |  | X | X |  | Only S&F related DDoS attack which can lead to e.g., buffer overflow during S&F is the focus of this solution. Pre-provisioning of shared secrets which enables puzzle solving with valid UEs (step 0) and broadcasting puzzles (step 5), would require coordination with RAN. This solution ensures that only valid UEs capable of solving puzzles can perform registrations in a regular manner. Unprovisioned UEs cannot solve the puzzles and hence they cannot launch DDoS by filling up S&F buffers. | Solution 21, 32 |  |
| 33 |  |  |  | X | X |  | Only S&F related DDoS attacks which can lead to e.g. buffer overflow during S&F are the focus of this solution. Pre-provisioning of shared secrets which enables puzzle solving with valid UEs (steps 0 and 1) and broadcasting puzzles (step 7(4)), would require coordination with RAN and SA WG2. This solution does not require pre-provisioning the UE with satellite-specific parameters. | Solutions 8, 21, 33 |  |
| 34 |  |  |  | X | X |  | Only S&F related DDoS attack which can lead to buffer overflow during S&F is the focus of this solution. S&F Policy provisioning (step 0) and offering puzzles (step 6), would require coordination with RAN. This solution does not require pre-provisioning the UE with satellite-specific parameters while reducing the rate of DDOS attacks. On the other hand, this solution allows the authorization of the UEs with pre-provisioned satellite-specific parameters when available. This solution also enables skipping DDOS remediation when the Feeder Link is available. | Solutions 9, 21, 34, 35 |  |
| 35 |  |  |  | X | X |  | Only S&F related DDoS attacks which can lead to buffer overflow during S&F are the focus of this solution. reducing the rate of DDOS attacks. On the other hand, this solution allows the authorization of the UEs with pre-provisioned satellite-specific parameters when available. This solution also enables skipping DDOS remediation when the Feeder Link is available. | Solutions 15, 21, 34, 35 |  |
| 36 |  | X | X | X |  |  | To provide authentication capabilities when feeder link is not available, one possible approach is to deliver Authentication vector to the satellite, which enables the AKA procedure between the UE and the satellite. The solution proposes to perform authentication and NAS SMC between UE and MME-onboard. New key derivations are proposed in this solution. This solution requires the MME-ground to derive new key (i.e. KASME* derivation based on KASME), and key transfer from MME-ground to MME-onboard. |  |  |
| 37 |  | X | X | X |  |  | The MME-T needs to update the key KASME* and distribute to MME-NT. The UE and the MME-NT protects uplink and downlink NAS messages vis using NAS keys derived from KASME*. Each MME-NT can use an isolated set of NAS keys to protect NAS messages. However, this solution also requires sharing keys between MME entities. |  |  |

NOTE: Conclusions for full CN onboard and split MME deployment scenarios, and for S&F related DDoS attacks are captured in subsequent clauses. This table captures a short summary of all solutions and can be used as a quick reference and intends to improve the readability of this TR.

# * * * * End of changes * * * *