3GPP TSG-SA3 Meeting #119	S3-245122

# Orlando, USA  11 - 15 November 2024

# Source:	Nokia, Nokia Shanghai Bell

# Title:	Conclusion for KI#1 - Third party specific user identities

# Document for:	Approval

Agenda Item:	5.2

# 1	Decision/action requested

It is required to approve the conclusion proposed for KI#1 in TR 33.790

# 2	References

[1]	3GPP TR 23.700-77: "Study on system architecture for next generation real time communication services; Phase 2"

[2]	3GPP TR 33.790: "Study on the security support for the next generation real time communication services phase 2"

[3]	S2-2411009:  CR for KI#4 to Support of third party user identity information in IMS

# 3	Rationale

KI#4 on "Extensible IMS framework to support authorization and authentication of third-party identities in IMS sessions" was concluded in TR 23.700-77, and CR to TS 23.228 for KI#4 to support third party user identity information in IMS was agreed.

The CR defined overall system architecture and functional entities to support authorization, authentication, and verification of third party user identity information in IMS (Annex XX.2) as below:

The CR also described procedures to support authorization and verification of third party user identity information in IMS (Annex XX.3).

Potential security requirements of Key issue #1 (Third party specific user identities) are defined in 5.1.3 of TR 33.790 as follows:

-  The IMS system shall be able to coordinate with the third party to verify and authorize the third-party specific user identities.

-  The IMS network shall be able to support the integrity protection of the third-party specific user identities on the originating side and terminating side.

According to the agreed CR to TS 23.228, the first requirement on the third-party specific user identities verification and authorization is supported in the System architecture to support authorization, authentication, and verification of third party user identity information in IMS, see Annex XX of TS 23.228.

This contribution proposes conclusion for KI#1 of TR 33.790 to focus on the second requirement on supporting the integrity protection of the third-party specific user identities on the originating side and terminating side, and to align with the architecture defined in TS 23.228.

# 4	Detailed proposal

******************START of CHANGES****************************************

# 7	Conclusions

Editor’s Note: This clause contains the agreed conclusions that will form the basis for any normative work.

## 7.1	KI#1: Third party specific user identities.

### 7.1.1 Evaluation

According to threat analysis, the potential security requirements for this key issue are defined in 5.1.3 of the present TR, which include supporting verification and authorization of the third-party specific user identities and supporting the integrity protection of the third-party specific user identities on the originating side and terminating side.

KI#4 on "Extensible IMS framework to support authorization and authentication of third-party identities in IMS sessions" was concluded in TR 23.700-77, and CR to TS 23.228 for KI#4 to support of third party user identity information in IMS was agreed.

According to the agreed CR to TS 23.228, the first requirement on the third-party specific user identities verification and authorization is supported in the System architecture and feature description in Annex XX of TS 23.228, e.g.:

Based on the user profile retrieved from HSS, IMS AS authorizes the usage of third-party user identity information in a SIP INVITE request that originated from a UE or IP-PBX.

If originating UE or IP-PBX is authorized to use third party user identity information in the SIP INVITE request, and based on operator policies, S-CSCF, IMS AS or IBCF invoke signing of the provided third party user identity information with a signing AS.

Based on operator policies, the S-CSCF, IMS AS or IBCF in the originating IMS network will invoke signing of the RCD information or RCD URL included in the outgoing SIP INVITE request with a signing AS.

Based on operator policies, the S-CSCF, IMS AS or IBCF in the terminating IMS network will invoke signature verification of the RCD information or RCD URL included in the incoming SIP INVITE request with a verification AS. The S-CSCF, IMS AS or IBCF in the terminating IMS network may reject the call when the signature verification is unsuccessful.

The second security requirement to support  the integrity protection of the third-party specific user identities on the originating side and terminating side is not fully addressed by architecture and feature description in Annex XX of TS 23.228, especially when RCD information includes URL elements or RCD URL is exchanged between the originating and terminating IMS.

The conclusion should focus on the second requirement on supporting the integrity protection of the third-party specific user identities on the originating side and terminating side, and to align with the architecture defined in TS 23.228.

### 7.1.2 Conclusion

The following principles from the security point of view for the KI#1:

-  The architecture and description defined in TS 23.228 for authorization and verification of third party user identity information in IMS should be referred to address security requirements on verification and authorization of the third-party specific user identities.

- Security requirements for authorization and verification of third party user identity information in IMS should be captured in TS 33.328.

-  Solution to support integrity protection of the third-party specific user identities on the originating side and terminating side should be specified in normative phase in TS 33.328.

-  The security solution shall be aligned with architecture and interfaces defined in TS 23.228 for authorization and verification of third party user identity information in IMS.

******************END of CHANGES****************************************