3GPP SA4 #128	S4-241110

Jeju, Korea, 20 May 2024

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.822 | CR | pseudo | rev | - | Current version: | 0.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_5G_RTP_Ph2] Benefits of using AL-FEC for real-time communication in cellular networks |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5G_RTP_Ph2 |  |  |  |  |  | Date: |  |  | 05/20/2024 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | One aspect of Key issue #3: Enhancements for application-layer FEC support There are doubts on the benefit of using AL-FEC for real-time communication in cellur networks. We need to explain away or confirm the doubts. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Explanations on the benefits of using AL-FEC |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Whatever solutions out of Key Issue #3 will be based on a shaky foundation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

.

# Proposed changes

# * * * * 1st change * * * *

## 5.3.x	Benefits of using AL-FEC for real-time communication in cellular networks

3GPP 5G has defined 5QI with QoS characteristics for various intended applications including real-time communication, as shown in TS 23.501:

| 5QI Value | Resource Type | Default Priority Level | Packet Delay Budget (NOTE 3) | Packet Error Rate | Default Maximum Data Burst Volume (NOTE 2) | Default Averaging Window | Example Services |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | GBR | 30 | 50 ms (NOTE 11, NOTE 13) | 10-3 | N/A | 2000 ms | Real Time Gaming, V2X messages (see TS 23.287 [121]). Electricity distribution – medium voltage, Process automation monitoring |
| 80 | Non-GBR | 68 | 10 ms (NOTE 5, NOTE 10) | 10-6 | N/A | N/A | Low Latency eMBB applications Augmented Reality |
| 89 | Delay-critical GBR | 25 | 15 ms (NOTE 4) | 10-4 | 17000 bytes | 2000 ms | Visual content for cloud/edge/split rendering (see TS 22.261 [2]) |

For some 5QI’s, meeting the required packet error rate (PER) does not result in desired user experience. For example, for 5QI=3, the PER is 10-3. The PER looks small, but a PDU loss impacts the reconstruction of the whole ADU and can severely degrade the user experience. Suppose the frame rate is 60 frames/sec and each frame takes up 20 packets. This means that on average there is a packet loss in less than a second. This could cause video artifacts or freeze and severely degrade the user experience. Due to the low-latency requirement, it is typically not feasible to retransmit the lost packet without violating the latency requirement. However, with AL-FEC with a moderate amount of redundancy, such packet losses can be recovered.

Observation 1: AL-FEC is essential to the user experience for certain 5QI’s intended for real-time applications.

Another reason for needing AL-FEC is the RAN implementations, which typically has an instaneous BLER (iBLER) of 10% for high spectral efficiency. That requires a large number of HARQ retransmissions, resulting in large delays. If AL-FEC is used, the need for HARQ retransmission is greatly reduced. This is illustrated in the simulation study below.

Scenario: TDD with subframe format DDDSU, 30kHz SCS, HARQ turnaround time about 5ms, 100MHz bandwidth, 60fps, average SNR 5dB, and MDS AL-FEC code.

Table 5.3-1 Delay without and with AL-FEC

| Scheme | Redundancy ratio | Latency (ms) |  |
| --- | --- | --- | --- |
|  |  | 99.9 percentile | 99 percentile |
| No AL-FEC | 0 | 55 | 43.5 |
| With AL-FEC | 30% | 15 | 14 |

We see from the table that AL-FEC reduces the 99-percentile delay from 43.5ms to 14 ms and reduces the 99.9-percentile delay from 55ms to 15ms.

Observation 2: AL-FEC can greatly reduce the delay for practical RAN implementations.

Based on the observtions, we have the following:

Conclusion: AL-FEC is important to the media performance for real-time applications in cellular networks.

# * * * * End of 1st change * * * *