3GPP TSG SA WG4#130	S4-241835

Orlando, Florida, USA, 18 - 22 November 2024

Agenda item: 	10.6

Source: 	Huawei, HiSilicon

Title: 	[FS_5G_RTP_Ph2] Additional statistics for RTP Senders and Receivers in Solution #13, KI#12 for dynamic traffic characteristics

Document for	Agreement

# reference

[1]   TR 26.822 Study on 5G Real-time Transport Protocol Configurations, Phase 2

# Introduction

In solution #13 in TR 26.822 clause 6.13 experiments are described involving RTP Senders based on common implementations, the following 3 implementations are tested.

- Hangout/WebRTC case in a browser (edge/chrome) on windows/Ubuntu PC

- GStreamer sender/receiver implementation in a windows/linux PC

- Server implementations such as MTX media and Wowza streaming server

The current experimental results illustrate graphs showing the corresponding packet data traffic.

This paper presents additional statistics and the methodology of collecing the statistics

# Dynamic Traffic Characterstics and Statistics

In Real Time Communication the communication is usually duplex.

For the burst statistics traffic from host A to Host B (a) and Host B to Host A (b) are considered separately.

Therefore each packet trace will need to be analyzed twice, once for traffic from A to B (a) and vice versa (b).

NOTE: Additional hosts can take part but in this study only 2 hosts were considered so far.

NOTE: In some cases the return channel has traffic (duplex scenarios), in case of unidirectional flow there is no need to monitor and report traffic

The following statistics for dynamic traffic characteristics are retrieved from the packet traces.

Table 1	Metrics and statistics considered for dynamic traffic characteristics

| Code | Metric | meaning | Additional notes | Statistics |
| --- | --- | --- | --- | --- |
| Bt | Burst Time (Bt) | The time of the burst (corresponding to the first packet), measured in seconds | Bursts are measured as instantaneous transmission of 2 or more packets | Deterministic |
| Ibt | Inter Burst Time (Ibt) | The average duration between 2 consecutive bursts (burst), measured in seconds | This does not considerer single ‘lone’ packets transmitted between the bursts | Average and Standard Deviation |
| Bd | Burst Duration (Bd) | The duration between start and end of a burst, measured in seconds | Measures the duration of the burst (in a maximum timeslot of 2 milliseconds ) | Average and Standard Deviation |
| Bs | Burst Size (Bs) | The Size of the Data Burst measured in bytes | Measures the size of the entire burst (in a maximum timeslot of 2 milliseconds) | Average and Standard Deviation |
| Ppb | Packets per burst (Ppb) | Number of packets in each burst | Only bursts of 2 packets or more are considered | Average and Standard deviation |
| #bp | Total Burst packets (#bp) | Number of packets in a burst | This is the number of packets in the trace that is part of a burst | Deterministic |
| #lp | Total Lone packets (#lp) | Number of packets not in a burst | This is the number of packets in the trace that is part not part of a burst | Deterministic |
| #bb | Total Burst Bytes (#bb) | Number of bytes that are part of a burst | This is the number of bytes that is part of a burst | Deterministic |
| #lb | Total Lone Bytes (#lb) | Number of bytes that are not part of a burst | This is the number of bytes in lone packets | Deterministic |
| %lp | Percentage of lone packets (%lp) | Ratio of lone packets to all packets |  | Percentage |
| %bp | Percentage of burst packets (%bp) | Ratio of burst packets to all packets |  | Percentage |
| %lb | Percentage of lone bytes (%lb) | Ratio of lone packet bytes to all bytes |  | Percentage |
| %bb | Percentage of burst bytes (%bb) | Ratio of burst packet bytes to all bytes |  | Percentage |

For brevity we can leave out the deterministic metrics bt, tbs, #bp, #lp, #bb and #lb that depend heavily on the length of the trace. Instead we focus on the ratio between lone packets/bytes and burst packets/bytes and other metrics that are independent of the length of the trace.

Table 2  Scenario descriptions and coding in solution #13

| Scenario code |  |
| --- | --- |
| w_h | Windows hangout without network throttling duplex case (Figure 6.13.2.2.2-1) [1] |
| u_h | Ubuntu Hangout with network throttling simplex case (Figure 6.13.2.2.2-2)[1] |
| g_v_a | Gstreamer video only measured on machine A (Figure 6.13.2.3.2-2) [1] |
| g_v_b | Gstreamer video only measured on machine B (Figure 6.13.2.3.2-3) [1] |
| g_va_a | Gstreamer pcm audio + video at receiver (Figure 6.13.2.3.3-1) [1] |
| g_va_b | Gstreamer pcm audio + video at sender (Figure 6.13.2.3.3-2)[1] |
| w_s | Wowza at server measured at server (Figure 6.13.2.4.2-2) [1] |
| w_r | Wowza stream measured at player receiver (Figure 6.13.2.4.2-3) [1] |
| m_rv | MTX server measured vlc player receiver (Figure 6.13.2.4.2-5) [1] |
| m_sav | MTX server video + PCM (Figure 6.13.2.4.2-7) [1] monitored at server |
| m_rav | MTX server video + PCM (Figure 6.13.2.4.2-7) [1] monitored at vlc client |

Table 3 provides the statistical results for each of the experiments in different scenarios.

Table 3 Statistical results of experiments in different scenarios in scenario #13

| Scenario (direction) | ibt (mean) | Ibt (stdev) | Bd (mean) | Bs (mean) | Ppb (mean) | %lp (%) | %bp (%) | %lb (%) | %bb (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| w_h (a) | 0.0352 | 0.01437 | 0.0002 | 3463 | 3.55 | 23.2 | 76.7 | 6.6 | 93.4 |
| w_h (b) | 0.01592 | 0.019 | 0.00016 | 3756 | 3.3 | 19.2 | 80.8 | 17.4 | 82.6 |
| u_h (a) | 0.03132 | 0.00873 | 0.00017 | 13895 | 12,3 | 40.2 | 59.8 | 22.6 | 77.4 |
| u_h (b) | 0.42321 | 0.37918 | 0.00007 | 213.92 | 2 | 72.2 | 27.8 | 73.8 | 26.2 |
| g_v_a | 0.01642 | 0.01175 | 0.00052 | 2864 | 7.73 | 2.4 | 97.6 | 0.7 | 99.3 |
| g_v_b | 0.01796 | 0.00967 | 0.00029 | 3109 | 8.6 | 0.2 | 99.8 | 0.2 | 99.8 |
| g_va_a | 0.01252 | 0.0114 | 0.00071 | 4144 | 6.6 | 14.2 | 85.8 | 20.1 | 79.9 |
| g_va_b | 0.01414 | 0.00955 | 0.00033 | 4472 | 7.5 | 14.8 | 85.2 | 24.7 | 75.3 |
| w_s (a) | 0.08732 | 0.03483 | 0.00079 | 24822 | 42.3 | 0.2 | 99.8 | 0.0 | 100.0 |
| w_r (a) | 0.03778 | 0.04627 | 0.00054 | 10671 | 18.24 | 0.7 | 99.3 | 0.1 | 99.9 |
| m_rv | 0.02749 | 0.0404 | 0.00084 | 6630 | 6.38 | 2.5 | 97.5 | 0.5 | 99.5 |
| m_sav (a) | 0.04777 | 0.03838 | 0.00056 | 11906 | 12.18 | 12.6 | 87.4 | 3.3 | 96.7 |
| m_rav (a) | 0.02573 | 0.03412 | 0.00086 | 6692 | 6.8 | 12.2 | 87.8 | 3.2 | 96.5 |

Some observations of the statistics

- A large percentage of the traffic and packets is part of a burst, highlighting the relevance of burst handling

- Burst durations are shorter than a millisecond

- Results do not say anything on predictability, this is implementation dependent

- PCM audio is tested, transmitted in lone packets, this increases the share of lone bytes ( compared to more efficiently compressed audio)

# Methodology

The wireshark traces packet dissection exports are stored in comma separated files.

The script to analyse the traces is included for completeness with the contribution.

The direction a to b and b to a are analysed separately.

Bursts are detected on maximum intervals of 2 milliseconds.

# Proposal

- Agree on considering these statistics for analyzing dynamic traffic characteristics

The statistics will be added in a separate clause in separate pseudo change request.