3GPP TSG-SA4 Meeting #130	S4-241876

# Orlando,  FL, United States, 19 - 22 Nov 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.841 | CR | pseudo | rev | - | Current version: | 0.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_MeMe] Spatial Video Messaging Format Updates |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Germany GmbH |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_MeMe |  |  |  |  |  | Date: |  |  | 2024-11-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 26.265 and TR 26.956 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is not yet completed, but will be extended with the detailed definition. It is also expected that test material will be provided for the operation points. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# =====  CHANGE =====

## 5.5	Key Topic #5: Additional media experiences

### 5.5.1	Description

In 3GPP, advanced media experiences are added to different services, in particular immersive media, new image formats, interactive content, etc. The proper integration of new media experiences should be studied.

A specific format of interest is the ability to share 3D video with audio. Existing and emerging capturing systems on mobile devices are used to capture video formats that go beyond regular 2D video formats. The captured scene is recorded and shared with another UE, for example another mobile device with a 2D screen, a mobile device with an Autostereoscopic Display, a VR or MR HMD, or it consumed on AR glass. While sharing may be done by different means (upload and download, stream, communication), the focus in the context of this document is message exchange.

The scenario is shown in figure 5.5.1. On the upper part, two examples of UE camera setup are shown that allow to generate a Beyond 2D (B2D) message on the device. The input to the encoder and packager is the result of a proprietary capturing system that converts the capture to a well-defined B2D format. The B2D message is shared with the network that stores B2D messages in a well-defined format. On the lower end, a regular 2D UE may produce content and upload the content to a network server. The server processes the data and produces again a B2D format that can be encoded and packaged to conform to a well-defined B2D formats. In yet another scenario, some professional content is produced and exported in a well-defined B2D format, that is then encoded and packaged and made available as B2D message.

Any of these messages can then be accessed/downloaded by B2D UEs that can unpack and decode in the included data and provide the B2D formatted content to the proprietary rendering systems.

Figure 5.5.1: Beyond 2D Messaging

Note that video formats are expected to be defined in TS 26.265 [26265] and detailed discussions on the properties and benefits of such representation formats are provided in 3GPP TR 26.956 [26956].

While it is understood that there is currently no harmonized set of formats for production and device playback, the attempt towards improved format compatibility and is the most relevant driver for global standards. A baseline format for global B2D message formats is stereoscopic video. Stereoscopic video presents one image to the user’s left eye and another image (typically correlated) to the user’s right eye to produce the stereopsis effect, defined as "the perception of depth produced by the reception in the brain of visual stimuli from both eyes in combination; binocular vision." [AMVHEVC].

The initial focus on the definition of stereoscopic formats in TS 26.265 is on providing a consistent description of stereoscopic video based on professional production, for example Hollywood movies. Signal representations for Left and Right eyes comply to regular image formats such as Rec ITU-R BT.709 or Rec ITU-T BT.2100 and any necessary 3D-specific metadata is incorporated with the data. Hence, for stereoscopic video, two synchronized video signals are available, each with identical parameters. A summary of the parameters of a 3GPP Stereoscopic 3D TV format is provided in Table 4.4.2-1 of TS 26.265.

In the context of this specification, the focus is on user generated content from modern devices that allow to capture beyond 2D video formats.

Stereoscopic video for user-generated content may use projections to left and right eye as follows [SPATIAL]:

-	rectangular, traditional 3D video

-	either of the two may be extended with additional depth data, also referred to as video contour maps [CONTOUR].

-	extensions with additional alpha maps.

A detailed study for suitable content formats for UCG in messaging services is addressed in TR 26.956.

3GPP decided based on the study documented in 3GPP TR 26.966 [26966] to support stereo MV-HEVC and the work in currently ongoing to add this format to 3GPP TS 26.265 [26265]. In addition, 3GPP has adopted also audio codecs that can augment 3D video formats including IVAS and MPEG-H audio, already included in 3GPP TS 26.143 [26143].

This issue addresses the analysis and requirements to support 3D video in advanced messaging based on MV-HEVC as defined in 3GPP TS 26.265 [26265].

### 5.5.2	Gap Analysis and Requirements

In order to identify the gaps to add 3D video to advanced messaging, a high-level flow chart is provided in Figure 5.5.2-1 and also elaborated further below based on the architecture shown in Figure 4.1-1 and the player model shown in Figure 4.1-2. The call flow is based on the generic description in clause 4.1.

Figure 5.5.2-1 High-Level Call flow for 3D video messaging

The following high-level call flow is executed to address the 3D video messaging and gaps are indicated in bold:

1) 	A Message Service Sender instructs and MMBP generator to generate a message with 3D format where the format is defined through a set of parameters handed over on an MMBP-GEN-API.

2) 	The MMBP generator creates an MMBP that includes 3D video formats encoded with MV-HEVC, and adds relevant static and dynamic metadata.

3) 	The generated file with associated relevant static and dynamic metadata is provided in a container message that is uploaded to a messaging server, together with a signaling in the media type and associated parameters. The media signaling may include that the file is suitable for different clients, for example it includes a 2D and a Beyond 2D experience.

4)  A messaging service client is informed about the availability of a message in this format using the media type signaling together with appropriate sub-parameters.

5)	A messaging service client uses the MMBP-PLAY-API to query the MMBP player with it can process, decode and render the message, possibly in 2D legacy or as 3D video.

6)	If the MMBP player indicates that it supports the capability, the messaging service clients selects and downloads the message and hands it to the MMBP player for playback.

7)	The MMBP player processes, decodes and renders the 3D message based on its decoding and rendering capabilities together with other media types.

Based on the analysis in the call flow, the following aspects are missing:

a)	the definition of the 3D formats that are supported

b)	a generator for a 3D video message including API parameters

c)	a 3D video messaging format including MV-HEVC with relevant metadata, including depth, alpha. projection formats, disparity information – for details refer to TR 26.956 [26956].

d)	the signaling of the media type and the associated parameters, typicaly a MIME type and the codecs parameter.

e)	an API to query the MMBP player that it can process, decoder and render the message

f)	an MMBP player capability that processes, decodes and renders the message.

g)	an MMBP player capability renders the video message together withother media components in a synchronized and spatially aligned matter.

### 5.5.3	Potential Solutions

#### 5.5.3.1	Overview

To address the issues a) – g), a set of solutions are discussed in the following clause.

#### 5.5.3.2	3D Video Formats

For the 3D video formats according to a), the parameters are defined in TR 26.956 [26956], including different type of Beyond 2D video using projections to left and right eye:

-	rectangular, traditional 3D video

-	either of the two may be extended with additional depth data, also referred to as video contour maps [CONTOUR].

-	extensions with additional alpha maps.

In addition, the detailed signal properties of the video each eye needs to be defined:

-	Sample aspect ratio for each eye

-	Picture aspect ratio for each eye.

- 	Resolutions per eye

- 	Framerates for each eye are

-	Signal characteristics, Chroma subsampling, bit depth, colour primaries, transfer characteristics, matrix coefficients

-	Projection parameters: rectilinear, fisheye, equirectangular, Field-of-view and restricted coverage.

-	Additional metadata such as hero eye, camera parameters, disparity adjustments, disparity/depth map, camera intrinsics, line times,

-	hero eye: A value that indicates which eye is the primary eye when rendering in 2D.

-	camera parameters:

-	distance between the camera lens centers

-	disparity adjustment:

-	horizontal disparity adjustment, a value that indicates a relative shift of the left and right images, which changes the zero parallax plane.

NOTE: The parameters may be aligned with TS 26.118 [B]

-	Camera intrinsics

-	Line time (per camera)

-	Examples: https://github.com/MPEGGroup/FileFormatConformance/tree/m62054_exintrinsics/data/file_features/under_consideration

-	Disparity/depth map: 10bit, same resolution as source content, monochrome

Editor’s Note: a more detailed subset may be selected or inherited from TS 26.265 [26265] and TR 26.956 [26956].

#### 5.5.3.3	3D Video Encoding and Decoding capabilities

To address issue b) as well as partially e) and f), video encoding and decoding capabilities, as video operation points need to be defined. A candidate solution will be available in 3GPP TS 26.265 [26265].

The capabilities are expected to include abstract APIs to provide encoding parameters.

Editor’s note:	It is expected that TS 26.265 defines encoding and decoding capabilities for 3D video.

#### 5.5.3.4	3D Video Systems Operation Point

To address the integration of the video elementary stream into a container message, together with the metadata, a comprehensive 3D track format is required.

ISO/IEC 14496-12:2022 (ISOBMFF) defines the multiplexed metadata track format ('mebx'), which is capable of carrying multiple metadata items over a time range in a single track.

Additional metadata signaling for 3D video operation points are expected to be completed in 3GPP TS 26.265 [26265].

Editor’s note:	It is expected that TS 26.265 defineds a track format for 3D video.

#### 5.5.3.5	3D Video Media Type Signaling

One of the possibilities to carry MV-HEVCvideo in mp4 is by using the 'hvc1' or 'hev1' sample entry type as specified in clause 9 of ISO/IEC 14496-15 in a backward compatible manner. Some existing products in the market are using this concept to carry stereoscopic content and alpha using the Multiview extensions of the HEVC standard as L-HEVC in mp4. Such profiles with existing support in the mobile ecosystem are expected to be defined in TS 26.265 [26265]. However, when constructing the MIME types 'codecs' parameter, according to Annex E of ISO/IEC 14496-15, it does not provide the necessary signalling for all layers. Even if additional information can be specified with other MIME type parameters, these may not be processed by certain APIs. For example, the W3C API accepts a MIME type with no extra MIME parameters except for codecs.

Furthermore, the current signalling inside the codecs string does not expose other important information such as the types of auxiliary information related to rendering the stream. Given these challenges, it is important that a standardized solution be developed to enhance interoperability, accuracy, and efficiency of multi-layer video stream handling.The following high-level requirements for such a solution have been identified:

-	Comprehensive Layer Signalling: Enable the signalling of multiple video layers to be used for 3GPP-based services, including DASH MPDs (see TS 26.511), capability checks in the context of 5G Media Streaming (see TS 26.511), as well as for Media Messaging Services (see TS 26.143). This should include but not be limited to the number of layers, types of each layer, their inter-dependencies, etc.

-	Backward Compatibility: Ensure that the solution maintains backward compatibility, allowing existing players and systems to continue functioning without modifications, while enabling enhanced capabilities for updated systems.

-	Adaptability and Extensibility: Design the solution to be adaptable for future extensions and new types of layers or enhancements without requiring significant overhauls. Consider making the signalling codec agnostic.

Editor’s note:	It is expected that MPEG provides signaling on this matter to be added here.

#### 5.5.3.6	System integration

The 3D video needs to be synchronized and spatially aligned with other media types.

Time synchronization is provided by using the ISO BMFF/MP4 file format.

Editor’s note:	It is expected that the coordinate system from TS 26.118 can be re-used.

### 5.5.4	Summary and Conclusions

Editor’s note:	It is expected that all functionalities are available to support 3D video and general media experiences in a future messaging service.