3GPP TSG-SA4 Meeting #130	S4-241882

# Orlando, United States, 18  - 22 Nov 2024	revision of S4aI240179

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.802 | CR | 0002 | rev | 7 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_AMD] MBS User Service and Delivery Protocols for eMBMS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Germany |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_AMD |  |  |  |  |  | Date: |  |  | 2024-11-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The MBS User Service architecture and protocol follows the modern design philosophies of the 5G System with separation of user services from transport, a service-based architecture and RESTful APIs. At the same time, eMBMS and enTV as used for LTE-based 5G Broadcast support a transparent delivery mode. While interworking in between MBMS and MBS is addressed in TS 23.247, interworking between these two systems at the User Service level is not addressed. In order for MBMS and LTE-based 5G broadcast to leverage MBS User Service technologies, a study is warranted to identify the gaps to fully support this functionality. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addresses the work item objectives for this key issue - Documents the key issue in more detail, in particular how they relate to the 3GPP Media Delivery architecture and/or the MBS User Service architecture - Studies collaboration scenarios between the Application Service Provider and the 5G System and for each of the key topics. - Based on existing architectures, provides one or more deployment architectures that address the key topics and the collaboration models. - Maps the key topics to basic functions and develop high-level call flows. - Identifies the issues that need to be solved. - Provides candidate solutions including call flows, protocols and APIs for each of the identified issues. Identifies gaps and recommend potential normative work for stage-2 and stage-3, including which existing specifications would be impacted and/or if any new specifications would preferably be developed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.10 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TR 26.802 CR 0001 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Updated scope, references and abbreviations are in CR 0001 References [103720] ETSI TS 103 720, 5G Broadcast System for linear TV and radio services; LTE-based 5G terrestrial broadcast system [23247] 3GPP TS 23.247, 3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2". [26502] 3GPP TS 26.502, 5G multicast-broadcast services; User service architecture Abbrevations: XXX XXXX |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S4aI240151; [FS_AMD] MBS User Service and Delivery Protocols for eMBMS; Qualcomm Germany; Thomas Stockhammer. E-mail Discussion: Revisions: S4aI240151r01.docx; 2024/10/18 12:42; 568,6 KB. S4aI240151_BBC.docx; 2024/10/16 19:41; 569,5 KB. Presenter: Thomas Stockhammer Online Discussion: October 18, 2024 - Thomas presents version r01. - Small online revisions Decision: - October 18, 2024: r01 not presented, but revision is done. Revision is postponed S4aI240151 is revised to S4aI240179. S4aI240179; [FS_AMD] MBS User Service and Delivery Protocols for eMBMS; Qualcomm Germany; Thomas Stockhammer. Presenter: Thomas Stockhammer Online Discussion: Decision: - October 18, 2024: postponed - October 25, 2024: not treated => expect resubmission for SA4#130 S4aI240179 is not treated. This is a resubmission of 179. |

## =====  CHANGE =====

## 2	References

[15]	3GPP TS 26.348: "Northbound Application Programming Interface (API) for Multimedia Broadcast/Multicast Service (MBMS) at the xMB reference point", Release 16.

[16]	3GPP TS 26.346: "Multimedia Broadcast/Multicast Service (MBMS); Protocols and Codecs", Release 16.

…

[18]	3GPP TS 29.468: "Group Communication System Enablers for LTE (GCSE_LTE); MB2 Reference Point; Stage 3".

[19]	3GPP TS 23.468: "Group Communication System Enablers for LTE (GCSE_LTE); Stage 2".

…

[23247]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2".

[26502]	3GPP TS 26.502: "5G multicast-broadcast services; User service architecture".

[103720]	ETSI TS 103 720: "LTE-based 5G Broadcast System".

[23479]	3GPP TS 23.479: "UE MBMS APIs for Mission Critical Services".

## =====  CHANGE (new – presented w/o revision marks) =====

## 5.10	Key Issue #9: MBS User Service and Delivery Protocols for eMBMS

### 5.10.1	Description

The MBS User Service architecture and protocol follows the modern design philosophies of the 5G System with separation of user services from transport, a service-based architecture and RESTful APIs. At the same time, eMBMS and enTV (as used for LTE-based 5G Broadcast) support transparent delivery mode and group communication. While interworking between MBMS and MBS is addressed in clause 5.2 of TS 23.247 [23247] and clause 4.9 of TS 26.502 [26502], interworking between these two systems at the User Service level is not addressed. In order for MBMS and LTE-based 5G broadcast as defined in ETSI TS 103 720 [103720] to leverage MBS User Service technologies, a study is warranted to identify the gaps to fully support this functionality.

Figure 5.10.1-1 reproduces the MBS–eMBMS interworking system architecture as documented in figure 4.9-1 of TS 26.502 [26502]. The functional elements that fall within the scope of [26502] are highlighted in green.

Figure 5.10.1-1: MBS–eMBMS interworking system architecture (see TS 26.502 [26502], figure 4.9-1)

The interworking architecture defined in clause 4.9 of [26502] addresses the following functionalities:

1.	Using MBS northbound interfaces at reference point Nmb10 for MBS, and using eMBMS northbound interfaces at reference point xMB-C or MB2-C for eMBMS.

2.	Potential dynamic switching between MBS and eMBMS reception, if a UE implements both an MBS Client and an eMBMS Client.

3.	Common ingest of content through reference point Nmb8/xMB-U, if these reference points are compatible.

4.	Common MBS User Services distribution and eMBMS delivery methods such that the same ingested content can be delivered to an MBS Client and to an eMBMS Client. UEs supporting only eMBMS are served by this architecture as well.

However, there is no guarantee that 3 and 4 can generally be achieved in practice.

### 5.10.2	Collaboration scenarios and architecture mapping

#### 5.10.2.1	Joint BM-SC + MBSF functionality

A more common interest is the ability to deploy a system for which MBS User Services are distributed via eMBMS. This would allow a single, common User Service specification for MBS and eMBMS/5G Broadcast to be maintained going forward. A modification of the architecture is shown in figure 5.10.2-1 in which:

-	Only the MBS northbound reference points Nmb10 and Nmb8 are exposed respectively by the MBSF and MBSTF. These are extended as required to support eMBMS transport (as those are extended, they are marked with an asterisk).

-	The UE in the 5G System is extended to support eMBMS reception, for example an LTE-based 5G Broadcast profile as defined in ETSI TS 103 720 [103720]. Such an approach permits a single middleware client with unified APIs, etc. to be deployed in the UE that is capable of both MBS User Service reception and eMBMS User Service reception.

Figure 5.10.2-1: MBS User Services on top of eMBMS

#### 5.10.2.2	MBS User Services feeding only eMBMS

A further variant of the architecture is shown in figure 5.10.2-2, in which case MBS radio delivery is not even in scope, but the MBS User Service is used to deliver only eMBMS traffic.

Figure 5.10.2-2: MBS User Services on top of eMBMS

#### 5.10.2.3	MBS User Services on top of eMBMS using Group Communication

Another possible implementation architecture is shown in figure 5.10.2-3 where a subset of MB2 procedures and protocols is used southbound of the MBSF and MBSTF to communicate with the EPS via a function implementing the Group Communication funtionality of a BM-SC. Such a deployment architecture may be of interest in order to address a combination of MBS User services with eMBMS radio delivery.

According to TS 26.346 [16], the Group Communication Service (GCS) AS as defined by 3GPP TS 23.468 [19] uses the MBMS Group Communication delivery method on top of MBMS bearers for MBMS delivery. However, in general, the MBMS Group Communication delivery method is available for any application. In this case, the application interfaces to the BM-SC at reference point MB2. This carries control plane signalling (via reference point MB2-C) and user plane data (via reference point MB2-U) between the Application Server for Group Communication (GCS AS) and the BM-SC. The data transferred via MBMS bearer(s) is delivered from the BM-SC using the Group Communication delivery method as defined in TS 26.346. Stage 2 procedures between the GCS AS and the BM-SC at reference point MB2 are provided in TS 23 468. The stage 3 specification of the MB2 procedures and the protocol aspects of MB2-C and MB2-U are specified in TS 29.468 [18].

In this deployment scenario, the MBS User Service is treated as an application on top of the Group Communication delivery method. The MBSF implements the relevant subset of MB2-C provisioning operations at a new reference point MB2'-C and the MBSTF implements the relevant subset of MB2-U protocols at a new reference point MB2'-U. A UE connecting to the E-UTRAN implements the relevant MBS User Service functionalities above suitable eMBMS middleware (MBMS Client) to support the reception of MBS User Services via the Group Communicaton API as defined in TS 23.479 [23479]. The MBMS client would only include the Access Stratum as well as the functionality to establish the group communication API.

Figure 5.10.2-3: MBS User Services on top of eMBMS using Group Communication

#### 5.10.2.4	MBS User Services on top of eMBMS using Transparent Delivery

Yet another possible architecture is shown in figure 5.10.2-4 where a subset of xMB provisioning procedures and protocols are used southbound of the MBSF and MBSTF to communicate with the EPS via a function implementing the Transparent Delivery functionality of the BM-SC.

In this scenario, the MBSF implements the relevant subset of xMB-C provisioning operations at a new reference point xMB'-C and the MBSTF implements the relevant subset of xMB-U protocols at a new reference point xMB'-U. A UE connecting to the E-UTRAN would implement the relevant MBS User Service functionalities above suitable eMBMS middleware (MBMS Client) to support the reception of the MBS User Services via the transparent delivery mode API as defined in 3GPP TS 26.347 [26347].

Figure 5.10.2-3: MBS User Services on top of eMBMS using Transparent Delivery

NOTE: In a normative spec a client architecture diagram is expected to be provided.

### 5.10.3	High-level call flows

#### 5.10.3.1	Joint BM-SC and MBSF functionality

The extended high-level baseline procedures for MBS User Services for the architecture showing in figure 5.10.2-2 are shown in figure 5.10.3.1-1, highlighting in bold the extensions to the call flow in clause 5.2.1 of TS 26.502[26502].

Figure 5.10.3.1-1: MBS User Service high-level baseline procedures
with Joint BM-SC and MBSF functionality

The same procedures as defined in clause 5.2 of TS 26.502 [26502], apply, but the distribution of content in steps 9 and 11 is via eMBMS. However, the detailed procedures documented in the remainder of clause 5 in TS 26.502 need further consideration to support eMBMS distribution.

In clauses 5.3, 5.3A, and 5.5 of TS 26.502, the communication with the MB-SMF to allocate TMGIs, create sessions, update sessions, or delete sessions is extended with a communication with the MBMS-GW at reference point SGmb, and the procedures defined in clause 8 of TS 23.246 [6] apply instead. In particular:

-	The BM-SC sends a Session Start Request message to MBMS-GW to indicate the impending start of the transmission and to provide the session attributes (TMGI, Flow Identifier, QoS, MBMS service Area, list of cell IDs if available, Session identifier, estimated session duration, list of MBMS control plane nodes (MMEs, SGSNs) for MBMS GW, time to MBMS data transfer, MBMS data transfer start, access indicator, ...).

-	The MBMS-GW responds with a Session Start Response message with information for BM-SC to send MBMS data to the MBMS-GW.

According to TS 23.247 [23247], a common TMGI for MBS and eMBMS is used towards the AF/AS and the TMGI is also used as identifier for transport over E-UTRAN/EPC.

In the user plane, the MBSTF distributes the received data to the MB-UPF at reference point Nmb9 and/or to the MBMS-GW at reference point SGi-mb, when supported by operator network configuration.

The session description document specified in clause 5.2.5 of TS 26.517 [26517] describes the parameters of the MBS distribution session using either:

-	The session description for the MBS Object Distribution Method, as specified in clause 6.2.2 of TS 26.517, or

-	The session description for the MBS Packet Distribution Method, as specified in clause 7.2.3 of TS 26.517.

In either case, the service type is restricted to Multicast MBS and Broadcast MBS as shown in clause 6.2.2.2 of TS 26.517. The use of the mbms-mode as defined in TS 26.346 [16] is not currently permitted in TS 26.517.

#### 5.10.3.2	MBSF/MBSTF southbound interface to BM-SC via MB2′

According to TS 23.468 [19], reference point MB2 offers access to the MBMS bearer service from an application. MB2 carries control plane signalling (MB2-C) and user plane traffic (MB2-U) between a Group Communication Application Server (GCS AS) and a BM-SC. Some relevant properties of MB2 are summarized as follows:

-	MB2 is used by the GCS AS to interact with the BM-SC for MBMS bearer management.

-	The application data transferred via MBMS bearer(s) by the GCS AS is transparent to the BM-SC.

-	MB2 is a standardized secured interface to an GCS AS.

-	The GCS AS needs to be configured with the IP addresses or a FQDN of the MB2-C endpoint on the BM-SC. A separate MB2-C endpoint needs to be exposed by the BM-SC per PLMN ID.

-	The user plane transport information (e.g. IP address/UDP port) for delivering a Group Communication application data flow from the GCS AS to the BM-SC over reference point MB2-U is exchanged over reference point MB2-C.

Reference point MB2 provides the ability for the application to use the functionality of the MBMS System to deliver data to group members over MBMS. The procedures supported include:

-	allocation of a set of TMGIs (TS 23.246 [3]) by the BM-SC at the request of the GCS AS (see clause 5.1.2.2.2 of TS 23.468 [19]),

-	deallocation of a set of TMGIs by the BM-SC at the request of the GCS AS (see clause 5.1.2.2.3 of TS 23.468),

-	activating an MBMS bearer in the BM-SC (see clause 5.1.2.3.2 of TS 23.468):

NOTE: This may include configuration requesting the BM-SC to apply Application Layer Forward Error Correction (AL-FEC) or Robust Header Compression (RoHC), or both, to the MBMS bearer.

-	deactivating an active MBMS bearer in the BM-SC (see clause 5.1.2.3.3 of TS 23.468),

-	modifying the characteristics of an active MBMS bearer in the BM-SC (see clause 5.1.2.4 of TS 23.468), and

-	reporting of MBMS delivery status by the BM-SC to the GCS AS (see clause 5.1.2.5 of TS 23.468).

A session at reference point MB2 is established between the GCS AS and the BM-SC before any MB2 messages are exchanged between these two entities, and this session carries all MB2 messages between them for all MBMS bearers provisioned and used by the GCS AS. The TMGI/FlowID is the unique identifier used by the GCS AS and BM-SC to refer to the MBMS bearer.

The extended high-level baseline procedures for the MBS User Services architecture using Group Communication depicted in figure 5.10.2-3 are shown in figure 5.10.3.2-1, highlighting in bold the extensions to the call flow compared with that in clause 5.2.1 of TS 26.502 [26502].

Figure 5.10.3.2-1: MBS User Service high-level baseline procedures
using Group Communication enablers and APIs

The core extensions are:

-	The Distribution Session provisioning, TMGI allocation and MBMS bearer allocation in steps 2, 3 and 4 are extended to address the allocation of the bearers on the MBMS distribution. The variant shown in the figure let’s MBSF handle the communicaton with MBSTF and BM-SC.

-	In step 10, the MBSF Client provides information to the MBMS Client using the MC-MBMS-API in order to establish the MBMS bearer, involving also the MBSTF Client.

-	In step 11, the MBMS Client activates the MBMS session to receive Group Communication data and the MBSTF Client activates the MBS User Services session to receive MBS data conveyed in the MBMS session.

-	In step 13, MBS User Services session data is received through the MBMS bearer and directly provided to the MBSTF Client for relevant processing, for example FEC decoding, unicast repair determination and so on.

#### 5.10.3.3	MBSF/MBSTF southbound interface with xMB to BM-SC

The call flow is similar to clause 5.10.3.2:

### 5.10.4	Gap analysis and requirements

#### 5.10.4.1	Joint BM-SC and MBSF Functionality

For the joint BM-SC and MBSF Functionality, no architectural gaps are identified. However, the following stage-3 gaps have been identified:

-	The session description for the MBS Object Distribution Method in TS 26.517 is restricted to describing MBS Sessions.

-	The session description for the MBS Packet Distribution Method in TS 26.517 is restricted to describing MBS Sessions.

Editor’s Note: There may be non-architectual gaps at stage 2 as well, such as in the domain model.

To support the functionality, the removal of the above restrictions is needed.

#### 5.10.4.2	MBSF/MBSTF southbound interface to BM-SC via MB2

In order to support the extended implementation in clause 5.10.3.2, in addition to the extensions documented in clause 5.10.4.1, the following extensions are needed:

-	The MBSF and MBSTF need to be able to communicate with BM-SC using southbound instances of reference points MB2'-C and MB2'-U respectively based on what is presented in clause 5.10.3.2, figure 5.10.3.2-1, steps 2, 3 and 4.

-	The MBSF Client needs to be extended in order to provide information to the MBMS Client in order to establish the bearer using the MC-MBMS-API, involving also the MBSTF Client, as shown in figure 5.10.3.2-1, step 10.

-	The MBSTF Client and the MBMS Client need to be extended to activate the session to receive content via MBMS bearer using the MC-MBMS-API, as shown in figure 5.10.3.2-1, step 11.

-	The establishment of the data from MBMS client to MBSTF client needs to be established to receive content via MBMS bearer using the MC-MBMS-API, as shown in figure 5.10.3.2-1, step 13.

#### 5.10.4.3	MBSF/MBSTF southbound interface to BM-SC via xMB

The gaps are similar to clause 5.10.4.2.

### 5.10.5	Candidate solutions

As a minimum change, the full support of the joint BM-SC and MBSF Functionality is expected. For this purpose, the gap documented clause 5.10.4.1 needs to be addressed by permitting the signaling of MBMS sessions.

In an extended change, the deployment architectures to run MBS user services on top of MB2 and xMB to connect to the BMSC are valuably documented in TS 26.502. The addition of these reference points to the MBS reference architecture are expected to require an extension to TS 23.247.

### 5.10.6	Summary and conclusions

It is recommended to

- 	the full support of the joint BM-SC and MBSF Functionality is expected. For this purpose, the gap documented clause 5.10.4.1 needs to be addressed by permitting the signaling of MBMS sessions.

-	the document deployment architectures to run MBS user services on top of MB2 and xMB to connect to the BMSC are valuably documented in TS 26.502 after validation with SA2.

-	to address all relevant other stage-2 and stage-3 updates to support MBS User services on top of MBMS.