3GPP TSG-SA WG4 Meeting #130							S4-241899

USA, Orlando, 18 – 22 November 2024

Source:	Nokia

Title:	On differentiated QoS handling of RTCP packets in 5GS

Agenda item:	10.6

Document for:	Agreement

# Background

In LS S2-2411001 (S4-241812), SA2 asked SA4 the following question on QoS handling of RTCP packets:

Question 2:

SA2 would also like to ask if, when differentiated mapping of (S)RTP media streams to QoS Flows is applied, there may be also a need to map certain (S)RTCP packet types to the same QoS Flow as the (S)RTP media streams they control?

Therefore, SA2 would also like to ask SA4 to guide SA2 on differentiated handling of (S)RTCP packets, and also whether Packet Type and SSRC value would suffice as extended packet filters.

# Discussion

While answering the above question from SA2, potential use of PDU Set based QoS handling in the network needs to be considered, in addition to the regular QoS handling.

For regular QoS flow handling, knowledge of the RTCP Packet Type (PT) matters, since RTCP sender/receiver reports (SR/RR) that are used for RTT measurement benefit from the same QoS treatment as the RTP stream they belong to. If RTCP packets carrying SR/RR are mapped to a different QoS flow relative to the associated RTP stream, there may be errors in measurement of the roundtrip time. However, the same requirement may not apply for other types of RTCP packets such as RTCP SDES. Therefore, transporting them with more relaxed QoS may not cause any issues.

An additional consideration when PDU Set based handling used is the assignment of PSI to lone PDUs. As studied in TR 26.822, RTCP packets are lone PDUs since they cannot be marked by the application. The solution #15 provides a mechanism for the sender to signal pre-defined PDU Set Importance (PSI) values for different RTCP messages. For such PSI signaling to be useful, the 5GC needs to be able distinguish, at minimum, between RTP and RTCP packets, and preferably among different RTCP messages such that network can prioritize more critical RTCP messages based on their assigned PSI. When PDU Set based handling is used, this provides an additional reason why Packet Type (PT) needs to be included in the packet filter.

In addition to the RTCP packet types defined in RFC 3550, there are also RTCP feedback (FB) messages which have the following common format defined in RFC 4585:

Several RTCP feedback messages are defined in the AVPF profile [RFC 4585], other RFCs (e.g. RFC 8888 Feedback for Congestion Control), and TS 26.114 (e.g. video region-of-interest in clause 7.3.7). RFC 4585 defines two values for the RTCP Packet Type (PT):

Differentiation between different transport-layer and payload-specific FB messages is provided via the Feedback Message Type (FMT) parameter in the RTCP header. FMT values for transport layer FB messages (RTPFB) and payload-specific FB messages (PSFB) are defined in the IANA RTP parameters registry: https://www.iana.org/assignments/rtp-parameters/rtp-parameters.xhtml which are reproduced below.

FMT Values for RTPFB Payload Types

FMT Values for PSFB Payload Types

Therefore, if RTCP FB messages are used, inclusion of FMT parameter would also be necessary in order to be able to fully differentiate between all RTCP message types. This decision should be made by assessing the benefit of such fine-granular differentiation vs a potential increase in the complexity of the packet filter.

# Proposal

We propose to provide the following answer for the question 2:

Question 2:

SA2 would also like to ask if, when differentiated mapping of (S)RTP media streams to QoS Flows is applied, there may be also a need to map certain (S)RTCP packet types to the same QoS Flow as the (S)RTP media streams they control?

Therefore, SA2 would also like to ask SA4 to guide SA2 on differentiated handling of (S)RTCP packets, and also whether Packet Type and SSRC value would suffice as extended packet filters.

Answer 2:

Regarding guidance of potential differentiated handling of (S)RTCP packets, SA4 believes that applying the same QoS parameters for both (S)RTP and (S)RTCP packets will be desirable in the absolute majority of cases. Any other approach would need explicit motivation and study of potential benefits and drawbacks.

When PDU Set based handling is used, RTCP packets will be lone PDU, i.e., not marked by the application with a PDU Set Importance (PSI) value. It is essential to differentiate between RTP and RTCP packets, as well as among various RTCP message types, to be able to assign a PSI value defined by the sender to the RTCP packets. SA4 agreed a solution (#15) in TR 26.822 that provides such mechanism.

Considering the above, SA4 believes that (S)RTCP Packet Type and SSRC value would suffice as extended (S)RTCP packet filters. In case further differentiation among RTCP feedback messages [RFC 4585] is necessary, the Feedback Message Type (FMT) can also be considered.