Source:				Qualcomm Incorporated

Title:			An Analysis of RAN Downlink Multi-modal Synchronization

Document for:	Agreement

Agenda Item:	10.3

## Introduction

This paper analyzes whether RAN assisted inter-modal synchronization is beneficial for XR-related services and proposes a response to the incoming LS from RAN2 in S4-241802 that requests SA4 to discuss the feature highlighted below.

Furthermore, RAN2 also agreed that traffic synchronization may be achieved by packet level synchronization information, if it can be provided from CN to RAN, which is up to SA2/4 to discuss.

- For DL, whether traffic synchronization (on a per packet basis) can be achieved depends on whether packet level synchronization information can be provided from CN to RAN. - RAN2 thinks PDU Set discard across QoS flows of the same multi-modal service based on the dependency information between the mutli-modal flows can only be achieved in case the synchronization information can be available at the UE which is up to SA2/SA4.

## Background

In Rel18, SA1 identified synchronization requirements in TR 22.847 as follows:

Though not stated in the SA1 study, the typical synchronization requirements for audio and video are:

Table 1. Audio-Visual synchronization requirements

| Audio delay | Visual delay |
| --- | --- |
| 125ms | 45ms |

Now consider how the UE receives and processes media packets delivered on the downlink with an example 50ms Packet Delay Budget (PDB) as shown in Figure 1.

Figure 1. UE processing of media packets received on the downlink

The UE cannot determine when the E2E playout or PDB limit is reached so the buffering time has to be autonomously determined by the UE considering the following:

- It is preferable to have more buffering before processing so the UE waits for late packets which would otherwise not be rendered, and which can even affect the decoding of later packets that reference the late packet.

- On the other hand, more buffering adds to the end-to-end delay.

- An adaptive DJB attempts to balance between adding enough buffering time/depth so that the UE can still receive and process enough late packets while not extending the E2E delay more than necessary.  The adaptation may occasionally time-warp the playout timeline as it adjusts the target buffer depth.

## Decomposing the analysis of RAN assisted inter-modal synchronization

To completely analyze the pros and cons of having the RAN synchronize delivery of media from different modalities is challenging as a good UE implementation can use techniques such as adaptive de-jitter buffering/time-warping and error concealment to minimize the effects of delayed packets.  Not only are these techniques very implementation-dependent, their performance usually has to be evaluated against complex subjective or objective media-specific KPIs (e.g., MOS, PSNR,…).

To make the analysis more tractable, we decompose the problem to just focus on what can RAN assistance do to so that the UE does not have to employ any of the above mitigation techniques because, regardless of the implementation, they all incur a performance cost.

## 3.1 Perfect RAN-Assisted Downlink Synchronization

First consider the case where packets are delivered exactly at the time needed for synchronized playout at the E2E requirement as shown in Figure 2.

Figure 2. Perfect RAN-Assisted Downlink Synchronization

This is ideal for the UE as it can playout all the media completely synchronized without having to perform any of the de-jitter buffering or error concealment for late packets.  However, this is very costly for the RAN due to the following:

- RAN Capacity impact for delivery at exact time

- Need to determine inter-media delays prior to packet arrival at gNB (e.g., RTP timestamps against common wall clock)

- Need to determine decoding + processing delays of UE

The above challenges may be solvable but the solutions are fairly complex and costly.

## 3.2 RAN-Assisted DL Synchronization with buffering

By relaxing the perfect RAN-assisted downlink synchronization, packets can arrive earlier than needed (are then buffered at the UE), to provide the scheduler a larger delivery window as show in Figure 3.

Figure 3. RAN-Assisted Downlink Synchronization with buffering

The above works as long as the scheduler does not introduce too much skew between the delivery of the different media types.  There is typically an anchor media type whose timeline drives the playout of all the media (e.g., speech playout for video telephony sessions).  It is important that the scheduler does not deliver the anchor media type consistently much earlier than another media type as this could cause the delayed media’s playout to exceed acceptable inter-modal synchronization requirements as illustrated for audio and visual media in Figure 4.

Figure 4. Potential loss of synchronization caused by RAN DL scheduling

The potential risk here is that since the PDBs are a maximum limit for the delivery delay, having larger PDB values provides the scheduler more opportunity to introduce unacceptable skew between media flows because it has the option to deliver some flows much earlier than others.

## Evaluating synchronization risks in real-time XR Scenarios

In the next sections we analyze how much the risk described in section 3.2 poses for two real-time XR scenarios:

- XR Split rendering sessions

- Conversational XR sessions

## 4.1 RAN-assisted DL multi-modal synchronization for XR Split Rendering

For split rendering, the need to minimize motion-to-render-to-photon/sound/haptic latency to meet XR requirements requires very short PDBs.  For example, to achieve the visual M2R2P roundtrip latency of 50~70ms, the DL PDB for visual media has to be 20ms or less.

Figure 5 demonstrates the possible worst case where the RAN delivers the audio packets within 1ms latency and delivers the visual packets at the 20ms PDB limit.  However, even in this worst case it can be seen that audio-to-visual delay is much less than the acceptable limit of 45ms.

Figure 5. Maximum potential delay of visual media relative to audio for split rendering

In the converse problem, where the visual media is delivered within 1ms but the audio is delayed until a relatively long 30ms PDB limit is illustrated in Figure 6.  In this case, it is also clear that the resulting skew between the audio and visual media is well below the maximum inter-modal synchronization requirement.

Figure 6. Maximum potential delay of audio relative to visual media for split rendering

Similar analysis can be shown for synchronization with haptic data, where the very short PDBs limit the RAN scheduler from introducing too much skew that could exceed the inter-modal synchronization requirements.  It is therefore suggested that RAN assisted inter-modal packet synchronization is not relevant for XR split rendering sessions.

## 4.2 RAN-assisted DL multi-modal synchronization for Conversational XR Services

For conversational services, the PDBs used are larger than for split-rendering as they are not part of the Motion-to-Rendering-to-Photon/Audio/Haptics loop, e.g., 100ms (5QI = 1) for speech and 150ms (5QI = 2) for video.  This can result in worst case scenarios where the scheduler delivers one type of media at a very low latency and others at a very high latency as demonstrated in Figure 7.

Figure 7. Large potential delay of visual media relative to audio for conversational XR services

In this scenario the large amount of skew introduced by the RAN scheduler can cause the visual media playout to be unacceptably delayed with respect to the audio playout.

While Figure 7 shows a possible scenario where the delivery of conversational media traffic could exceed inter-model synchronization requirements, it is not obvious to the authors that this would happen in actual deployments.  The long PDBs for voice and video have been defined since LTE and we have not encountered deployments that introduce unacceptable skew between audio and visual media.

## Conclusion

For split-rendering of multi-modal XR traffic, the PDBs required to meet the roundtrip M2R2photon/audio/haptic latency requirements are tight enough that the RAN DL scheduler should meet the inter-modal synchronization requirements without performing any explicit packet synchronization.

For conversational XR traffic, where PDBs can be longer, there is the potential for the RAN DL scheduler to introduce inter-modal delays that could exceed synchronization requirements if some media types are consistently delivered much earlier than others.  However, this has not been identified as a problem in actual video telephony deployments, even prior to 5G.

## Proposal

SA4 communicate the above conclusion in its reply to RAN2 and SA2.