Source:				Qualcomm Incorporated

Title:			UE Energy Consumption Reporting

Document for:	Agreement

Agenda Item:	8.7

## Introduction

S4aI240159 proposed the “energy index” solution to Key Issue #1.

In trying to address UE vendors concerns about reporting their device’s energy consumption metrics to the network, the solution proposes sending an energy index to obfuscate any sensitive information.  However, the index has the following limitations:

- It is proposed that the calculations to determine the index values be left to the implementation which makes the interpretation of the index values unclear and potentially inconsistent. It is unclear how the network or any other entity can use a reported value since the definition is unspecified.

- Developing a standardized formula to calculate the index values could solve the inconsistency problem.  However, standardizing a formula would be extremely challenging since there is no clear principle for deciding what are the relevant energy metrics (e.g., battery life, capacity, discharge rate, and UE energy consumption rate, energy source, etc…) and how these metrics should influence the calculated index value.

The authors do not see a good way to resolve the above limitations and instead propose a simpler approach after reviewing the requirements for exposing the UE’s media-related energy consumption.

## Requirements for exposing media-related energy consumption

Our understanding of the key requirements to expose media-related energy consumption information are as follows:

- The regulatory goal to raise user awareness of environmental issues requires informing users of the environmental impact of audiovisual media consumption.

- The MNO goal to reduce scope 3 (indirect) emissions, which is the most difficult aspect for reaching its net-zero strategy, requires informing users of the environmental impact of audiovisual media consumption.

For both of the above requirements, the energy consumption only has to be exposed to the user, not necessarily the network.  Therefore, the UE energy consumption could be reported to the user, e.g. via an App like the one identified in S4aI240159 and pasted below.

This approach would not require the UE to report its media-related energy consumption to the network.  If needed, the App could even be updated to show the energy consumption using other metrics such as “grams of CO2 emission”, which could also be further refined if the application is made aware of the UE’s energy source (e.g., zero-emission portable solar panel).

The overall energy consumption (device + the network and other components of the content delivery) for the user’s audiovisual services could also be reported to the user by an App that is given access to the end-to-end consumption information. The 5GMS client could be specified to collect the necessary information from the UE, network, and other entities, and provide this to the App for exposing to the user.

## Proposal

Agree on the principal of the above approach and the authors develop a pCR to update TR 26.942 accordingly.