3GPP TSG-SA4 Meeting #130	S4-241989

# Orlando, 18th - 22nd November 2024

Source:	Apple Inc.

Title:	On Derived chroma formats using layered HEVC

Agenda item:	9.11

Document for:	Information

# 1 Overview

This informative contribution serves as a follow-up to our previous contribution from the Jeju meeting, S4-241079 [1]. It summarizes the colour format enhancement methods utilizing layered HEVC. This contribution informs the group that the work at JVET has progressed, and the proposed SEI message has been incorporated into the Technologies under Consideration document of V-SEI [3].

# 2 Details

The majority of the deployed decoder implementations of HEVC and even VVC are only capable of supporting 4:2:0 profiles. Although there is considerable interest in the delivery of, especially, 4:4:4 signals, e.g. for screen and gaming content among other applications, unfortunately such content cannot always be natively delivered using the existing infrastructure.

The current Amendment 1 to HEIF 3rd edition [4] introduces the "colour format enhancement derivation" method, which utilizes derived image item to reconstruct a 4:4:4 signal from multiple 4:2:0 items. Additionally, JVET-AI0257 [5] introduced a new SEI message that enables the delivery of such content using the layered extensions of HEVC and 4:2:0 signals. Furthermore, during the last MPEG meeting, JVET-AJ0215 [2] presented a tool named ColorPacker, which was incorporated into HDRTools and can support such functionality. Consequently, the proposed SEI message was added to the Technologies under Consideration (TuC) for future extensions of V-SEI at JVET [3].

Below is the outline how layered extensions of HEVC can distribute colour components across different layers within the HEVC bitstream. A new SEI message, containing the necessary metadata for the reconstruction of images or videos with enhanced colour formats, is defined similarly to the ColourFormatEnhancement derivation in HEIF [4].

To ensure backward compatibility, layer 0 of the HEVC bitstream can carry the YCbCr 4:2:0 signal. Higher layers can carry enhancement information used to reconstruct higher capability colour formats, such as YCbCr 4:4:4 or RGB, while these additional components are still coded using traditional 4:2:0 coding. Furthermore, a new SEI message is defined to indicate how the data from different layers can be combined for reconstruction. Key elements of the metadata include:

- Layer Identification: Identifies which layers contain the base and enhancement streams.

- Colour Format Information: Specifies the target enhanced colour format (e.g. YCbCr 4:4:4).

- Component Mapping: Describes how to combine colour components from different layers.

- Component Packing Information: Indicates if multiple components are packed in the referenced video.

The figures below show a couple of examples:

Figure 1: YCbCr 4:4:4 with YCbCr 4:2:0 in layer 0 and full chroma in layers 1 and 2.

Figure 2: YCbCr 4:4:4 with YCbCr 4:2:0 in layer 0 and both full resolution chroma information packed in layer 1.

Obviously, the second solution may impact the decoder level, as higher capability may be required to decode all the layers. Nevertheless, it can be advantageous to achieve 4:4:4 decoding if the hardware does not provide native support for 4:4:4.

# 3 Implementations

A new module is introduced in HDRTools, named as ColorPacker, which can enable the desired functionality. The tool is capable of reading a single 4:4:4 chroma format video sequence, and the result will be 2 or 3 separate video sequences. The first sequence can contain a 4:2:0 representation of the signal, while in the 2 output configuration the second sequence contains an over-under packed version of the two chroma components at their full resolution placed in the luma colour plane of the output. In the 3 output configuration the two chroma components are placed in separate sequences instead. The inverse reconstruction is also supported by this tool.

A reference implementation of the metadata in the HM, and if desired in the VTM, will also be provided.

# 4 Proposal

It is proposed to note this informative document.

# References

- Apple Inc., "Colour Format Enhancement using layered HEVC", S4-241079

- D. Podborski, A. M. Tourapis, S. Paluri, J. Kim, "Derived chroma formats using layered HEVC (Status update)", JVET-AJ0215, Nov 2024

- WG 05 MPEG Joint Video Coding Team(s) with ITU-T SG 16, "Technologies under consideration for future extensions of VSEI (version 6)", JVET-AJ2032, Nov 2024

- MPEG WG3 Output document, "Text of ISO/IEC 23008-12 3rd edition DAM 2 Support for tone map derivation and others", MDS24143, July 2024

- A. M. Tourapis, D. Podborski, S. Paluri, J. Kim, "Derived chroma formats using layered HEVC", JVET-AI0257, July 2024