Source:	Philips International B.V., Qualcomm Incorporated

Title:	IVAS Characterization - Room Acoustics Testing

Document for:	Discussion and Agreement

Agenda Item:	7.5

# Introduction

IVAS renderer is an integral part of the selected IVAS standard. The IVAS renderer supports binaural rendering employing room acoustics synthesis to create an immersive audio effect. Rendering is planned to be tested during IVAS characterization testing. For room acoustics synthesis the traditional testing methodologies based on comparison of audio presentations, such as P.800 or MUSHRA appear insufficient. The P.800 method does not implicitly cover plausibility aspects and MUSHRA is not applicable as there are no actual reference signals available. Therefore, the discussion on testing room acoustics synthesis has been initiated with documents S4-240257 [1] and S4-240261 [2].

This contribution provides an update on preparation for room acoustics synthesis testing, addressing open points as well as reflecting on comments and questions raised during discussions on the previous documents.

# Background

This section provides a summary of the IVAS room acoustics synthesis and related testing methodologies as proposed in the previous documents.

## IVAS room acoustics synthesis

The IVAS decoder/renderer supports synthesis of room acoustics using either BRIR convolution or late reverb generation with additional early reflections synthesis combined with HRIR binauralization. Late reverb is driven by acoustic parameters RT60 and DSR accompanied by the pre-delay time, while early reflections synthesis uses parameters describing physical room properties.

## Room acoustics testing paradigm

The aim of related testing is to assess the quality and accuracy of room acoustics synthesis achieved with the IVAS renderer. Both objective and subjective tests are to be conducted. The tests should cover a variety of simulated rooms of different acoustic properties. Figure 1 illustrates general test setup for room acoustics synthesis using integrated IVAS decoder (a.) and external IVAS renderer (b.). A simple convolution-based reference renderer should be used, for instance the reference Python renderer as available in the IVAS processing scripts.

Figure 1. Room acoustics rendering test setup

It was proposed to use recorded impulse responses of actual rooms. Such recorded impulse responses need to be converted to BRIR datasets by combining them with HRIR data. These BRIR datasets can be used with the reference renderer and with the IVAS decoder/renderer after applying relevant conversion steps. The recorded room impulse responses can be also used for generating reverb parameters for the IVAS reverberator. The process of generation of BRIR datasets from recorded impulse responses and HRIR sets is discussed further in this contribution. The listening test process and the test tools are another subject discussed in this contribution.

# Room impulse response recordings

Several impulse response recordings representing a variety of room types have been made by Qualcomm. The measurements were taken for a stereo loudspeaker setup (Genelec 8240A), as shown in Figure 2. The recordings were made using the Eigenmike EM32 microphone connected to a PC using RME MADIface XT interface.

Figure 2. Room impulse response recording setup

For the recordings a sinusoidal sweep signal from 20Hz to 20kHz of a length of 2 seconds was used. The recordings were made for each loudspeaker separately. Multiple sequential recordings were averaged (four or eight). The background noise levels in the rooms were low enough to achieve 80dB headroom. The impulse response signals were achieved by deconvolving the recordings with the original sweep signal. Impulse responses of four rooms were recorded, of which two were provided in two variants, namely an auditorium, a listening room, a kitchen (with curtains open or closed), and a lounge (with doors open or closed). These recordings are further referred to as audio scenes.

Next to the impulse response recordings also 360° photos were taken in the rooms with Ricoh Theta camera. These photos are to be rendered with a VR headset during testing.

Because of the logarithmic sweep method used to measure the impulse responses  [9], the non-linearities of the measurement setup (e.g. from a loudspeaker or a microphone) end up in the tail of the impulse response.  As shown in Figure 3, both the non-linearities and the background noise levels show up in the energy decay curve of the raw impulse response as a plateau in the tail.  Using these raw impulse responses for further processing can lead to audible artifacts, especially in the tail of the reverb. One method to suppress this is applying a time window to remove or suppress the tail.

Figure 3. Energy decay of impulse response recordings: raw and time-windowed.

# BRIR generation from spatial impulse response recordings

To obtain a Binaural Room Impulse Response (BRIR), ideally a spatial impulse recording in a real room must be made, for all the required source positions and using a rotating dummy-head-microphone. This a very time-consuming  task. Therefore, a vast amount of research work has been performed on generating the BRIR datasets based on actual spatial impulse response recordings and Head Related Transfer Function (HRTF) sets. The most common approach is to record spatial room impulse responses with a Soundfield microphone and to render these using anechoic binaural HRTF sets. There are two main approaches prevailing: Spatial Decomposition Method (SDM) [5], and Higher-Order Spatial Impulse Response Rendering (HO-SIRR) [6]. Below, a short summary of these methods was provided aimed at their applicability for IVAS room acoustics synthesis testing.

## Spatial Decomposition Method

The spatial decomposition method (SDM) converts spatial room impulse responses into a set of image-sources. The recorded room impulse responses are converted to an omnidirectional pressure signal and a series of computed directions of arrival per pressure signal sample. Such image-sources representation can be used for room acoustics analysis. By convolving image-sources representation with an anechoic HRTF dataset, a BRIR set can be generated. The SDM method supports various multiple microphone arrays with an assumption that either actual or synthesized omnidirectional pressure signal is available.

The Binaural SDM Toolbox MATLAB code is available in [7]. This toolbox is supposed to support Eigenmike input “out-of-the-box”. The code required minor updates to get it work with the recorded impulse responses and the default IVAS HRTF dataset. It is necessary to provide input in both raw and Ambisonics B-format, since the latter provides the omnidirectional pressure signal. The Ambisonics B-format representation was generated using MATLAB Audio Toolbox. The Binaural SDM Toolbox enables storing BRIR data using standard SOFA file format.

The sources have generated several BRIR sets based on the recorded room impulse responses combined with the IVAS default HRIR set. For the sake of complexity and the resulting BRIR size, a reduced number of listener poses was used corresponding to 7.1+4 virtual loudspeakers setup (CICP19). The BRIR datasets obtained were evaluated using convolution-based renderer implemented using Python. The rendered audio sounds as expected, including plausible room acoustics synthesis.

## Higher-Order Spatial Impulse Response Rendering

Higher-order Spatial Impulse Response Rendering (HO-SIRR) [6] is a rendering method, that synthesizes room impulse responses (RIRs) of loudspeaker arrays using  spherical harmonic input RIRs (Ambisonics B-format). HO-SIRR makes assumptions regarding the composition of the sound-field and extracts spatial parameters over time, which are mapped on a specified 3D loudspeaker configuration, which in turn can be mapped into a BRIR using an anechoic HRTF dataset.

The HO-SIRR toolbox is available in [8]. Using HO-SIRR, RIRs were processed to a CICP19 loudspeaker setup which was rendered using the default IVAS HRTF dataset to a BRIR dataset. The rendered audio had a strong coloration; emphasizing the high frequencies, also some unexpected echo-like artefacts were audible.

## BRIR synthesis method selection

Based on the findings as discussed above it has been decided to use the Binaural SDM method for the synthesis of BRIR datasets based on recorded room impulse responses and the default HRIR sets of IVAS. The Binaural SDM method results in good sounding BRIR datasets allowing for plausible rendering in a virtual room environment based on recorded room impulse responses.

# Listening test process and tools

In the previous contribution [2] on room acoustics testing several constraints on test process and tools were proposed. It has been recommended to use head tracking as it can significantly improve sound externalization and general immersive qualities of rendered audio. Furthermore, the importance of providing visual references was discussed, along with implementation options combining visual and audio presentation. Finally, the MuSCR (Multiple Stimulus Category Rating) methodology was recommended. MuSCR tests can be compared to MUSHRA without a hidden reference.

## Room acoustics synthesis test tools

The sources started developing prototypes of the listening test tools based on the constraints discussed in the previous contribution. The listening test tool prototype provides a VR gaming platform (Unity) based 3DoF interactive environment. It allows for live head tracking, immersive rendering of the 360° photo of a room, as well as providing head orientation data to the IVAS decoder/renderer instances and managing audio buffers rendered by these instances using socket communication. The plausibility test GUI displays the selection of the test cases (i.e., conditions under test) along with the unipolar 100-point quality scale per test case. A screenshot of an example of such a GUI is shown in Figure 4.

Figure 4. VR environment screenshot showing MuSCR voting scale

A virtual reality game experience is created in Unity, which acts as a comprehensive subjective testing environment for room acoustics simulation and rendering features of IVAS. The use of a virtual reality platform implicitly incorporates head tracking as part of a larger pose tracking mechanism. This is made possible by a head tracker that is incorporated within the VR headset. The VR application acts as a prototype for a testing environment that will be used for room acoustics testing during IVAS characterization.

As shown in Figure 4, the listener will be provided with 4 options for a given scene, each of which will represent a different condition under test (CuT). These conditions under test are discussed in section 5 of this contribution.

Each CuT refers to a rendering mode associated with a single IVAS decoder/renderer or a reference renderer instance. The virtual listening environment allows a listener to switch between CuT outputs and to assign them scores using a MuSCR-compliant GUI within the virtual environment.

The Unity application uses a lightweight and low-latency socket architecture based on TCP for connecting Unity and IVAS. The IVAS decoder instances decode the output in a frame-by-frame manner and stream these frames to the output buffers of the listening test tool. Audio output buffers in Unity are set up using the NAudio library [10]. The audio output buffer of the listening test tool streams data frames from a selected CuT. This solution requires only minimal changes to the current IVAS implementation. Furthermore it provides a simple method of streaming live head tracking data into IVAS by using the same TCP sockets. For added robustness, the Unity project has a dedicated socket for communication with each different CuT instance distinguished by the port number.

The CuT instances are launched as command line applications using C# from Unity so that multiple applications run in parallel. Owing to the TCP-based streaming function, this feature can also be extended to execute IVAS instances on a different host, allowing for effective CPU load distribution.

The unity testing environment thus serves as a comprehensive testing tool, enabling listeners the full immersive experience based on room acoustics rendering by IVAS.

## Conditions under test

In the previous contribution the proposed conditions under test included synthetic reverb (with early reflections if available), IVAS default BRIRs, measured BRIRs, and HRIRs without reverb (as an anchor condition). The sources propose to slightly update this list to contain the conditions, as provided in Table 1.

Table 1. Conditions under test for MuSCR tests

| Identifier | Description |
| --- | --- |
| CuT0 | Reference renderer using synthesized BRIRs |
| CuT1 | IVAS decoder/renderer using synthesized BRIRs |
| CuT2 | IVAS decoder/renderer using synthetic reverb (with early reflections if applicable) |
| Anchor | IVAS decoder/renderer using default HRIR (no room acoustics) |

Both CuT0 and CuT1 use synthesized BRIR datasets obtained by combining recorded room impulse responses and IVAS default HRTF datasets, as discussed in section 4. The recorded room impulse responses are also used to compute reverb parameters used in CuT2. The CuT0 condition can be seen as a hidden reference. In this case, a reference renderer (i.e., based on the reference Python renderer as available in the IVAS Processing scripts repository) is used in combination with the SOFA representation of the synthesized BRIRs. Such a reference renderer shall support socket-based interfaces for real-time head-tracking input and audio buffer output connection with the listening test environment. The synthesized BRIRs converted to the IVAS binary HRTF/BRIR format are used as input for IVAS decoder/renderer in CuT1. IVAS decoder/renderer with reverb (CuT2) conditions use reverb parameters for room acoustics synthesis optionally with early reflection parameters based on the room characteristics. Note that not all conditions under tests are applicable for all rendering modes, see Table 2. This results in a varying number of conditions per test case.

Table 2. Condition under tests availability for rendering modes

|  |  | CuT0 | CuT1 | CuT2 |
| --- | --- | --- | --- | --- |
| Rendering mode | ParamBin | ● | ● | ● |
|  | FastConv | ● | ● | ● |
|  | CREND | ● | ● | ●1 |
|  | TDREND | ● | - | ● |

1 with early reflection synthesis

Rendering modes are selected implicitly based on the format of the bitstream file used as input for a given IVAS instance. Owing to the implications of each rendering mode on the conditions under test (as shown in the table above), a trial is defined as a test carried out for one rendering algorithm in each scene (room configuration), creating “Scene-Algorithm Pairs”. A trial is then set up based on these pairs, using the right number of IVAS instances.

The combinations of the rendering modes and the audio scene presentations need to be limited to remain within a focus timespan of a listener. It is assumed that a listening test session should not exceed 20-30 minutes and should contain a maximum of 10-15 trials, leaving about 2 minutes to complete a trial of 3 to 5 conditions. With an average of 4 trials, assuming a sample duration of 10 seconds, each sample could be listened to three times per trial.

As there are six room configurations impulse recordings available and there are four rendering modes, a proper selection of permutations of the above should be made. Permutations of room configurations and rendering modes are made keeping in mind the different artifacts expected from each room configuration, and the features of each rendering mode, such as compatibility with early reflections for binaural room reverb.

# Conclusions

In this contribution matters related to IVAS room acoustics testing are discussed. The methods of synthesizing BRIR datasets from recorded room impulse responses and anechoic HRIR datasets are discussed. It has been proposed that the Binaural SDM method should be used for the BRIR synthesis.

Furthermore, the prototype of a VR testing environment has been developed in such a way that minimal development effort is required for  adding new features. The prototype can confidently serve as the basis for room acoustics testing during the IVAS characterization phase. The VR test environment provides a platform for subjective testing of different IVAS rendering configurations. The environment supports head tracking to fully utilize the immersive rendering capabilities of IVAS. The GUI is based on MuSCR standards combined with trial results logging to effectively consolidate subjective testing.

The IVAS room acoustics test methodologies are subject to further discussion and updates based on feedback from IVAS Proponents and the Test Labs and are intended to be included in the characterization phase test plan (IVAS-8b).

# References

- S4-240257 – IVAS Characterization Testing – Binaural Rendering, Philips International B.V., Qualcomm Incorporated

- S4-240261 – IVAS Room Acoustics Testing Methodologies, Philips International B.V., Qualcomm Incorporated

- 3GPP TS 26.258 v0.1.0, Codec for Immersive Voice and Audio Services; C code (floating-point).

- Koppens, J., Kechichian, P., Jelfs, S., “A Generic Reverberation Characterization Metric for Accurate Simulation in Virtual and Augmented Reality Environments”, Proceedings of the I3DA 2023, Immersive and 3D Audio From Architecture to Automotive.

- Tervo, S., Patynen, J., Kuusinen, A., Lokki, T., (2013). “Spatial Decomposition Method for Room Impulse Responses,” Journal of the Audio Engineering Society, vol. 61, no. 1/2, pp. 17-28

- McCormack, L., Pulkki, V., Politis, A., Scheuregger, O. and Marschall, M., (2020). “Higher-Order Spatial Impulse Response Rendering: Investigating the Perceived Effects of Spherical Order, Dedicated Diffuse Rendering, and Frequency Resolution”. Journal of the Audio Engineering Society, vol. 68, no. 5, pp. 338-354

- Facebook Research, BinauralSDM GitHub repository: https://github.com/facebookresearch/BinauralSDM

- McCormack, L., Pulkki, V., Politis, A., Scheuregger, O. and Marschall, M., (2020). "Higher-Order Spatial Impulse Response Rendering: Investigating the Perceived Effects of Spherical Order, Dedicated Diffuse Rendering, and Frequency Resolution". Journal of the Audio Engineering Society, 68(5), pp.338-354. https://nl.mathworks.com/matlabcentral/fileexchange/88211-ho-sirr

- Müller, S., Massarani, P. (2001). Transfer-Function Measurement with Sweeps. Journal of the Audio Engineering Society. 49.

- Heath, M, NAudio .NET audio library GitHub repository:
GitHub - naudio/NAudio: Audio and MIDI library for .NET