- 3GPP TSG-SA5 Meeting #154	S5-243147

- 15 - 19 April 2024, Changsha, Hunan, China 	revision of S5-242374

- Source:	Nokia, Nokia Shanghai Bell

- Title:	pCR 28.867 Triggering CCL decision escalation

- Document for:	Approval

- Agenda Item:	6.19.4

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1] 	3GPP TR 28.867-010 “Closed control loop management”.

# 3	Rationale

CCLs will make decisions in different network contexts. Since not all decisions are equally effective, the CCL should be enabled to escalate its decision making to another entity. This pCR adds a UC on enabling the CCL to escalate its decisions.

# 4	Detailed proposal

Start of modification

# 5. 			Use Cases

### 5.X	CCL feedback

5.X.1	Description

5.X.1.1 Overview

CCLs will make decisions in different contexts (states, status, conditions, etc.) of the network and not all decisions are equally effective. In some cases, the CCL may need to provide feedback indication the challenges faced on reaching a definite decision. The feedback may include a request to escalate its decision making to another entity.

5.X.1.2 CCL decision escalation

A CCL can be fully or partially autonomous on derivation or execution of its decisions. The level of autonomy of the CCL, which is the degree to which it independently executes decisions or escalates them, should be flexibly configurable by the MnS consumer. The degree of autonomy could be configured based on the sensitivity of the operations under its control, the trust level in its decisions and the necessity to consider a bigger picture at times. The CCL should be able to escalate a decision or situation to an escalation recipient which has this bigger picture (say has wider scope), can execute a different(larger) set of actions or has better capabilities, e.g., a larger and more capable ML model.

The escalation recipient CCL enables the escalator CCL to request for escalation for a given network context or state with e.g., information about the escalator CCL preferences and observed constraints when driving decisions. Based on its evaluations, the escalation recipient CCL should provide to the escalator CCL a report that holds the outcomes that the CCL (acting as an escalation recipient) has derived for a given escalation request.

Note: The relation with existing Assurance closed loop execution supervision use case in TS 28.535 needs to be clarified.

#### 5.X.2	Potential requirements

REQ- CCL-ESC-1: The CCL MnS producer should have a capability to enabling an authorized consumer to configure the degree of autonomy  of the CCL as characerisation of the conditions under which the ACCL should escalate a decision

REQ- CCL-ESC-2: The CCL MnS producer should have a capability to enabling an authorized consumer to configure the enity to which a decision should be escalated

REQ-CCL-ESC-3: The CCL MnS producer (acting as an escalation recipient CCL) should have a capability to enabling an authorized MnS consumer (e.g., an escalator CCL) to to request escalation of a decision or escalation of decision-making for a given network context or state to the CCL associated with the CCL MnS producer

REQ-CCL-ESC-4: The 3GPP management system (or the CCL Mns producer) should have a capability enabling an MnS consumer to provide information related to its previous decisions, decision constraints, preferences, … as input to be used in resolving escalations sent towards the CCL associated with the CCL MnS producer.

REQ-CCL-ESC-5: The CCL MnS producer (acting as an escalation recipient CCL) should have a capability to provide to an authorized MnS consumer (e.g., an escalator CCL) a report that holds the outcomes that the CCL (acting as an escalation recipient) has derived for a given escalation request.

### 5.x.3	Possible solutions

5.x.3.1	CCL decision escalation

- Introduce an attribute defining the entity to which the decision is escalated to, say called the escalationRecipient.

- Introduce on the CCL representing the escalationRecipient. an IOC representing the request for escalation, say called EscalationRequest, that holds all information related to the request for escalation.

- The escalationRecipient may send an output represnetin the outcom o the escalation.

5.X.4			Evaluation of solutions

TBD

End of modifications