3GPP TSG-SA5 Meeting #155 	S5-243249

Jeju, South Korea, 27 - 31 May 2024

- Source:	Huawei

- Title:	Update terms

- Document for:	Approval

Agenda Item:	6.19.20

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1]		3GPP TR 28.880 v0.2.0: Study on energy efficiency and energy saving aspects of 5G networks and services

# 3	Rationale

This contribution proposes some update on the terms in TR 28.880 [1].

- Adding two new terms used in the TR;

- Enhancing some terms for better readability.

# 4	Detailed proposal

This document proposes the following changes in TR 28.880 [1].

1st Change

# 3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

carbon emission: quantity of equivalent carbon dioxide emitted (e.g. kg of CO2 equivalent).

NOTE 1: this definition is taken from TS 22.261 [7].

carbon emission factor: kilograms of equivalent carbon dioxide emitted per kWh (kg of CO2eq/kWh).

NOTE 2: this definition is taken from ETSI GS OEU 020 [8].

NOTE 3: in this document, the carbon emission factor considered is the operational carbon emission factor, which considers only emissions that directly result from the operation of a given type of energy source. Operational carbon emission factor is different from life-cycle carbon emission factor which considers all emissions, including those generated during the construction of power plants, the extraction of fuels, etc.

carbon emission efficiency: ratio between the carbon emission and the performance of a network entity.

NOTE 4: the definition of the performance of a network entity depends on the type of network entity. In this document, network entity can represent a gNB, a 5GC NF, etc. The performance of a network entity can be e.g. data volume.

Energy Consumption (EC): See definition in TS 28.310 [12].

Energy Efficiency (EE): See definition in TS 28.310 [12].

renewable energy: energy from renewable non-fossil sources.

NOTE 5:	This definition is taken from TS 22.261 [7].

NOTE 6: Examples of renewable energy sources include wind, solar, aerothermal, geothermal, hydrothermal and ocean energy, hydropower, biomass, landfill gas, sewage treatment plant gas and biogases

NOTE 7: A renewable energy source is an energy source not depleted by extraction as it is naturally replenished at a rate faster than it is extracted ISO/IEC 30134-3:2016 [10].

NOTE 8: Criteria to categorize an energy as renewable can differ among jurisdictions, based on local environmental or other reasons ISO/IEC 30134-3:2016 [10].

renewable energy factor: ratio of the renewable energy to the total energy.

NOTE 9: this definition is taken from ISO/IEC 30134-3:2016 [10].

End of change