3GPP TSG-SA5 Meeting #157	S5-246088

Hyderabad, India, 14 - 18 October 2024

- Source:	Nokia

- Title:	pCR 28.572 Add content for activation

- Document for:	Approval

Agenda Item:	6.19.9.1 - Management of planned configurations

# 1	Decision/action requested

The group is requested to discuss and approve the pCR below.

# 2	References

[1]		3GPP TS 28.572: "Management of planned configurations"

# 3	Rationale

None.

# 4	Detailed proposal

The following changes are proposed for TS 28.572[1].

Begin of modifications

## 6.4	Activation of planned configurations

### 6.4.1	Definition

Activation is the process of applying a planned configuration to the current configuration (instead of to a copy of the current configuration as for validation). This process can take a bit of time.

Next modification

## 6.13	Managing activations

Activation of a planned configuration is requested by MnS consumers and performed by MnS producers. The planned configuration to be activated is identified in the request by its plan identifier. Alternatively, if no planned configuration descriptor has been created for the planned configuration to be activated, the planned configuration may also be specified in the activation request.

Optionally, a MnS consumer can request activation of multiple plans by specifying multiple plan identifiers or a planned configuration group identifier in the activation request.

Editor's note: Result details incl. error details and error overview are ffs

The following table specifies the information elements of the information model for plan activation.

| Information element name | Description |
| --- | --- |
| activation-job-id | The identifier of the activation job. |
| plan-ids | The ordered list of the identifiers of the plans to be activated. |
| planned-configuration-group-id | or, alternatively, the planned configuration group id for the planned configuration group to be activated. |
| planned-conf | or, alternatively, the planned configuration to be activated, if no planned configuration was created earlier. |
| activation-mode | The activation mode for the planned configuration, either "ATOMIC" or "BEST_EFFORT". Only when "planned-conf" is present. |
| relationship-between plans | When more than one plan shall be activated, this attribute allows to specify the relationship between the plans to be activated. Allowed values are "INDEPENDENT", "SEQUENTIAL_ORDERED and "COMBINED". |
| conflict-mode | When more than one plan shall be validated, this attribute allows to specify for the "INCREMENTAL_ORDERED" and "COMBINED" modes, if conflicting changes in different plans shall result in a failure, or if conflicting changes shall be applied in order. Allowed values are "STOP_ON_CONFLICT" or "CONTINUE_ON_CONFLICT". |
| enable-fallback | This boolean attribute allows to specify if the MnS producer shall create a plan with the current configuration to allow for a later fallback. |
| activation-service-impact | The service impact mode, either "LEAST_SERVICE_IMPACT" or "SHORTEST_TIME". |
| cancel-request | This boolean attribute allows to request to cancel the activation process. |
| activation-state | The activation state, either "NOT_STARTED", "RUNNING", "CANCELLED" or "COMPLETED". |
| activation-start-time | The date and time at which the activation process started. |
| activation-stop-time | The date and time at which the activation process stopped. |
| activation-result | The activation result, either "ACTIVATED" or "ACTIVATION_FAILED". When "activation-mode" is "BEST_EFFORT" also "ACTIVATION_PARTIALLY_FAILED". |
| error-details | In case validation failed or partially failed, error reasons are specified here. |

End of modifications