3GPP TSG SA WG5 Meeting #158	S5-246964

Orlando, Florida, USA 18 - 22 November 2024

# Source:	Huawei, Ericsson, Nokia

# Title:	Evaluation and conclusion for CAPIF charging topic 3

# Document for:	Approval

Agenda Item:	7.5.2

1 	Discussion

This is a pCR to add the evaluation and conclusion for CAPIF charging in TR 28.849.

2	References

[1]	3GPP TR 28.849: "Study on charging aspects of Common API Framework for Northbound APIs (CAPIF) phase 2".

3	Rationale

This pCR proposes to introduce the evaluation and conclusion for CAPIF charging topic 3, API invocation charging.

4	Detailed proposal

Propose to incorporate the following change into the TR 28.849 [1].

First change

6.3.4	Key issues

6.3.4.1	Key issue#3.1: Charging events and charging information required

This key issue is for investigating how to support the charging considering REQ-3GPPCH-APIInvo-01 and REQ-3GPPCH-APIInvo-02. This investigation covers the following:

-	identification of the charging information for API invocation;

-	identification of the chargeable event for API invocation;

Next change

6.3.5.1	Solution #3.1: API invocation via CAPIF

6.3.5.1.1	General description

This solution #3.1 which relying on CHF/5G Converged Charging System for CAPIF Converged Charging, addresses the Key Issue #3.1.

6.3.5.2	Solution #3.2: Use of Exposure function Northbound Application Program Interfaces (APIs) charging

##### 6.3.5.2.1	General description

This solution covers key issue #3.1, and requirements REQ-3GPPCH-APIInvo-01and REQ-3GPPCH-APIInvo-02. It reuses the current exposure function northbound APIs charging, TS 32.254 [3], with some adaptations.

Next change

6.3.6	Evaluation

6.3.6.1	Solutions evaluation for Key issue #3.1

Solution #3.1 provides the CAPIF Core Function (CCF) capability to support CAPIF Service APIs invocation/notification charging. A new charging specification is required focusing on the CAPIF charging or a new clause for CAPIF charging in the TS 32.254 [3] for the CAPIF service API invocation/notification charging.

Solution #3.2 provides the capability to support charging of API invocation, by using the current NEF charging architecture and information.

End of change