3GPP TSG-SA6 Meeting #64	S6-245065

# Orlando, USA, 18th - 22nd November 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0221 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | API invoker split into frontend and backend components |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple (UK) Limited |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3 |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | It is proposed to introduce the high-level architectural aspects from solution #1 of TR 23.700-22 in which, in support of enhanced security for OAuth classified “public clients”, a server-side backend component is introduced to handle OAuth responsibilities and optionally act as a proxy for all (application) client requests. |  |  |  |  |  |  |  |  |
| Summary of change: |  | SA6 sent an LS to SA3 asking whether SA3 would consider supporting the BFF patten in Rel-19 (S6-243764), to which SA3’s response (S3-244340) states that the “BFF patten issue is not captured in the scope of TR 33.700-22 v.0.1.0. If SA3 decides to study it in the scope of TR 33.700-22, SA3 will inform SA6”. Of note, the SA3 SID (SP-240654) has a single WT: To investigate the security impacts and study potential solutions for the Rel-19 CAPIF enhancement studied in SA6 (Study on CAPIF Phase 3). The BFF pattern was studied by SA6 and captured as solution#1. In the associated TR 23.700-22 evaluation it is stated the security aspects related to the proposed enhanced architecture are to be further studied. Based on this it is proposed to capture that an API invoker may have a client-side (frontend) and server-side (backend) component. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Deployment options in support of OAuth 2.0 public clients (e.g., clients executing on the device used by the resource owner such as an installed native application or a web browser-based application) are inadequate. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 & 7.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 6.3.2	API invoker

The API invoker is typically provided by a 3rd party application provider who has service agreement with PLMN operator. The API invoker may reside within the same trust domain as the PLMN operator network. The API invoker may be either an application on a server or an application on a UE or an application comprising of a frontend component on a UE plus a server-side backend component.

The API invoker supports the following capabilities:

-	Triggering API invoker onboarding/offboarding;

-	Supporting the authentication by providing the API invoker identity and other information required for authentication of the API invoker;

-	Supporting mutual authentication with CAPIF;

-	Obtaining the authorization prior to accessing the service API;

-	Discovering service APIs information; and

-	Invoking the service APIs.

## NOTE:	The details of the specific service APIs are out of scope of the present document.

* * * Second Change * * * *

## 7.5	RNAA deployments

CAPIF supports RNAA and has enabled API invoker(s) to have authorized access to resources of a resource owner provided by service APIs offered by the AEF. The CCF acts as the Authorization Function and supports the authentication and authorization of the resource owner. Based on resource owner's authorization, the CCF provides the access token for a service API access to the API invoker. The API invoker performs service API invocations on the AEF by utilizing the access token.

The API invoker may be deployed in the following ways:

a.	API invoker may be deployed as AF on the UE (i.e. 3rd party application).

b.	API invoker may be deployed as AF on the UE supporting several other 3rd party applications deployed on the UE.

c.	API invoker may be deployed on the network as AF.

d. 	API invoker may be deployed with a frontend component on the UE with a backend component deployed on the network

The resource owner is connected via a UE and can use a Resource Owner Function deployed on the UE to interact with the CCF acting as the Authorization Function for authentication and authorization i.e., granting permission to the API invoker to access resource(s) of the resource owner provided by the service API.

NOTE:	The details of the protocol for CAPIF supporting RNAA is specified in 3GPP TS 33.122 [12].

When API invoker is deployed on a UE (cases a, b and d), the API invoker is allowed to access the resources of the resource owner corresponding to the UE.

When the API invoker is deployed with a backend component in the network (case d), the backend component is responsible for authentication and authorization procedures towards the CCF Authorization Function. The backend component may proxy service API discovery and invocation requests initiated by the API invoker frontend.

Figure 6.2.2-1: High level functional RNAA architecture for CAPIF supporting a split frontend / backend API invoker

Editor’s note: The security aspects of supporting a split frontend / backend API invoker will be specified by SA3, including considerations for exchanging UE identity information when seeking resource owner authorization.

* * * End of Changes * * * *