3GPP TSG-SA WG6 Meeting #64	S6-245076

Orlando, USA, 18th – 22nd November 2024

Source:	InterDigital Inc.

Title:	Pseudo-CR on SM service discovery in SEAL service

Spec:	3GPP TS 23.437 v0.3.0

Agenda item:	9.11

Document for:	Approval

Contact:	michel.roy@interdigital.com

1. Introduction

This contribution proposes to resolve the following EN for SM service discovery:

Editor’s Note: Whether the SM service discovery procedure is defined in 3GPP TS 23.437 and whether CAPIF API discovery filters in 3GPP TS 23.222 [7] can be enhanced to support discovery of an API provider with specific spatial map coverage is FFS.

2. Reason for Change

It is proposed to define an SM service discovery procedure in 3GPP TS 23.437.

3. Conclusions

NA

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.437 v0.3.0.

* * * First Change * * * *

### 9.6.5	SM service discovery

#### 9.6.5.1	General

SM service discovery enables an authorized SM service consumer (VAL server or SEAL SM client) to discover VAL servers with SM capabilities.

#### 9.6.5.2	Procedure

Figure 9.6.5.2-1 depicts the procedure for SM service discovery.

Pre-conditions:

1.	The VAL server or SEAL SM client has received information (e.g. URI, IP address) related to the SEAL SM server;

2.	The VAL server or SEAL SM client has received security credentials authorizing it to communicate with the SEAL SM server.

Figure 9.6.5.2-1: SM service discovery procedure

1.	The VAL server or SEAL SM client sends a SM service discovery request message to the SEAL SM server to discover VAL servers with SM capabilities. The request includes a requestor ID, security credentials, three-dimensional area of interest or localization information to discover VAL servers with SM capabilities in the area of interest and optionally discovery filters as described in clause 9.6.5.3.1.

2.	The SEAL SM server authorizes the requestor and validates the request. If the request is authorized, the SEAL SM server then determines if any VAL server(s) registered with the SEAL SM server can offer spatial map(s) according to the requested area of interest.

3.	The SEAL SM server sends a SM service discovery response message to the requestor. If the SEAL SM server has determined at least one VAL server with SM capabilities registered with the SEAL SM server, the response includes an indication of success and information about VAL server(s) with SM capabilities. If the SEAL SM server cannot determine VAL server(s) with SM capabilities, the SEAL SM server sends a failure indication with a reason for failure.

NOTE: 	A requestor can obtain the VAL server APIs via CAPIF as specified in 3GPP TS 23.222 [7] if a CAPIF core function is deployed within the SM server to support publish and discovery of VAL server APIs.

#### 9.6.5.3	Information flows

##### 9.6.5.3.1	SM service discovery request

Table 9.6.5.3.1-1 describes information elements of the SM service discovery request sent from the VAL server or SEAL SM client to the SEAL SM server.

Table 9.6.5.3.1-1: SM service discovery request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | The identifier of the requestor (e.g., SEAL SM client, UE, VAL client, VAL server) |
| Security credentials | M | The security credentials of the requestor |
| Discovery filters | M | Characteristics of the requested spatial map. |
| > Area of interest | M | Three-dimensional area of interest of the requested spatial map. |

##### 9.6.5.3.2	SM service discovery response

Table 9.6.5.3.2-1 describes information elements of the SM service discovery response sent from the SEAL SM server to the VAL server or SEAL SM client.

Table 9.6.5.3.2-1: SM service discovery response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the discovery request was successful |
| > SM service profiles | M | The list of SM service profiles that can offer a spatial map as described in Table 9.6.2.1-2. |
| Failure response | O | Indicates that the request failed |
| > Cause | O | Indicates the cause of the failure |

* * * End of Changes * * * *