3GPP TSG-SA WG6 Meeting #64	S6-245094

Orlando (FL), USA, 18th – 22nd November 2024	(revision of S6-xxxxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.542 | CR | 0063 | rev |  | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update for clause 8.2.2.3 & 8.5.4.2.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | PINAPP |  |  |  |  |  | Date: |  |  | 2024-11-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1. Clause 8.5.4.2.1 PIN configuration with default and backup PEGCs, the PIN Client Profile information in step 1 & step 2 include UE location IE, but in 8.2.2.3 PIN client profile there is no UE location IE. 2. Clause 8.5.4.2.1, the PIN Client Profile information in step 2 include application client KPIs, application client schedule, but in 8.2.2.3 PIN client profile there are application KPIs, application schedule. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update clause 8.2.2.3 & 8.5.4.2.1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent IE within steps in procedure of PIN configuration with default and backup PEGCs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2.3, 8.5.4.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

8.2.2.3	PIN client profile

Table 8.2.2.3-1 describes the list of information elements that needs to be shared by the PIN element when requesting to join the PIN.

Table 8.2.2.3-1: PIN client profile

| Parameter Name | Status | Parameter Description |
| --- | --- | --- |
| PIN ID | M | The identifier of the PIN to which the PIN element wants to join |
| UE identifier | O | PIN Element or UE identifier |
| PIN client ID | M | The unique identity of the PIN client within PIN |
| Name of the device | O | Human-readable name of the device (i.e. door sensor, watch, smart TV, etc) along with manufacturer details |
| PINE role | O | Role of PINE: PEMC, PEGC, other |
| Application List | O | List of application identities |
| > Application Identity | O | Identity of the application |
| > Application schedule | O | Operational schedules of each application (e.g., time windows) which the |
| > Application KPIs | O | Minimum KPIs required by each application to operate effectively within the PIN (e.g., PIN bandwidth, PIN request rate, PIN response time) |
| capabilities | M | capabilities of the PIN client like whether it can be assigned with the role of PEMC or PEGC etc. |
| Visibility | M | Determines whether this PIN element is discoverable by other PIN elements within PIN, discoverable by other UEs outside the PIN etc., |
| access type | M | Access type supported for the communication |
| Layer-2 details | O | Layer-2 address of the PIN element |
| Required services | O | Identifies the list of services the PINE wants to consume |
| Supported services | O | Identifies the list of services the PINE is providing. |
| Port number | M | Port number of PIN client on PINE to support PIN enabler layer communication. NOTE: The port number is the port used by a PINE to expose a service within the PIN. |
| UE location | O | The location information of the UE. The UE location is described in clause 7.2.7. |

* * * Next Change * * * *

##### 8.5.4.2.1	PIN configuration with default and backup PEGCs

Figure 8.5.4.2.1-1 illustrates a procedure for configuring a new PIN Element, when requesting to join, with a default PEGC and one or more backup PEGCs.

Pre-conditions:

1.	The PEMC is pre-configured or has discovered the address (e.g. IP address, FQDN, URI) of the PIN server.

2.	The PEMC is authorized to access the PIN server.

3.	The PEMC has registered to the PIN server.

4.	The PEMC has requested and has been authorized by the PIN server to create a PIN.

5.	The required KPIs of the PIN have been configured.

6.	PINE-1, PINE-2, PEGC-A and PEGC-B have registered to the PEMC and PIN server.

7. 	PINE-1, PINE-2, PEGC-A and PEGC-B have subscribed with the PEMC for the PIN status as specified in clause 8.5.X.2.1.

Figure 8.5.4.2.2-1: Multiple PEGC configuration for a PIN

1.	PEGCs (e.g., PEGC-A and PEGC-B) shall join the PIN by sending PIN management PINE join into PIN requests to the PEMC. The PIN Client Profile information (supported PIN roles, supported PEGC KPIs, Supported PEGC schedule, UE location) shall be included in the requests. The PEMC shall use this information to determine whether multiple PEGCs are needed to meet the required KPIs of the PIN and which PIN elements (e.g., PEGC-A and PEGC-B) are able to serve as PEGCs. The PEMC shall configure multiple PEGCs (e.g., PEGC-A and PEGC-B) with a role of PEGC if needed. The PEMC shall then send a PIN profile update to the PIN server informing the PIN server that of the multiple PEGCs.

2.	When PINEs (e.g., PINE-1) send a PIN management PINE join into PIN request to the PEMC, PIN Client Profile information (application KPIs, application schedule, UE location) shall be included in the request. The PEMC shall use this information and the PEGC information the PEMC received in step 1 to determine the optimal default and backup PEGCs for the PINE. For example, PEMC assigns PEGC-A as the default PEGC for PINE-1, and PEGC-B as the backup PEGC for PINE-1.

3.	The PEMC shall send a default PEGC (e.g., PEGC-A) PIN status notification to update its local PIN Profile information to reflect that a PINE (e.g., PINE-1) is authorized to relay PIN communications via the PEGC.

4.	The PEMC shall send any backup PEGCs (e.g., PEGC-B) PIN status notification to update their local PIN Profile information to reflect that a PINE (e.g., PINE-1) is authorized to relay PIN communications via the PEGC.

5.	The PEMC shall return a PIN management PINE join into PIN response to PINE-1. The response shall include updated PIN client profile information informing the PINE that it shall use a specified PEGC (e.g., PEGC-A) as its default PEGC, and a specified PEGC (e.g., PEGC-B) as its backup PEGC.

6.	The PEMC shall send a PIN profile update to the PIN server informing the PIN server of a PEGC (e.g., PEGC-A) serving as the default PEGC for a PINE (e.g., PINE-1) and any PEGCs (e.g., PEGC-B) serving as backup PEGCs for a PINE.

7.	Steps 2 to 6 may be repeated for any additional PINEs joining the PIN. For example, based on PIN client profile information provided by PINE-2, the PEMC configures PEGC-B to serve as the default PEGC for PINE-2 and PEGC-A to serve as the backup PEGC for PINE-2.

* * * End of Change * * * *