3GPP TSG-SA WG6 Meeting #64	S6-245095

Orlando (FL), USA, 18th – 22nd November 2024	(revision of S6-xxxxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.286 | CR | 0084 | rev |  | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Solve EN for service continuity & dynamic group management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | V2XAPP_Ph3 |  |  |  |  |  | Date: |  |  | 2024-11-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1. EAS service continuity is already supported in EDGEAPP. 2. Dynamic group management is special for V2X, no need move to SEAL. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Clause 9.10.4, remove EN. 2. Clause 9.12.2.1 & 10.2.1, remove EN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EN remains in TS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.10.4, 9.12.2.1, 10.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

9.10.4	Dynamic local service information

9.10.4.1	General

For the V2X scenarios like extended sensors, platooning, the V2X UEs may be connected to the serving PLMN, but the V2X application server information may change as per the geographical location. Also it is not practically feasible to provide all the V2X server USDs to the V2X UE via the initial V2X UE configuration by the PLMN. It is hence required that the V2X server USDs may be provided considering the mobility of the V2X UE. This subclause describes the procedure for obtaining the local service information by the V2X UE via V1-AE.

9.10.4.2	Procedure

Figure 9.10.4.2-1 illustrates the procedure for obtaining the dynamic local service information by the V2X UE via V1-AE.

Pre-conditions:

1.	The V2X UE is connected to the VAE server in geographical area A.

2.	The V2X UE has no local service information for geographical area B.

3.	If multicast delivery mode is used, the MBMS bearer being used is activated by the VAE server.

Figure 9.10.4.2-1: Obtaining dynamic local service information by V2X UE via V1-AE

1.	The VAE client sends a local service information request to the VAE server (responsible for geographic location A) which may include mobility information like the geographic locations (geographic location B).

2.	The VAE server determines the local service information (e.g. V2X server USD(s), V2X USD) corresponding to the geographic locations information received in step 1.

3.	The VAE server provides the local service information (e.g. V2X server USD(s), V2X USD) with the corresponding geographic locations information to the VAE client via a local service information response. The V2X USD information consists of TMGI, list of SAIs, frequency and SDP information with the local service information for local V2X application server discovery. The details of V2X server USD are specified in subclause 4.4.7.3 in 3GPP TS 23.285 [5]. This message can be sent via unicast or multicast.

4.	Upon receiving the local service information in step 3, the VAE client stores the received information.

NOTE:	The dynamic local service information can be supported by EAS service continuity function in EDGEAPP, refer to clause 8.8 in 3GPP TS 23.558 [25].

* * * Next Change * * * *

9.12.2.1	Push Layer-2 group ID mapping

Table 9.12.2.1-1 describes the information flow push layer-2 group ID mapping from the VAE server to the VAE client and between the VAE clients.

Table 9.12.2.1-1: Push Layer-2 group ID mapping

| Information element | Status | Description |
| --- | --- | --- |
| Dynamic group information | M | Group information including dynamic group ID, group definition, group leader ID |
| ProSe Layer-2 Group ID | M | ProSe Layer-2 Group ID corresponding to the dynamic group information |

* * * Next Change * * * *

10.2.1	General

Table 10.2.1-1 illustrates the VAE server APIs.

Table 10.2.1-1: List of VAE server APIs

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| VAE_MessageDelivery API | Deliver_DL_Message | V2X application specific server | Request/ Response |
|  | Deliver_UL_Message | V2X application specific server | Subscribe/notify |
|  | V2X_MessageDelivery_Subscribe |  |  |
|  | V2X_MessageDelivery_Unsubscribe |  |  |
| VAE_FileDistribution API | Distribute_File | V2X application specific server | Request/ Response |
| VAE_ApplicationRequirement API | Reserve_NetworkResource | V2X application specific server | Subscribe/notify |
|  | Notify_NetworkResource |  |  |
| VAE_DynamicGroup API | Configure_DynamicGroup | V2X application specific server | Request/Response |
|  | Notify_DynamicGroup |  |  |
| VAE_ServiceContinuity API | Query_ServiceContinuity | VAE server | Request/Response |
| VAE_HDMapDynamicInfo API | Subscribe_HDMapDynamicInfo | V2X application specific server | Subscribe/notify |
|  | Notify_HDMapDynamicInfo |  |  |
| VAE_SessionOrientedService API | Establish_Session | V2X application specific server | Subscribe/notify |
|  | Notify_Establish_Session |  |  |
|  | Update_Session |  |  |
|  | Notify_Update_Session |  |  |
|  | Terminate_Session |  |  |
|  | Notify_Terminate_Session |  |  |
| VAE_PC5ProvisioningRequirement API | Config_PC5ProvisioningRequirement | V2X application specific server | Subscribe/notify |
|  | Notify_PC5ProvisioningRequirement |  |  |
| VAE_V2VConfigRequirement API | Request_V2VConfigRequirement | V2X application specific server | Request/Response |
| VAE_ServiceAdaptationQoSControlInfo API _ | Subscribe_ ServiceandQoSControlInfo | V2X application specific server | Subscribe/notify |
|  | Notify_ ServicandQoSControlInfo |  |  |
| VAE_ VRUzoneManagement API | Subscribe_VRUzoneManagement operation | V2X application specific server | Subscribe/notify |
|  | Notify_ Enter/Leave |  |  |
| VAE_V2PApplicationRequirement API | Request_ V2PApplicationRequirement | V2X application specific server, V2X application specific client | Request/Response |

The SEAL server(s) APIs available via SEAL-S reference point as specified in 3GPP TS 23.434 [6] are consumed by the V2X application specific server via the VAE server (acting as a VAL server) over the Vs reference point. When V2X application specific server invokes a SEAL server API via the VAE server over the Vs reference point, the VAE server shall interact with the corresponding SEAL server over the SEAL-S reference point for the API invocation request and response.

NOTE:	When V2X application specific server invokes a SEAL server API directly, the V2X application specific server acting as VAL server shall interact with the corresponding SEAL server over the SEAL-S reference point for the API invocation request and response as specified in 3GPP TS 23.434 [6].

* * * End of Change * * * *