3GPP TSG-SA WG6 Meeting #64	S6-245135

Orlando, USA, 18th – 22nd November 2024

Source:	ZTE Corporation

Title:	The alignment between ML model retrieval and ML model discovery

Spec:	3GPP TS 23.482 v0.3.0

Agenda item:	9.10

Document for:	Approval

Contact:	Yang Li (li.yang1226@zte.com.cn)

1. Introduction

This paper fixes the following Editor’s Notes.

Editor’s Note: The alignment between ML model retrieval request and ML model information discovery request is FFS Editor’s Note: The alignment between ML model retrieval response and ML model information discovery response is FFS Editor’s Note: The alignment between ML model retrieval request and ML model information discovery request is FFS. Editor’s Note: The alignment between ML model retrieval notification and ML model information discovery response is FFS Editor’s Note: The alignment between ML model retrieval subscription update request and ML model information discovery request is FFS

2. Reason for Change

To improve the readability of the specification, the alignment between ML model retrieval and ML model discovery is necessary, therefore, some modifications to the IEs related to these two procedures are required.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.482 v0.3.0.

8.2.3.1	ML model retrieval request

Table 8.2.3.1-1 describes information elements for the ML model retrieval request from the AIMLE client or VAL server to the AIMLE server.

Table 8.2.3.1-1: ML model retrieval request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., AIMLE client or VAL server). |
| Security credentials | M | The security credentials of the requestor. |
| Filtering criteria | O | Represents the filtering criteria, which can be any of the ML model information as in Table 8.11.4.1-2. |

* * * First Change * * * *

8.2.3.2	ML model retrieval response

Table 8.2.3.2-1 describes information elements for the ML model retrieval response from the AIMLE server.

Table 8.2.3.2-1: ML model retrieval response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the ML model retrieval request was successful. |
| List of ML models | O | List of retrieved ML models. This information element shall be provided if the ML model retrieval request was successful. |
| > ML model identifier | O | The ML model identifier. |
| > ML model retrieval endpoint | O (NOTE 1) (NOTE 2) | The endpoint (e.g., URL, URI, IP address) where the ML model file can be retrieved. This information element may be provided if the ML model retrieval request was successful. |
| > ML model | O (NOTE 1) (NOTE 2) | The ML model containing a retrieved ML model. This information element may be provided if the ML model retrieval request was successful. |
| Failure response | O | Indicates that the request has failed. |
| > Cause | O | The cause for the request failure. |
| NOTE 1: Only one of these information elements shall be provided. NOTE 2: For transfer learning, the list of all possible models matching the filtering criteria can be included along with model address. |  |  |

* * * Second Change * * * *

8.2.3.3	ML model retrieval subscribe request

Table 8.2.3.3-1 describes information elements for the ML model retrieval subscription request from the AIMLE client or VAL server to the AIMLE server.

Table 8.2.3.3-1: ML model retrieval subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., AIMLE client or VAL server). |
| Security credentials | M | The security credentials of the requestor. |
| Notification Target Address | M | Notification target address (e.g. URL) where the notifications should be sent. |
| Proposed expiration time | O | Proposed expiration time for the subscription. |
| Filtering criteria | O | Represents the filtering criteria, which can be any of the ML model information as in Table 8.11.4.1-2. |

* * * Third Change * * * *

8.2.3.5	ML model retrieval notification

Table 8.2.3.5-1 describes information elements for the ML model retrieval subscription response from the AIMLE server.

Table 8.2.3.5-1: ML model retrieval notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the subscription. |
| List of ML models | O | List of retrieved ML models. |
| > ML model identifier | O | The ML model identifier. |
| > ML model retrieval endpoint | O (NOTE 1) (NOTE 2) | The endpoint (e.g., URL, URI, IP address) where the ML model file can be retrieved. |
| > ML model | O (NOTE 1) (NOTE 2) | The ML model containing a retrieved ML model. |
| NOTE 1: Only one of these information elements shall be provided. NOTE 2: For transfer learning, the list of all possible models matching the filtering criteria can be included along with model address. |  |  |

* * * Forth Change * * * *

8.2.3.6	ML model retrieval subscription update request

Table 8.2.3.6-1 describes information elements for the ML model retrieval subscription update request from the AIMLE client or VAL server to the AIMLE server.

Table 8.2.3.6-1: ML model retrieval subscription update request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., AIMLE client or VAL server). |
| Security credentials | M | The security credentials of the requestor. |
| Subscription ID | M | Subscription identifier corresponding to the subscription to be updated. |
| Notification Target Address | O | Notification target address (e.g. URL) where the notifications should be sent. |
| Proposed expiration time | O | Proposed expiration time for the subscription. |
| Filtering criteria | O | Represents the filtering criteria, which can be any of the ML model information as in Table 8.11.4.1-2. |

* * * Fifth Change * * * *

8.11.4.3	ML model information discovery request

Table 8.11.4.3-1 describes the information flow from the AIMLE server to the ML repository as a request for the ML model information discovery.

Table 8.11.4.3-1: ML model information discovery request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the ML repository consumer performing the request. |
| Security credentials | M | The security credentials of the requestor. |
| Filtering criteria | M | Represents the filtering criteria, which can be any of the ML model information as in Table 8.11.4.1-2. |

* * * Fifth Change * * * *

8.11.4.4	ML model information discovery response

Table 8.11.4.4-1 describes the information flow from the model repository to the AIMLE server as a response for the ML model information discovery request.

Table 8.11.4.4-1: ML model information discovery response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE 3) | Indicates that the ML model information discovery request was successful. |
| List of ML models | O | List of discovered ML models. This information element shall be provided if the result is success. |
| > ML model retrieval endpoint | O (NOTE 1) (NOTE 2) | The endpoint (e.g., URL, URI, IP address) where the ML model file can be retrieved. This information element may be provided if the ML model information discovery request was successful. |
| > ML model | O (NOTE 1) (NOTE 2) | Represents the ML model(s). This information element may be provided if the ML model information discovery request was successful. |
| Failure response | O (NOTE 3) | Indicates that the ML model information discovery request has failed. |
| > Cause | O | The cause for the request failure. |
| NOTE 1: Only one of these information elements shall be provided. NOTE 2: For transfer learning, the list of all possible models matching the filtering criteria can be included along with model address. NOTE 3: At least one of these elements is needed. |  |  |

* * * End of Changes * * * *