3GPP TSG-SA WG6 Meeting #64	S6-245162

Orlando, USA, 18th – 22th Nov 2024	(revision of S6-24xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0346 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resvole the EN for Geofencing monitoring events |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eLSAPP |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This contribution main resolves the following EN: Editor's Note: Whether the Geofencing monitoring events and Triggering events can be merged or not is FFS. IE “Geofencing monitoring events” are different from the IE “trigger events”. - “Geofencing monitoring events” means the VAL server expects to monitor the Geofencing related events including the numbers/time duration of UEs moving in/out the given location area. - “trigger events” indicate when the Geofencing events will be reported to the VAL server. For example, the trigger events can be the maximum number(e.g.100) of UEs moving in the pre-defined location area. That means the LMS will be always monitoring the UEs entering the pre-defined location area and counting the numbers and when the number of UE is up to 100, it will trigger the notification report to the VAL server. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the EN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Geofencing events can’t be monitored in Location area monitoring subscription procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.2.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

9.3.2.14	Location area monitoring subscription request

Table 9.3.2.14-1 describes the information flow from the VAL server to the location management server for location area monitoring subscription request.

Table 9.3.2.14-1: Location information monitoring subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL server, VAL UE or SEAL Server |
| VAL service ID | O | Identity of the VAL service for which the VAL server requests (e.g.Geofencing). |
| Location Information criteria | M | Location information to be monitored. It includes the geographic location information or a VAL service area identified by the VAL service area ID or a reference UE along with the application defined proximity range from the reference UE. This information also applies for Geofencing service. |
| Geofencing monitoring events | O | It indicates the Geofencing events to be monitored. |
| > UEs moved in/out | O | It indicates to monitor the UE(s) moving in/out of the area as defined in location information criteria, |
| > Numbers of UEs moved in/out | O | Indicates the numbers of UE(s) moving in/out of the area as defined in location information criteria are to be monitored, |
| > Time of UEs moved in/out | O | Indicates the time stamp/time duration of UE(s) moving in/out of the area as defined in location information criteria is to be monitored, |
| Time between consecutive reports | O | It indicates the interval time between consecutive reports |
| Immediate Report Indicator | O | Indicates whether an immediate location report is required |
| Triggering events | O | Identifies when the server will send the notification (e.g. distance travelled, the maximum number of UEs moves in/out of the given location area) |
| Notification Target URI | M | Target URI where the VAL server wishes to receive the notifications about location area monitoring. |

* * * End of Changes * * * *