3GPP TSG-SA WG6 Meeting #64	S6-245178

Orlando, USA, 18th – 22nd November 2024	(revision of S6-245xxx)

Source:	Samsung

Title:	Pseudo-CR on Split operation procedures

Spec:	3GPP TS 23.482 (v0.3.0)

Agenda item:	9.10

Document for:	Approval

Contact:	Arunprasath R, arun.prasath@samsung.com

1. Introduction

This contribution provides split operation pipeline update and delete procedure.

2. Reason for Change

In previous meeting SA6 agreed to create Split operation pipeline instance along with following EN

Editor’s Note 1: The procedures for modifying and deleting a split operation pipeline instance are FFS.

It is required to remove the EN.

It is possible that the VAL client or AIMLE client who created the split operation pipeline may want to update the pipeline by adding new nodes or removing some nodes. Furhter, once split operation is completed, and if there is no need to keep the pipeline, the AIMLE client may decide to delete the pipeline instance.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.482 (v0.3.0).

* * * First Change * * * *

#### 8.14.2.3	Split operation pipeline creation

Figure 8.14.2.3-1 illustrates the procedure for an AIMLE client to create an instance of a split operation pipeline at the AIMLE server. If the required number of nodes are not discovered in using the discovery procedure (i.e., the number of nodes in the discovery response message are less than the required minimum number of nodes), then the AIML enablement client can discover the remaining required processing nodes from other AIMLE server(s).

Pre-conditions:

1.	The AIMLE client has received information (e.g. URI, IP address) related to the AIMLE Server;

2.	The AIMLE client has received security credentials authorizing it to communicate with the AIMLE server;

3.	The AIMLE client has discovered information about processing nodes available with the AIMLE server;

4.	The VAL client (along with the AIMLE client) selects the nodes, which can take part in the split operation and decides the order in which nodes should perform the split operation inference.

Figure 8.14.2.3-1: Split operation pipeline creation

1.	The AIMLE client sends a split operation pipeline create request to the AIMLE server. The request includes information defined in Table 8.14.3.4-1.

2.	Upon receiving the request from the AIMLE client, the AIMLE server validates if the requestor is authorized for the request. If the requestor is authorized, the AIMLE server validates if the requested split operation pipeline can be created based on the information included in the request.

If the requestor is authorized and the AIMLE server has determined that an instance of a split operation pipeline can be created, the AIMLE server creates a split operation profile along with its identifier and notifies the appropriate processing nodes indicated in the request about their inclusion in the split operation pipeline as described in clause 8.14.2.5.

3.	The AIMLE server sends a split operation pipeline create response message to the AIMLE client. If the AIMLE server has created an instance of a split operation pipeline, the response includes an indication of success, and the corresponding split operation profile. Otherwise, the response includes an indication of failure and may include a reason for failure.

Upon receiving a response including a split operation profile, the AIMLE client can store the split operation profile. The AIMLE client provides split operation profile information to VAL client(s). Split operation profile information can be used to access the split operation as described in clause 8.14.2.2.

Editor’s Note: Whether the procedures to create or update or delete a split operation pipeline can be performed by the VAL server is FFS.

* * * Next Change * * * *

#### 8.14.2.x	Split operation pipeline modification

Figure 8.14.2.x-1 illustrates the procedure for an AIMLE client to modify an instance of a split operation pipeline at the AIMLE server. The AIMLE client or VAL client determines that some of the nodes in the split operation pipeline may not be able to perform the required operation and decides to modify the pipeline either to add or remove the nodes.

Pre-conditions:

1.	The AIMLE client has received information related to split operation pipeline profile as specified in clause 8.14.2.3;

Figure 8.14.2.x-1: Split operation pipeline update

1.	The AIMLE client sends a split operation pipeline update request to the AIMLE server. The request includes information defined in Table 8.14.3.x-1.

2.	Upon receiving the request from the AIMLE client, the AIMLE server validates if the requestor is authorized for the request. If the requestor is authorized, the AIMLE server validates the pipeline identifier to be modified.

If the requestor is authorized and the pipeline is available, the AIMLE server updates the split operation pipeline profile and notifies the appropriate processing nodes indicated in the request about their inclusion or exclusion in the split operation pipeline as described in clause 8.14.2.5. The AIMLE server also notifies the already existing nodes about modification of the existing pipeline as described in clause 8.14.2.5.

3.	The AIMLE server sends a split operation pipeline update response message to the AIMLE client. If the AIMLE server has updated an instance of a split operation pipeline, the response includes an indication of success, and the corresponding split operation profile. Otherwise, the response includes an indication of failure and may include a reason for failure.

* * * Next Change * * * *

#### 8.14.2.a	Split operation pipeline delete

Figure 8.14.2.a-1 illustrates the procedure for an AIMLE client to delete an instance of a split operation pipeline at the AIMLE server. The AIMLE client or VAL client determines to delete the split operation pipeline as the split operation is complete or no longer needed.

Pre-conditions:

1.	The AIMLE client has received information related to split operation pipeline profile as specified in clause 8.14.2.3;

Figure 8.14.2.a-1: Split operation pipeline update

1.	The AIMLE client sends a split operation pipeline delete request to the AIMLE server. The request includes information defined in Table 8.14.3.b-1.

2.	Upon receiving the request from the AIMLE client, the AIMLE server validates if the requestor is authorized for the request. If the requestor is authorized, the AIMLE server validates the pipeline identifier to delete.

If the requestor is authorized and the pipeline is available, the AIMLE server deletes the split operation pipeline profile and notifies the appropriate processing nodes about deletion of the split operation pipeline as described in clause 8.14.2.5.

3.	The AIMLE server sends a split operation pipeline delete response message to the AIMLE client. If the AIMLE server has delete an instance of a split operation pipeline, the response includes an indication of success. Otherwise, the response includes an indication of failure and may include a reason for failure.

* * * Next Change * * * *

#### 8.14.3.4	Split operation pipeline create request

Table 8.14.3.4-1 shows the request sent by a AIMLE client to the AIMLE server for split operation pipeline create request.

Table 8.14.3.4-1: Split operation pipeline create request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identity of the requestor (e.g., VAL client ID, AIMLE client ID, UE identifier). |
| Security credentials | M | The security credentials of the requestor. |
| Split operation requirements | M | Split operation requirements. |
| > model information | M | Information about the ML models to be used in each stage (e.g., identifiers, versions, etc.). |
| > usage information | O | Information about the planned usage of the split operation (e.g., inputs frequency/size, output frequency/size, etc.). |
| > notification target | O | Endpoint information where the result of the split operation is sent by the nodes. |
| > stage information | M | Information about the split operation stages (e.g., number, order, etc.). |
| >> head node | M | Endpoint of the head node - for providing initial the inference data. |
| >> tail node | M | Endpoint of the head node - for obtaining inference results. |
| >> node information | M | List of Information about the nodes in order of the stage (e.g., identifier, endpoint, etc.). |

* * * Next Change * * * *

#### 8.14.3.3	Split operation pipeline discovery response

Table 8.14.3.3-1 shows the response sent by sent by the AIMLE server after processing of split operation pipeline discovery request.

Table 8.14.3.3-1: Split operation pipeline discovery response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the request was successful. |
| > List of nodes (NOTE) | O | The list of discovered nodes. |
| > List of split operation profiles (NOTE) | O | The list of split operation profiles as specified in Table 8.14.3.3-2. |
| Failure response | O | Indicates that the request failed. |
| > Failure cause | O | Indicates the failure cause. |
| NOTE: Only one of the IE shall be present for successful response. |  |  |

Table 8.14.3.3-2: Split operation profile

| Information element | Status | Description |
| --- | --- | --- |
| Split operation pipeline identifier | M | The identifier of the AI/ML split operation pipeline. |
| Head endpoint | M | The endpoint information of the head node for providing intermediate data. |
| Tail endpoint | M | The endpoint information of the tail node for obtaining results. |
| Usage information | O | The usage information of the AI/ML split operation (e.g., inputs frequency/size, output frequency/size, etc.). |
| Stage information | M | The list of stage(s) of the AI/ML split operation, the nested information is provided for each stage. |
| > Stage identifier | M | The identifier of the stage. |
| > number of nodes | O | Number of nodes included in the stage |
| > head node | M | Endpoint of the head node - for providing initial the inference data. |
| > tail node | M | Endpoint of the head node - for obtaining inference results. |
| > order of the nodes | M | The order of the nodes in the AI/ML split operation (including head node, tail node). |
| >> list of nodes | M | List of all discovered node in the order in which they process the data. |
| >> notification target | O | Endpoint information where the inference result of the split operation is sent by the nodes |
| > Model information | M | The ML model information used in the stage (e.g., identifiers, versions, etc.). |

* * * Next Change * * * *

#### 8.14.3.x	Split operation pipeline update request

Table 8.14.3.x-1 shows the request sent by an AIMLE client to the AIMLE server for split operation pipeline update request.

Table 8.14.3.x-1: Split operation pipeline update request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identity of the requestor (e.g., VAL client ID, AIMLE client ID, UE identifier). |
| Security credentials | M | The security credentials of the requestor. |
| Split operation pipeline identifier |  | The identifier of the AI/ML split operation pipeline. |
| Split operation requirements | M | Split operation requirements. |
| > Inclusion list | O | List of information about the nodes to be added in order |
| > exclusion list | O | List of information about the nodes to be removed from the pipeline |

#### 8.14.3.y	Split operation pipeline update response

Table 8.14.3.y-1 shows the response sent by an AIMLE server after processing of split operation pipeline update request.

Table 8.14.3.y-1: Split operation pipeline create response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the request was successful. |
| > Split operation profile | O | The split operation profile as specified in Table 8.14.3.3-2. |
| Failure response | O | Indicates that the request failed. |
| > Failure cause | O | Indicates the failure cause. |

#### 8.14.3.b	Split operation pipeline delete request

Table 8.14.3.b-1 shows the request sent by an AIMLE client to the AIMLE server for split operation pipeline delete request.

Table 8.14.3.b-1: Split operation pipeline delete request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identity of the requestor (e.g., VAL client ID, AIMLE client ID, UE identifier). |
| Security credentials | M | The security credentials of the requestor. |
| Split operation pipeline identifier |  | The identifier of the AI/ML split operation pipeline. |
| Split operation requirements | M | Split operation requirements. |
| > Inclusion list | O | List of information about the nodes to be added in order |
| > exclusion list | O | List of information about the nodes to be removed from the pipeline |

#### 8.14.3.c	Split operation pipeline delete response

Table 8.14.3.c-1 shows the response sent by an AIMLE server after processing of split operation pipeline delete request.

Table 8.14.3.c-1: Split operation pipeline delete response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the request was successful. |
| > Split operation profile | O | The split operation profile as specified in Table 8.14.3.3-2. |
| Failure response | O | Indicates that the request failed. |
| > Failure cause | O | Indicates the failure cause. |

* * * End of Change * * * *