3GPP TSG-SA WG6 Meeting #64	S6-245180

Orlando, USA, 18th – 22nd November 2024	(revision of S6-245xxx)

Source:	Samsung

Title:	Pseudo-CR on Split operation procedures

Spec:	3GPP TS 23.482 (v0.3.0)

Agenda item:	9.10

Document for:	Approval

Contact:	Arunprasath R, arun.prasath@samsung.com

1. Introduction

This contribution provides enhancements to subscirbe procedure.

2. Reason for Change

In previous meeting SA6 agreed to subscribe Split operation pipeline event along with following EN.

Editor’s Note: The "split operation pipeline information" event support for modifying and deleting a split operation pipeline instance can be added when the related procedures are defined.

The subscription is only for create event. It is reuqired for consumer to subscribe for events related to pipeline modification and removal.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.482 (v0.3.0).

* * * First Change * * * *

##### 8.14.2.5.2	Subscribe

Figure 8.14.2.5.2-1 illustrates the procedure for an AIMLE client or VAL server to subscribe with the AIMLE server to be notified of events related to split AI/ML operation.

Pre-conditions:

1.	The AIMLE client or VAL server has received information (e.g. URI, IP address) related to the AIMLE Server;

2.	The AIMLE client or VAL server has received security credentials authorizing it to communicate with the AIMLE server;

Figure 8.14.2.5.2-1: Split operation subscription

1.	The requestor (e.g., AIMLE client or VAL server) sends a split operation subscribe request to the AIMLE server. The request includes information defined in Table 8.14.3.12-1.

a.	The requestor (e.g., AIMLE client or VAL server) may include the "split operation pipeline information" event to indicate the AIMLE server to notify the requestor when a split operation profile is created, modified (e.g. new node is either added or removed, or change in stages, etc.) or removed and satisfy the discovery filters of the subscription request.

b.	The requestor (e.g., AIMLE client or VAL server) may include the " split operation assistance information" event to indicate the AIMLE server to notify the requestor when the AIMLE server determines assistance information related to the split operation usage.

2.	Upon receiving the request from the requestor, the AIMLE server validates if the requestor is authorized for the request. If the requestor is authorized, the AIMLE server creates the subscription and stores the subscription information.

3.	The AIMLE server sends a split operation subscribe response to the requestor. If the AIMLE server has created the subscription, the response includes an indication of success, the subscription identity and may include an expiration time; to maintain the subscription, the requestor shall send a subscription update request before the expiration time, otherwise the split operation subscription expires. If the AIMLE server has not created the subscription, the response includes an indication of failure and may include a reason for failure.

If the subscription is for "assistance information", the AIMLE Server considers the assistance information included in the subscription request and can perform the following:

The AIMLE Server aggregates the collected assistance information from NEF, NWDAF, and/or ADAES to generate assistance information, e.g. Time (time point(s) or time window(s)) to deliver the task or data for the split operation. Or the AIMLE server performs inference using ML model to generate assistance information, e.g. achievable QoS with current configuration for task or data delivery, or suggestion of QoS for task or data delivery.

-	For assistance information from NEF, the AIMLE Server sends a request to NEF for assistance information (e.g. PDTQ on recommended time windows for AI/ML operations with QoS, QoS monitoring). The request may include the information of the AI/ML task/ML model/data (e.g. size of the task or ML model, or data volume), information of the receiving nodes, requirement on the delivery/distribution (e.g. time budget, time critical or not, QoS), maximum time for complete the distribution/delivery.

Editor’s Note: Whether and what additional information needed in the request to NEF for the services is FFS and requires coordination with SA2.

-	For assistance information from NWDAF, the AIMLE Server can send a request or subscription request to NWDAF for analytics (e.g. E2E data volume transfer time, DN performance, Network performance, UE mobility). The details parameters in the requests to NWDAF for analytics are given in 3GPP TS 23.288 [2].

-	For assistance information from ADAES, the AIMLE Server can send a request or subscription request to ADAES for analytics (e.g. slice-specific application performance analytics, UE-to-UE application performance analytics). The details parameters in the requests to ADAES for analytics are given in 3GPP TS 23.436 [4].

##### 8.14.2.5.3	Notify

Figure 8.14.2.5.3-1 illustrates the split operation notify operation between the AIMLE server and an AIMLE client or VAL server.

Pre-conditions:

2.	The AIMLE client or VAL server has subscribed for split operation with the AIMLE Server;

Figure 8.14.2.5.3-1: Split operation notification

1.	The AIMLE server detects event(s) that satisfies trigger conditions for notifying a subscriber (e.g., AIMLE client or VAL server) according to subscribed events.

a.	If the subscribed event is for "split operation pipeline information" and the AIMLE server detects that a new split operation profile is created or an existing pipeline is modified (e.g. new node is added or removed or change in stage, etc,) or an existing pipeline is removed, and satisfy the discovery filters of the subscription, the AIMLE server notifies the subscriber (e.g., AIMLE client or VAL server) accordingly;

b.	If the subscribed event is for "split operation assistance information" and the AIMLE server has determined assistance information related to split operation usage, the AIMLE server notifies the subscribers (e.g., AIMLE client or VAL server) accordingly.

2.	The AIMLE server sends a split operation notification to the requestor indicating the event. The notification includes a subscription identity and may include a split operation profile(s), split operation node information, or split operation assistance information dependent on the associated event.

* * * Next Change * * * *

#### 8.14.3.12	Split operation subscribe request

Table 8.14.3.12-1 shows the request sent by an AIMLE client or a VAL server to the AIMLE server for split operation subscribe request.

Table 8.14.3.12-1: Split operation subscribe request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identity of the requestor (e.g., VAL server ID). |
| Security credentials | M | The security credentials of the requestor. |
| pipeline identifier | M | Identifier of the split operation pipeline for which the requestor is subscribing |
| Notification endpoint | M | The notification endpoint (e.g. URL/URI/IP address) where the notifications should be sent to. |
| Event identifier | M | The event identifier for the subscription: - Split operation pipeline information - Split operation assistance information |
| Discovery filters | O | The set of characteristics to determine matching split operation profiles or nodes (as detailed in Table 8.14.3.2-1). Applicable for "Split operation pipeline information". |
| Assistance information | O | The assistance information for the subscription: - Aggregate the collected assistance information from NEF, NWDAF, and/or ADAES to generate assistance information, e.g. Time (time point(s) or time window(s)) to deliver the task or data for the split operation. - Perform inference using ML model to generate assistance information, e.g. achievable QoS with current configuration for task or data delivery, or suggestion of QoS for task or data delivery. Applicable for "Split operation assistance information". |
| Expiration time | O | The proposed expiration time of the subscription. |

#### 8.14.3.14	Split operation notification

Table 8.14.3.14-1 shows the request sent by an AIMLE server to notify a subscriber of detected split operation events.

Table 8.14.3.14-1: Split operation notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription identifier | M | The identifier of the subscription. |
| Event identifier | M | The identifier for the detected event, either: - Split operation pipeline information - Split operation assistance information |
| Availability information | O | The split operation availability information. Applicable for "Split operation availability" event. |
| > List of split operation profiles | O | The list of newly available split operation profiles as specified in Table 8.14.3.3-2. |
| > List of nodes | O | The list of newly available nodes. |
| Pipeline information | O | The split operation pipeline information. Applicable for "Split operation pipeline information" event. |
| > Sub-event | O | Indicates one of the possible sub-event. - Created - Modified - Removed |
| > Split operation profile | O | The split operation profile that the VAL server participates to as specified in Table 8.14.3.3-2. |
| Assistance information | O | The split operation assistance information. Applicable for "Split operation assistance information" event. |
| > Delivery time | O | Time (time point(s) or time window(s)) to deliver the task or data for the split operation. |
| > Achievable QoS | O | The achievable QoS with current configuration for task or data delivery. |
| > Suggestion of QoS | O | The suggestion of QoS for task or data delivery. |

#### 8.14.3.15	Split operation subscribe update request

Table 8.14.3.15-1 shows the request sent by an AIMLE client or a VAL server to the AIMLE server for split operation subscription update request.

Table 8.14.3.15-1: Split operation subscription update request

| Information element | Status | Description |
| --- | --- | --- |
| Subscription identifier | M | The identifier of the subscription. |
| Security credentials | M | The security credentials of the requestor. |
| Notification endpoint | O | The notification endpoint (e.g. URL/URI/IP address) where the notifications should be sent to. |
| Event identifier | O | The event identifier for the subscription: - Split operation pipeline information - Split operation assistance information |
| Split operation discovey filters | O | The set of characteristics to determine matching split operation profiles or nodes (as detailed in Table 8.14.3.2-1). Applicable for "Split operation availability" event. |
| Expiration time | O | The proposed expiration time of the subscription. |

* * * Next Change * * * *

<Proposed change in revision marks>