3GPP TSG-SA WG6 Meeting #64	S6-245182

Orlando, USA, 18th – 22nd November 2024	(revision of S6-245xxx)

Source:	Samsung

Title:	Pseudo-CR on updates to the AIMLE client participation procedure

Spec:	3GPP TS 23.482 V0.3.0

Agenda item:	9.10

Document for:	Approval

Contact:	Arunprasath R, arun.prasath@samsung.com

1. Introduction

AIMLE client participation procedure would help the AIMLE server to determine if the AIMLE client is willing and available to participate in a particular AIML operation. We also have ML model training capability evaluation procedure in Clause 8.19 to determine if the client is capable enough to participate in the VFL.

2. Reason for Change

The procedures in 8.19 and 8.10 more or less serves the same purpose of determining whether the selected clients are capable and available to participate in the AIML operation. This pCR updates the AIML client participation procedure to add the missing parts from 8.19 so that same procedure can be re-used instead of defining redundnat procedures and APIs. Exposing more APIs serving the same purpose will be confusing to the consumers. In principle it is better to optimize the APIs exposed to the consumers.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.482 V0.3.0.

* * * First Change * * * *

## 8.10	AIMLE client participation and capability evaluation

### 8.10.1	General

The following clauses specify procedures, information flows, and APIs for AIMLE client participation. Through this the AIMLE server can validate with the selected AIMLE clients if they are capable and willing to participate in the requested AIML operation

### 8.10.2	Procedure

#### 8.10.2.1	AIMLE client participation and capability evaluation

Figure 8.10.2.1-1: AIMLE client participation

1.	An AIMLE server sends an AIMLE client participation request to an AIMLE client to verify participation and evaluate the capability of the AIMLE client(s) in AI/ML operation(s). The request includes information as described in Table 8.10.3.1-1.

2.	The AIMLE clients evaluate their capability and availability to join the AIML operation as requested in step 1. The AIMLE client acknowledge its willingness to be part of (or removed from) the AIMLE client set in an AIMLE client participation response sent to the AIMLE server. If the test task is provided in the request in step 1, the AIMLE clients run the test task to determine if it can participate in the requested AIML operation. The response includes information as described in Table 8.10.3.1-2.

### 8.10.3	Information flows

#### 8.10.3.1	AIMLE client participation request

Table 8.10.3.1-1 shows the request sent by the AIMLE server to each AIMLE client selected for AIMLE client participation procedure.

Table 8.10.3.1-1: AIMLE client participation request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identifier of the requestor. |
| AIMLE client set identifier | M | An identifier for the AIMLE client set. |
| Add/remove indicator | M | Indicator for adding/removing the AIMLE client to/from the AIMLE client set. |
| AI/ML model ID and model parameter(s) | M | Information about the AI/ML model (e.g. model ID) and model parameters to use for the AI/ML operation. |
| AI/ML operations | M | A list of AI/ML operations (e.g., training) required to be performed. |
| Operational schedule | O | Schedule for the AI/ML operations. |
| Test task | O | The task for test ML model training capability. |
| Dataset requirement | E | Dataset requirements for the AI/ML operations. |
| >Dataset identifier | M | Identifer of the dataset |
| >Feature ID(s) | O (NOTE 1) | Identifier(s) of the required feature of the dataset from the UE |
| >Domain(s) | O (NOTE 1) | Domain(s) of the datasets at the UE |
| >Data source | O (NOTE 2) | Data source for the FL training |
| Service requirement | M | Information about the required Service ID and its service permission level (e.g., premium resource usage standard resource usage, limited resource usage) |
| NOTE 1 : If requirement on dataset is provided, atleast one of these IEs shall be present if the AIML operation is VFL NOTE 2: IF requirement on dataset is provided, the IE shall be present if the AIML operation is HFL |  |  |

#### 8.10.3.2	AIMLE client participation response

Table 8.10.3.2-1 shows the response sent by the AIMLE clients to the AIMLE server for the AIMLE client participation procedure.

Table 8.10.3.2-1: AIMLE client participation response

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identifier of the requestor. |
| Acknowledge | M | An indicator to acknowledge whether the AIMLE client accepts being added to the AIMLE client set. |
| Test result | O | The test result if the test task is provided in the request. This shall be present if the AIMLE client is acknowledging to participate in the AIML operation. |

* * * Next Change * * * *

* * * Next Change * * * *