3GPP TSG-SA WG6 Meeting #64	S6-245203

Orlando, USA, 18th – 22nd November 2024	(revision of S6-245xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0229 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Service API activation and instantiation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3 |  |  |  |  |  | Date: |  |  | 2024-11-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Base on the conclusion, the Solution#6 and solution#7 is concluded as candicate solution to normative work. The service API may be dynamically instantiated in different cases, e.g., triggered by service API discovery from a particular API invoker, or due the statistics of service API discovery failure with no service API is found, or due to the service API is overload. This CR is proposed to introduce the dynamic API instantiation as a standalone procedure to the TS 23.222, and update the other related procedures accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the API publish procedure and the API discover procedure. Introduce a new dynamic API instantiation procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Dynamic service API activation is not supported by CAPIF |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2, 8.3.2.1, 8.7.2.2, 8.7.2.aa, 8.7.2.bb, 8.7.3, new {8.aa, 8.aa.1, 8.aa.2, 8.aa.3} |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /**************************** First changes **********************/

4.2.2	Requirements

[AR-4.2.2-a] The CAPIF shall provide a mechanism to publish the service API information to be used by the API invokers to discover and subsequently invoke the service API.

[AR-4.2.2-b] The CAPIF shall provide a mechanism for the API invokers to discover the published service API information as specified in [AR-4.2.2-a] according to the API invokers' interest.

[AR-4.2.2-c] The CAPIF shall provide a mechanism to restrict the discovery of the published service API information by the API invokers, based on configured policies.

[AR-4.2.2-d] The CAPIF shall provide a mechanism to configure policies to restrict the discovery of the published service API information.

[AR-4.2.2-e] The CAPIF shall provide mechanism to support Serving Area Information related to service APIs.

[AR-4.2.2-g] The CAPIF shall provide mechanism to support publish the service API which is not instantiated.

# /**************************** Next changes **********************/

8.3.2.1	Service API publish request

Table 8.3.2.1-1 describes the information flow service API publish request from the API publishing function to the CAPIF core function.

Table 8.3.2.1-1: Service API publish request

| Information element | Status | Description |
| --- | --- | --- |
| API publisher information | M | The information of the API publisher may include identity, authentication and authorization information |
| Service API information | M (see NOTE 1) | The service API information includes the service API name, API provider name (optional), List of public IP ranges of UEs (optional), service API category (e.g. V2X, IoT), service API status (e.g. active, inactive), communication type, description, Serving Area Information (optional), AEF location (optional), interface details (e.g. IP address, port number, URI), protocols, version numbers, data format, Service KPIs (optional), and Network Slice Info (optional). |
| Shareable information | O (see NOTE 2) | Indicates whether the service API information or the service API category can be published to other CCFs. And if sharing, a list of CAPIF provider domain information where the service API information or the service API category can be published is contained. |
| NOTE 1: The interface details shall be not present, if the API status is inactive. NOTE: If the shareable information is not present, the service API information is not allowed to be shared. |  |  |

The Service KPIs is defined as below:

Table 8.3.2.1-2: Service KPIs

| Information element | Status | Description |
| --- | --- | --- |
| Maximum Request rate | O | Maximum request rate from the API Invoker supported by the server. |
| Maximum Response time | O | The maximum response time advertised for the API Invoker's service requests. |
| Availability | O | Advertised percentage of time the server is available for the API Invoker's use. |
| Available Compute | O | The maximum compute resource available for the API Invoker. |
| Available Graphical Compute | O | The maximum graphical compute resource available for the API Invoker. |
| Available Memory | O | The maximum memory resource available for the API Invoker. |
| Available Storage | O | The maximum storage resource available for the API Invoker. |
| Connection Bandwidth | O | The connection bandwidth in Kbit/s advertised for the API Invoker's use. |

# /**************************** Next changes **********************/

8.7.2.2	Service API discover response

Table 8.7.2.2-1 describes the information flow service API discover response from the CAPIF core function to the API invoker.

Table 8.7.2.2-1: Service API discover response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of the discovery of the service API information |
| Service API information (see NOTE 2) | O (see NOTE 1) | List of service APIs corresponding to the request, including service API information such as service API name, API provider name (optional), service API category, communication type, description, Serving Area Information (optional), interface details (e.g. IP address, port number, URI) (NOTE 3), protocols, version, data format, Service KPIs (optional), and Network Slice Info (optional). |
| CAPIF core function identity information | O (see NOTE 1) | Indicates the CAPIF core function serving the service API category provided in the query criteria |
| NOTE 1: The service API information or the CAPIF core function identity information or both shall be present if the Result information element indicates that the service API discover operation is successful. Otherwise both shall not be present. NOTE 2: If topology hiding is enabled for the service API, the interface details shall be the interface details of AEF acting as service communication entry point for the service API. NOTE: The interface details (e.g. IP address, port number, URI) and service API status shall not present, if the AEF status is instantiable but not yet instantiated. |  |  |

8.7.2.aa	Service API activate request

Table 8.7.2.aa-1 describes the information flow service API activate request from the CAPIF core function to the API management function.

Table 8.7.2.aa-1: Service API activate request

| Information element | Status | Description |
| --- | --- | --- |
| Service API identification(s) | M | The identification(s) of the service API(s). |
| Active / inactive indication | M | Request the service API to be activated or de-activated. |

8.7.2.bb	Service API activate response

Table 8.7.2.bb-1 describes the information flow service API activate response from the API management function to the CAPIF core function.

Table 8.7.2.bb-1: Service API activate response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of the activation of the service API. |

8.7.3	Procedure

Figure 8.7.3-1 illustrates the procedure for discover service APIs.

The service API discovery mechanism is supported by the CAPIF core function.

Pre-conditions:

1.	The API invoker is onboarded and has received an API invoker identity.

2.	The CAPIF core function is configured with a discovery policy information (e.g. to restrict discovery to category of APIs) for API invoker(s).

3.	The CAPIF core function obtains the service API information with the corresponding API status through service API publish procedure.

4.	The CAPIF core function has obtained the AEF status from the management function (e.g. API management function), the AEF status may include instantiated, and instantiable but not yet instantiated.

Figure 8.7.3-1: Discover service APIs

1.	The API invoker sends a service API discover request to the CAPIF core function. It includes the API invoker identity and query information.

2.	Upon receiving the service API discover request, the CAPIF core function verifies the identity of the API invoker (via authentication). The CAPIF core function retrieves the stored service API(s) information from the CAPIF core function (API registry) as per the query information in the service API discover request. Further, the CAPIF core function applies the discovery policy and performs filtering of service APIs information retrieved from the CAPIF core function.

3.	If the service API is not available due to AEF status is instantiable but not be instantiated, the CAPIF core function may request the service API management system (e.g. OAM system) to instantiate the related AEF instance. After the AEF is instantiated, the CAPIF core function obtains the service API information including the API interface details (e.g. IP address, port number, URI).

4.	If the service API status is inactive, the CAPIF core function sends service API activation request to the API management function to enable the service API (step 4a). the API management function sends service API activation response to the CAPIF core function (step 4b).  After the service API status is changed to active, the API publish function update the service API information towards the CAPIF core function via the service API publish update procedure (step 4c).

5.	The CAPIF core function sends a service API discover response to the API invoker with the list of service API information for which the API invoker has the required authorization.

# /**************************** Next changes **********************/

8.aa	Dynamic service API instantiation

8.aa.1	General

The service API may be dynamically instantiated in different cases, e.g., triggered by service API discovery from a particular API invoker, or due the statistics of service API discovery failure with no service API is found, or due to the existing service API instance is overload.

8.aa.2	Information flows

NOTE:	The instantiation information flow is specified by SA5.

8.aa.3	Procedure

Figure 8.aa.3-1 illustrates the procedure for dynamic service API or AEF instantiation.

Figure 8.aa.3-1: Procedure for dynamic service API instantiation

1.	The triggers at the CAPIF core function or the API management is met, e.g., the CAPIF core function receives service API discovery request but failing to retrieve service API, the API management function detects the service API instance is overload. The CAPIF core function or the API management function sends the service API instantiate request to the service API management system.

2.	The service API management system performs authorization check. If successful, it initiates the service API instantiation or AEF instantiation.

3.	After the service API instantiation or AEF instantiation is complete, the service API management system sends the service API instantiate response to sending entity (i.e., either the CAPIF core function or the API management function).

NOTE:	The detailed service API instantiate request and service API instantiate response are defined by SA5.