3GPP TSG-SA WG6 Meeting #64	S6-245263

Orlando, USA, 18th – 22nd November 2024	(revision of S6-245xxx)

Source:	Convida Wireless LLC

Title:	Add AIMLE client registration update procedure

Spec:	3GPP TS 23.482 V0.3.0

Agenda item:	9.10

Document for:	Approval

Contact:	Ly.Quang@convidawireless.com, Liu.Lu@convidawireless.com

1. Introduction

This contribution adds AIMLE client registration update procedure in TS 23.482.

2. Reason for Change

This contribution adds a registration update procedure to the AIMLE client registration procedure, which can be used by the AIMLE client to provide updated information (e.g. updated AIMLE client profile) to the AIMLE server. Accordinaly, an AIMLE client registration ID is added to the registration response, which can provide functionality equivalent to an AIMLE client profile ID. Therefore, it is suggested to remove the following Editor’s Note:

Editor’s Note: The addition of AIMLE client profile ID is FFS.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.482 V0.3.0.

* * * First Change * * * *

## 8.7	AIMLE client registration

### 8.7.1	General

Prior to participation in AI/ML operations, AI/ML capable UEs register to an AIMLE server and provide an AIMLE client profile and optionally a list of supported services. The AIMLE server uses information provided by the AIMLE client to discover and select suitable AIMLE clients for requested AI/ML operations.

### 8.7.2	Procedures

### 8.7.2.1	AIMLE client registration

The following clauses specify procedures, information flows, and APIs for AIMLE client registration.

Pre-conditions:

1.	The AIMLE client has been pre-configured or has discovered the address (e.g., URI) of the AIMLE server.

2.	The AIMLE client has been pre-configured with an AIMLE client profile.

Figure 8.7.2-1: AIMLE client registration

1.	The AIMLE client sends an AIMLE client registration request to the AIMLE server, the registration request includes information as described in Table 8.7.3.1-1. The AIMLE client indicates in the registration request its AI/ML capabilities such as supported ML model types and supported AI/ML operations, supported AI/ML client task capabilities with compute and task performance capabilities to assist with performing AIMLE client discovery and AIMLE client selection.

2.	The AIMLE server validates the registration request and performs an authentication and authorization check to determine if the AIMLE client is permitted to register to the AIMLE server and participate in AI/ML operations. The AIMLE server saves the context of the AIMLE client registration in the ML repository.

3.	The AIMLE server returns an AIMLE client registration response to the AIMLE client with the status of the request.

* * * Next Change * * * *

### 8.7.2.2	AIMLE client registration update

The following clauses specify procedures, information flows, and APIs for AIMLE client registration update.

Pre-conditions:

1.	The AIMLE client has registered AIMLE client profile and was provided with an AIMLE client registration identifier.

Figure 8.7.a-1: AIMLE client registration update

1.	The AIMLE client sends an AIMLE client registration update request to the AIMLE server, the registration request includes information as described in Table 8.7.3.x-1.

2.	The AIMLE server validates the registration update request. The AIMLE server updates the context of the AIMLE client registration.

3.	The AIMLE server returns an AIMLE client registration update response to the AIMLE client with the status of the request as described in Table 8.7.3.y-1.

* * * Next Change * * * *

### 8.7.3	Information flows

#### 8.7.3.1	AIMLE client registration request

Table 8.7.3.1-1 shows the request sent by an AIMLE client to an AIMLE server for the AIMLE client registration procedure.

Table 8.7.3.1-1: AIMLE client registration request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| AIMLE client profile | M | Information about the capability of the AIMLE client to support AI/ML operations as detailed in Table 8.7.3.1-2. |
| List of supported services | O | Supported service information. |
| > List of service ID(s) | O | List of service ID(s) that the AIMLE client supports |
| > Corresponding service permission level(s) | O | Service permission level(s) (e.g., premium resource usage, standard resource usage, limited resource usage). |

Table 8.7.3.1-2: AIMLE client profile

| Information element | Status | Description |
| --- | --- | --- |
| Supported AI/ML model types | O | AI/ML model types supported by the AIMLE client (e.g., decision trees, linear regression, neural networks). |
| Supported AI/ML operations | M | AI/ML operations supported by the AIMLE client such as training, model transfer, model inference, model offload, model split). |
| AIMLE client schedule | O | Indicates the availability schedule of the AIMLE client for the AIML service. |
| Application layer AIMLE client capabilities | M | Application layer AIMLE client capability information (e.g. ML application type, allowed resource usage level). |
| Dataset availability | O | Dataset availability such as dataset size, age, list of dataset features, and dataset identifiers. |
| Data capabilities | O | A list of data capabilities such as the type of data that can be collected (e.g. raw data), supported data processing capabilities (e.g. processed data), and supported exploratory data analysis functions. |
| AIML client task capability | O See NOTE | Indicates the AIML task performing capabilities. It includes compute capabilities (e.g., high, low), task performance preference capabilities (e.g., Green task, energy-efficient, low costs) |
| NOTE: The Green and Energy-efficient task performance may not be applicable to a UE. |  |  |

#### 8.7.3.2	AIMLE client registration response

Table 8.7.3.2-1 shows the response sent by the AIMLE server to the AIMLE client for the AIMLE client registration procedure.

Table 8.7.3.2-1: AIMLE client registration response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the request: success or fail. |
| AIMLE client registration ID | M | An identifier for the AIMLE registration. |

* * * Next Change * * * *

#### 8.7.3.x	AIMLE client registration update request

Table 8.7.3.x-1 shows the request sent by an AIMLE client to an AIMLE server for the AIMLE client registration update procedure.

Table 8.7.3.x-1: AIMLE client registration update request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| AIMLE client registration ID | M | An identifier for the AIMLE client registration. |
| Updated AIMLE client profile | M | Updated AIMLE client profile as detailed in Table 8.7.3.1-2. |
| Updated list of supported services | O | Updated list of service ID(s) the AIMLE client supports, as detailed in Table 8.7.3.1-1. |

#### 8.7.3.y	AIMLE client registration update response

Table 8.7.3.y-1 shows the response sent by the AIMLE server to the AIMLE client for the AIMLE client registration update procedure.

Table 8.7.3.y-1: AIMLE client registration update response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the request: success or fail. |
| Updated registration information | O | The information that was updated with the request. |

* * * End of Changes * * * *