3GPP TSG-SA WG6 Meeting #64	S6-245280

Orlando, USA, 18th – 22nd November 2024	(revision of S6-245xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0233 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resource owner consent upon service API invocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3 |  |  |  |  |  | Date: |  |  | 2024-11-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As per the agreed pCR (S6-244719), the solution #20 in TR 23.700-22 was concluded as way forward for key issue #1. It was concluded to enhance the procedure to obtain resource owner consent upon service API invocation at the AEF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Procedure in clause 8.16 is enhanced with solution #20 captured in TR 23.700-22. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The study conclusions to handle resource owner consent upon service API invocation are not specified in normative specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.16.2.2, 8.16.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * * *

#### 8.16.2.2	Service API invocation response

The information flow service API invocation response from the AEF to the API invoker is service API specific and the complete detail of the service API invocation response is out of scope of the present document. Table 8.16.2.2-1 describes only the CAPIF related information elements which are included in the service API invocation response.

Table 8.16.2.2-1: Service API invocation response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of service API invocation. |
| Authorization information with resource owner consent required | O | This information element may be present when the result is failure. Indicates that the required authorization information with resource owner consent is required. |
| Resource owner(s) | O | List of resource owner identifiers for which the resource owner consent is required. |
| CCF information | O | Information related to the CCF. For example, CCF URL, Identifier of the CCF, etc. If this information is not present, the API invoker may contact the CCF that it is configured with. |

* * * Next Change * * * *

### 8.16.3	Procedure

Figure 8.16.3-1 illustrates the procedure for API invoker authorization to access service APIs.

Pre-conditions:

1.	The API invoker has been authenticated.

2.	The API invoker associated authorization information is available at AEF.

Figure 8.16.3-1: Procedure for API invoker authorization to access service APIs

1.	The API invoker triggers service API invocation request to the AEF, including the service API to be invoked.

NOTE 1:	Authentication can also be performed if not authenticated previously.

NOTE 2:	The API invoker can trigger several service API invocations asynchronously.

2.	Upon receiving the service API invocation request, the AEF checks whether the API invoker is authorized to invoke that service API, based on the authorization information. The AEF determines that the service API invocation needs authorization information with resource owner consent.

2a.	If the AEF does not have information required to authorize service API invocation, the AEF obtains the authorization information from the CAPIF core function.

2b. If the AEF does not have the required resource owner consent information to authorize service API invocation, then the AEF responds to API Invoker with failure status message indicating that the authorization information with resource owner consent is not available for resource owner (s) in the service API request. The information included in the response message is shown in Table 8.16.2.2-1.

2c. Based on the information received from API exposing function, the API Invoker obtains the authorization information with resource owner consent for the required resource owners, using the procedure in clause 8.31.

3.	The API invoker triggers service API invocation request to the AEF with the authorization information with resource owner consent information.

4.	The API exposing function checks whether the API invoker is authorized to invoke that service API, based on the authorization information and the resource owner consent information.

5.	The AEF executes the service logic for the invoked service API.

6.	The API invoker receives the service API invocation response as a result of the service API invocation.

* * * End of Changes * * * *