3GPP TSG-SA WG6 Meeting #64	S6-245313

Orlando, USA, 18th – 22nd November 2024	(revision of S6-245xxx)

Source:	Lenovo

Title:	Resolve ENs in FL registration and events notification

Spec:	3GPP TS 23.482 v0.3.0

Agenda item:	9.10

Document for:	Approval

Contact:	Emmanouil Pateromichelakis <epateromiche@lenovo.com>

1. Introduction

This paper resolves the ENs in clause 8.4 and 8.5 on registration and events notification for an FL process.

2. Reason for Change

This paper resolves the ENs in clause 8.4 and 8.5 on registration and events notification for an FL process.

In particular, since ML repository cannot directly interact with VAL server, we update the text to cover that VAL server can indirectly be the consumer of the MLR services. In 8.5, further event type is introduced and the subscriber is mentioned to also be VAL server (indirectly via AIMLE server).

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.482 v0.3.0.

* * * First Change * * * *

## 8.4	Support for FL member registration

### 8.4.1	General

This clause provides the procedures for the registration and registration update of candidate FL members in the ML repository which serves as service registry for the FL members undertaking a task related to the ML model lifecycle, such as ML model local training. Candidate FL members can be application layer entities at the server side (e.g., VAL server, AIMLE server), which can potentially be selected as FL clients or FL server for a particular ASP /vertical requirement. If VAL server is the candidate FL member, he registers indirectly via AIMLE server to the ML repository.

* * * Next Change * * * *

## 8.5	Support for FL events subscription and notification

### 8.5.1	General

This clause consists of the following procedures 1) the subscription for the FL related events in clause 8.5.2 and 2) the event notification procedure in clause 8.5.3.

This capability targets the enhancements to a global ML repository acting as AIML service registry which monitors the status and changes to the availability and capabilities of the FL members. The subscriber can be the AIMLE server or VAL server which requires to get notified on the FL member availability, since this information is vital for enabling the subscriber to perform the FL-related AIML operations (e.g. AIMLE server selecting FL members for a certain FL task). If VAL server is the subscriber, he registers indirectly via AIMLE server for FL-related events.

### 8.5.2	Procedure on subscription for FL related events

This procedure, as illustrated in Figure 8.5.2-1, describes the subscription for events related to FL member availability.

Pre-conditions:

- The AIMLE server has the authorization to subscribe for the FL-related events (events described in clause 8.5.4).

Figure 8.5.2-1: Procedure for FL-related event subscription

1.	The subscriber (AIMLE server or VAL server) sends an FL-related event subscription request to the ML repository to receive notification of FL related events and in particular the availability of FL member for a target area and time.

2.	Upon receiving the event subscription request from the AIMLE server, the ML repository checks for the relevant authorization for the event subscription.

3.	If the authorization is successful, the ML repository stores the subscription information.

4.	The ML repository sends an FL-related event subscription response to subscriber indicating successful operation.

### 8.5.3	Procedure on FL related event notification

This procedure describes the event notification procedure for the FL member availability. This can be triggered based on a change at the availability or capabilities of the candidate or selected FL members.

Pre-conditions:

- The subscription procedure in clause 8.5.2 was performed by the AIMLE server or VAL server (via AIMLE server).

- The FL member has registered its capabilities or availability to the ML repository based on the support for FL registration capability.

Figure 8.5.3-1: Procedure for FL-related event notifications

1.	The ML repository detects a change related to the availability of an FL member and generates events to be consumed by the AIMLE server, based on the notification, or based on a received updated registration from FL member, who has already registered at the ML repository.

2.	For the generated event, the ML repository retrieves the list of corresponding subscriptions.

3.	The ML repository sends FL-related event notification to all entities (e.g., AIMLE servers) that have subscribed for the FL-related event matching the criteria. If a notification reception information is available as part of the FL-related event subscription, then the notification reception information is used by the ML repository to send event notifications to the corresponding AIMLE servers.

4.	The notified subscriber (s) send an event notification acknowledgement to the ML repository.

### 8.5.4	Definition of FL-related Events

The definition of the FL related events is provided in Table 8.5.4-1.

Table 8.5.4-1: Definition of FL related events

| Events | Events Description |
| --- | --- |
| Availability changes of FL member | The event type relates to the availability change of a FL member. Such availability change can be an indication of availability or unavailability for a given service area, or for a given ML model ID/profile. |
| FL model information | Information such as accuracy, time schedule and latency for the FL training. Latency is considering the target latency, i.e., when the FL model training shall be completed. |
| FL member load / energy status | The event type relates to the monitoring of the computational load or energy status for the requested FL member. Such load can be the estimated (based on measurements) or expected/predicted. This event may be triggered upon reaching a threshold for high load or energy consumption or on-demand or can be monitored periodically. |

* * * Next Change * * * *