3GPP TSG-SA WG6 Meeting #64	S6-245314

Orlando, USA, 18th – 22nd November 2024	(revision of S6-245xxx)

Source:	Lenovo

Title:	Support for ML model performance monitoring

Spec:	3GPP TS 23.482 v0.3.0

Agenda item:	9.10

Document for:	Approval

Contact:	Emmanouil Pateromichelakis <epateromiche@lenovo.com>

1. Introduction

This paper proposes a new capability in AIMLE for monitoring the ML model performance.

2. Reason for Change

In TR 23.700-82, Solution #12 provided a procedure to support AI/ML model lifecycle management for ML model re-training and update (e.g., for transfer learning) if requested by the AIMLE consumer when model performance degradation is observed. In this feature, one necessary step was the detection of the ML model degradation.

In this procedure, the AIML enablement consumer which could be a VAL server or ADAES detected the ML model performance degradation and was triggering the model update (re-training).One aspect that was kept open for the normative phase is how the ADAES as AIML enablement consumer is capable of detecting model performance degradation and this is supposed to be revisited during normative phase (according to NOTE in clause 8.12.2).

In this direction, one key aspect which needs to be covered is how to detect a model performance degradation and how to support the correct action to avoid issues in case it is the fault of the model itself and new model is required for the target ML task (e.g. analytics task or VAL-triggered ML task). Hence, it is still not specified how the monitoring of model performance happens and what is the impact with respect to the target ML task (covering also the TL case where the model is going to be used for more than one tasks).

This paper proposes a new capability for monitoring and detecting ML model degradation on AIMLE server.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.482 v0.3.0.

* * * First Change * * * *

## 6.x	Support for ML model performance monitoring

This functionality covers a new capability of AIMLE server for monitoring detecting a degradation relation to an ML operation / analytics operation and translating to an ML model degradation (expected or predicted) and performing an action to alleviate this issue (new model training or re-training).

* * * Next Change * * * *

## 8.x	ML model performance monitoring

### 8.x.1	General

The following clauses specify procedures, information flows, and APIs for ML model performance monitoring and potential degradation detection.

### 8.x.2	Procedure

Pre-conditions:

1.	One or more AIMLE services (at the AIMLE server or clients) using the given ML model are ongoing.

Figure 8.x.2-1: ML model performance monitoring

1.	The VAL server sends an ML model performance monitoring subscription request to AIMLE server, requesting to assist in monitoring the ML model performance. This request consists of ML model information and optionally the AIML service information, etc.

2.	The AIMLE server checks whether the VAL server is authorized to perform the ML model performance monitoring request.

3.	If the VAL server is authorized, AIME server returns the success response, otherwise a failure response indication the reason for failure.

4.	The AIMLE server identifies the the AIMLE services which are utilizing the requested ML model. Such AIMLE services can be an ML model training service or an HFL service at the AIMLE server or clients.

NOTE: The identification of the AIMLE services may be performed via fetching ML model information from the ML repository using ML model management procedure as in clause 8.11.3.

Then, the AIMLE server then starts monitoring the AIMLE service performance (e.g. accuracy, a KPI or QoS metric related to the AIML operation). This step includes receiving information from one or more AIMLE clients performing an operation based on the target ML model, with an expected or experienced deviation of the required performance of the AIMLE service.

5.	The AIMLE server detects an expected ML model degradation based on the deviation of the performance of the AIMLE service as indicated in step 4.

6.	The AIMLE server based on the expected ML model degradation, it may also indicate and execute a trigger action to ensure meeting the AIMLE service requirement.

Such trigger action may be either an adaptation of the AIMLE service, such as training of a new ML model for the AIMLE by the same or a different AIMLE client, or re-training of the ML model by the same or different AIMLE client; or termination of the AIMLE service and initiating a new AIMLE service with a new ML model.

7.	The AIMLE server notifies the VAL server on the expected ML model degradation and if requested the triggered adaptation of the AIMLE service.

### 8.x.3	Information flows

#### 8.x.3.1	ML model perf monitoring subscription request

Table 8.x.3.1-1 shows the request sent by a VAL server to an AIMLE server for the ML model perf monitoring.

Table 8.x.3.1-1: ML model perf monitoring subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identifier of the requestor (e.g., VAL server). |
| ML model identifier | M | The identifier of the ML model for which the monitoring applies |
| AIML operation information | O | The AIMLE operation (ML model training, HFL, VFL, TL) for which the ML model is used. |
| > AIMLE service ID | O | The identifier of the AIMLE service using the ML model (if known by the requestor). |
| > AIMLE client ID(s) | O | The identifier(s) of the AIMLE client(s) training the ML model (if known by the requestor). |
| > AIMLE service KPI | O | One or more KPIs for the AIMLE service performance (latency, accuracy, etc). |
| Monitoring report configuration | M | The reporting configuration for the monitoring service (thresholds for triggering a monitoring event, e.g. minimum accuracy, delay, whether the reporting is one time or periodical or event-based). |
| Area of interest | O | The geographical or service area for which the monitoring applies. |
| Time validity | O | The time validity for the monitoring subscription. |
| Trigger Action requirement | O | These requirement identifies policies for triggering an action based on a monitoring event (e.g. if degradation is detected, to train a new model). |

#### 8.x.3.2	ML model perf monitoring subscription response

Table 8.x.3.2-1 shows the response sent by the AIMLE server to the VAL server for the ML model perf monitoring subscription.

Table 8.x.3.2-1: ML model perf monitoring subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the ML model perf monitoring request was successful. |
| > Subscription ID | M | Subscription identifier corresponding to the subscription. |
| > Expiration time | O | Indicates the expiration time of the subscription. To maintain an active subscription, a subscription update is required before the expiration time. |
| Failure response | O | Indicates that the request has failed. |
| > Cause | O | The cause for the request failure. |

#### 8.x.3.3	ML model perf monitoring notify

Table 8.x.3.2-1 shows the notification sent by the AIMLE server to the VAL server for the ML model performance degradation.

Table 8.x.3.2-1: ML model perf monitoring notify

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the subscription. |
| ML model ID | M | Identity of the ML model. |
| ML model degradation indication | M | Identifies the degradation of the ML model. |
| >ML model degradation parameters | O | The performance metrics which are expected to be degraded (F1-score, recall, precision, accuracy). |
| > Cause | O | The cause for the degradation of the ML model. |
| Trigger Action | O | The trigger action, which is notified, and may be one of the following: - the adaptation of the AIMLE service, such as training of a new ML model for the AIMLE by the same or a different AIMLE client, - the re-training of the ML model by the same or different AIMLE client, - the termination of the AIMLE service and initiating a new AIMLE service with a new ML model. |

* * * Next Change * * * *

### 9.2.x	ML model perf monitoring API

#### 9.2.x.1	General

Table 9.2.x.1-1 illustrates the API for ML model performance monitoring subscription and notification. This API enables the VAL server to communicate with the AIMLE server for subscribing for model performance monitoring.

Table 9.2.x.1-1: Aimles_MLModelPerfMonitor API

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Aimles_MLModelPerfMonitor | Subscribe | Subscribe/Notify | VAL Server |
|  | Notify |  |  |

#### 9.2.x.2	Subscribe

API operation name: MLModelPerfMonitor_Subscribe

Description: The consumer subscribes for ML model perf monitoring.

Inputs: See clause 8.x.3.1.

Outputs: See clause 8.x.3.2.

See clause 8.x.2 for details of usage of this operation.

#### 9.3.2.3	Notify

API operation name: MLModelPerfMonitor_Notify

Description: The consumer is notified by ML repository on the ML model perf monitoring.

Inputs: -

Outputs: See clause 8.x.3.3.

See clause 8.x2 for details of usage of this operation.