3GPP TSG-SA6 Meeting #64	S6-245595

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.436 | CR | 0060 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for DN Energy Analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_App |  |  |  |  |  | Date: |  |  | 2024-11-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In TR 23.700-82 solution #15 has been concluded on AI-enabled ADAES analytics on DN energy efficiency/consumption. This contribution provides the new feature in 23.436 based on this concluded solution. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new functionality description has been added in clause 6. Also, the procedure, information flows and APIs have been added for the new analytics service on DN energy analytics. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The new analytics service for ADAES will not be specified, based on the TR 23.700-82 conclusions. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause 9.2.2, New clauses: 6.x, 8.x, 8.x.1, 8.x.2, 8.x.3, 8.x.3.1, 8.x.3.2, 8.x.3.3, 9.2.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 6.x	Support for DN Energy Analytics

This feature supports a logical functionality at the ADAES to provide analytics on the DN energy consumption /efficiency. The DN energy analytics is performed per DNN/ DNAI and may be used to trigger the application server migration to different cloud. The analytics are based on NWDAF analytics and UPF/DN measurements on user plane load as well as edge/app side measurements on the energy consumption.

* * * Next Change * * * *

## 8.x	Procedure for supporting DN Energy Efficiency analytics

### 8.x.1	General

This clause describes the procedure for DN energy consumption/efficiency analytics, where the analytics are performed based on data collected from one or more DNs and A-ADRF.

### 8.x.2	Procedure

Figure 8.x.2-1 illustrates the procedure for DN energy efficiency analytics enablement solution.

Pre-conditions:

1.	Data producers (e.g. A-ADRF, EAS, EES) may be pre-configured with data producer profiles (as in Table 8.2.4.8-1) for the data they can provide. ADAES has discovered available data producers and their data producer profiles.

Figure 8.8.2.1-1: ADAES support for DN energy analytics

1.	The VAL server sends a DN energy analytics request to ADAES to perform analytics on the DN Energy Consumption/Efficiency for one or more DNs/EDNs, Event ID= “DN energy analytics”, for a given DN service area (or subarea) and a given time window.

2.	The ADAES authorizes the VAL request.

3.	The ADAES requests and receives from the EAS /VAL servers hosted at the serving and target DNs (within the VAL service area), expected application service load and traffic schedules for the ongoing or future sessions within the area. Such data include traffic schedule report for the VAL Server, and this step re-uses the step 3 to 10 of clause 8.8.2.1.

4.	The ADAES calculates the expected energy consumption or efficiency based on the received traffic and load data for the given DNN/DNAI based on the request.

NOTE: How the collected data are used to calculate energy efficiency metric is up to implementation.

5.	The ADAES obtains the corresponding trained ML model based on procedure in 3GPP TS 23.482 clause 8.3.2 and performs analytics to derive the predicted energy consumption at the target area and time horizon. The analytics outputs can be the predicted energy consumption / efficiency for the given DNN/DNAI.

6.	The ADAES sends a DN energy analytics response with the energy consumption/efficiency analytics output data to the VAL server.

Based on 6, the VAL server can use these analytics as input to trigger pro-actively:

-	an application server migration to a different edge cloud or to a centralized cloud as a way of reducing the energy consumption for the edge (if consumption is expected to be very high (e.g. higher than a pre-configured threshold)).

-	an application server offboarding and the instantiation of a new server at the target edge/centralized cloud to minimize energy consumption of the edge platform (taking into account the system wide energy efficiency).

### 8.x.3	Information flows

#### 8.x.3.1	General

The following information flows are specified for DN energy analytics based on 8.x.2.

#### 8.x.3.2	DN energy analytics request

Table 8.x.3.2-1 describes information elements for the DN energy analytics request from the VAL server to the ADAE server.

Table 8.8.3.2-1: DN energy analytics request

| Information element | Status | Description |
| --- | --- | --- |
| Analytics Consumer ID | M | The identifier of the analytics consumer (VAL server, EAS). |
| Analytics ID | M | The identifier of the analytics event. This ID can be for example “edge performance analytics”. |
| Analytics type | M | The type of analytics for the event, e.g. statistics or predictions. |
| DNN/DNAI | M | DNN or DNAIs information for which the subscription applies. |
| Energy Efficiency/Consumption metrics | O | The formula and necessary metrics for calculating the EE based on load and traffic information per DN. |
| Target data producer profile criteria | O | Characteristics of the data producers to be used. |
| Preferred confidence level | O | The level of accuracy for the analytics service (in case of prediction). |
| Area of Interest | O | The geographical or service area for which the subscription request applies. |
| Time validity | O | The time validity of the subscription request. |
| Reporting requirements | O | It describes the requirements for the energy analytics reporting. This requirement may include e.g. the type and frequency of reporting (periodic or event triggered), the reporting periodicity in case of periodic, and reporting thresholds. |

#### 8.x.3.3	DN energy analytics response

Table 8.x.3.3-1 describes information elements for the DN energy analytics request from the VAL server to the ADAE server.

Table 8.x.3.3-1: DN energy analytics response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the analytics request (positive or negative acknowledgement). |
| Analytics ID | O | The identifier of the analytics event. |
| Analytics Output | O | The predictive or statistical parameter, which can be stats or prediction related to the energy consumption or efficiency for the edge platform for a given area/time and based on the event type. |
| > DNN | M | Identifies the data network name for which analytics information is provided. |
| > DNAI | M | Identifier of a user plane access to one or more DN(s) of the DN. |
| > Energy metrics | O | The predicted energy metrics. |
| >> Energy Consumption (NOTE) | O | The predicted energy consumption per DNAI based on network and edge resource usage |
| >> Energy Efficiency (NOTE) | O | The energy efficiency per DNAI based on network and edge resource usage (given a certain optimal energy consumption metric, which can be pre-configured). |
| >> DN Data Volume (NOTE) | O | The predicted data volume per DNAI. |
| > Area of Interest | O | The area (topological or geographical or edge area) where the analytics apply. |
| > Applicable time period | O | The time period that the analytics applies to. |
| >Confidence level | O | For predictive analytics, the achieved confidence level can be provided. |
| NOTE: At least one of these shall be present. |  |  |

* * * Next Change * * * *

### 9.2.2	ADAE server APIs

Table 9.2.2-1 illustrates the ADAE server APIs.

Table 9.2.2-1: List of ADAE server APIs

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_ADAE_VAL_performance_analytics | Subscribe | VAL server | Subscribe/Notify |
|  | Notify |  |  |
| SS_ADAE_slice_performance_analytics | Subscribe | VAL server | Subscribe/Notify |
|  | Notify |  |  |
| SS_ADAE_UE-to-UE_performance_analytics | Subscribe | VAL server | Subscribe/Notify |
|  | Notify |  |  |
| SS_ADAE_server-to-server_performance_analytics | Subscribe | VAL server / EES | Subscribe/Notify |
|  | Notify |  |  |
| SS_ADAE_location_accuracy_analytics | Subscribe | VAL server | Subscribe/Notify |
|  | Notify |  |  |
| SS_ADAE_service_API_analytics | Subscribe | VAL server / Subscriber/ API invoker | Subscribe/Notify |
|  | Notify |  |  |
| SS_ADAE_slice_usage_pattern_analytics | Subscribe | VAL server / SEAL server | Subscribe/Notify |
|  | Notify |  |  |
| SS_ADAE_edge_analytics | Subscribe | VAL server / EAS / EES | Subscribe/Notify |
|  | Notify |  |  |
|  | Get | VAL server / EAS / EES | Request / Response |
| SS_ADAES_slice_usage_stats | Get | VAL server | Request / Response |
| SS_ADAES_edge_preparation_analytics | Subscribe | VAL server / ECS / EES | Subscribe/Notify |
|  | Notify |  |  |
|  | Get | VAL server / ECS / EES | Request / Response |
| SS_ADAE_collision_detection_analytics | Subscribe | VAL server / LM server / UAE server, UAS application specific server | Subscribe/Notify |
|  | Notify |  |  |
|  | Get | VAL server / LM server / UAE server, UAS application specific server | Request / Response |
| SS_ADAE_location-related_UE_group_analytics | Subscribe | LM server | Subscribe/Notify |
|  | Notify |  |  |
|  | Get | LM server | Request / Response |
| SS_ ADAE_AIML_member_capability_analytics | Subscribe | VAL server / AIMLE server | Subscribe/Notify |
|  | Notify |  |  |
|  | Get | VAL server / AIMLE server | Request / Response |
| SS_ADAE_DN_energy_analytics | Get | VAL server / EAS / EES | Request / Response |

* * * Next Change * * * *

### 9.2.x	SS_ ADAE_DN_energy_analytics API

#### 9.2.x.1	General

API description: This API enables the VAL server to communicate with the ADAE server for requesting for DN energy efficiency/consumption analytics.

#### 9.2.x.2	Get DN_energy_analytics

API operation name: DN_energy_analytics_get

Description: The consumer requests DN energy efficiency/consumption analytics.

Inputs: See clause 8.x.3.2.

Outputs: See clause 8.x.3.3.

See clause 8.x.2 for details of usage of this operation.