3GPP TSG-SA WG6 Meeting #64	S6-245573

Orlando, USA, 18th – 22nd November 2024	(revision of S6-245323)

Source:	KPN N.V., Ericsson

Title:	Solving EN in AIMLE client selection

Spec:	3GPP TS 23.482 v0.3.0

Agenda item:	9.10

Document for:	Approval

Contact:	Yonatan Shiferaw (yonatan.shiferaw@tno.nl)

1. Introduction

The contribution resolves the EN in the AIMLE client selection in clause 8.9.2.1.

2. Reason for Change

The AIMLE client selection subscription and notification procedure in step 5 of clause 8.13.2 already clarify the VAL Service ID can be used in order to determine then determine the QoS parameters for the AIML traffic session between the requestor and the candidate AIMLE client(s).  This is used to resolve the editors note in cluase 8.9.2.1.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.482 V0.3.0.

* * * First Change * * * *

### 8.9.2	Procedure

#### 8.9.2.1	AIMLE client selection

Pre-conditions:

1.	AIMLE clients that support AI/ML operations have registered with the AIMLE server and included their AIMLE client profiles and optionally a list of supported services.

2.	The AIMLE server can access a ML repository to obtain AIMLE client profiles and supported services associated with the AIMLE clients.

Figure 8.9.2.1-1: AIMLE client selection

1.	A VAL server sends an AIMLE client selection request to an AIMLE server to select AIMLE clients available for participation in AI/ML operations (e.g., is available and have required data to train an ML model). The AIMLE client selection request includes information as described in Table 8.9.3.1-1.

2.	The AIMLE server performs authentication and authorization checks to determine if the requestor is able to select AIMLE clients.

3.	If the requestor is authorized, the AIMLE server performs AIMLE client selection based on the provided inputs as described below.

For VAL server selection, the AIMLE server receives a list of AIMLE client IDs in the request and selects the AIMLE clients in the list as candidate AIMLE clients.

For AIMLE server selection, the AIMLE server retrieves a list of clients with AI/ML capabilities from the ML repository. From the list of clients, the AIMLE server selects a list of candidate AIMLE clients, whose client profiles fulfil the AIMLE client selection criteria.

The AIMLE server may then determine the QoS parameters for the AIML traffic session between the requestor and the candidate AIMLE client(s) and configure the AI/ML traffic session(s) via SEALDD (Sdd_RegularTransmission API) or NEF services (AfSessionWithQoS API).	The AIMLE server determine the application QoS parameters based on the VAL Service ID.

NOTE x:	To determine the application QoS parameters based on the VAL service ID, the AIMLE server can use the VAL service ID associated AIMLE client QoS requirements in the AIMLE client selection criteria provided in step 1.

NOTE 1:	If the available AIMLE clients (as determined by the AIMLE server) that fulfil discovery criteria are less than the required minimum number of AIMLE clients for AI/ML operation (e.g., split AI/ML), then the AIMLE server can discover the remaining required AIMLE clients from other AIMLE servers over AIML-E reference point and include them in the response message.

NOTE 2: The AIMLE server can reuse SEAL group management for any necessary group management.

4.	The AIMLE server performs AIMLE client participation procedure with each candidate AIMLE client as described in clause 8.10. For all candidate AIMLE clients that agreed to partiay cipate in AI/ML operations, the AIMLE server selects the AIMLE clients and assigns an AIMLE client set identifier for the selected clients. The AIMLE client set identifier may then be used for training a ML model.

5.	The AIMLE server sends an AIMLE client selection response that includes information in Table 8.9.3.2-1.

* * * Next Change * * * *

### 8.13.2	Procedure

Figure 8.13.2-1: AIMLE client selection subscription and notification

1.	A VAL server sends an AIMLE client selection subscription request to the AIMLE Server. The AIMLE client selection subscription request includes information as described in Table 8.13.3-1 which includes selection criteria.

2.	The AIMLE server validates the AIMLE client selection subscription request. The AIMLE server further performs authentication and authorization checks to determine if the requestor is able to subscribe to the selected AIMLE client selection subscription request.

3.	The AIMLE server sends the AIMLE client selection subscription response to the VAL server.

4.	The AIME server monitors AIMLE clients whether they fulfil the selection criteria as provided in step 1. The AIMLE server interacts with the NEF and/or SEAL services (including SEALDD) to establish monitoring.

5.	The AIMLE Server obtains the identifiers of the AIMLE clients from the monitoring and selects the clients that fulfil the selection criteria and remove the AIMLE clients which do not fulfil the selection criteria. The AIMLE Server may determine the application QoS parameters (e.g. bandwidth, latency, jitter) for the AIML traffic session between the VAL server and the selected AIMLE client and configure the AIML traffic session(s) via SEALDD (Sdd_RegularTransmission API) or NEF services (AfSessionWithQoS API). When the AIMLE clients no more meet the criteria, the QoS adjustment is reversed.

The AIMLE server may determine the application QoS parameters based on the VAL Service ID.

NOTE x:	To determine the application QoS parameters based on the VAL service ID, the AIMLE server can use the VAL service ID associated AIMLE client QoS requirements in the AIMLE client selection criteria provided in step 1.

NOTE:	In case of AIMLE server determining the application QoS parameters based on the VAL service ID, how it does this is up to implementation.

6.	The AIMLE Server notifies the VAL server about the selected and re-selected AIMLE clients e.g., the AIMLE Client A is re-selected and replaced by AIMLE Client B.

Editor’s Note 1: Service operation definition in the API clause is FFS.

* * * End of Changes * * * *