3GPP TSG-SA WG6 Meeting #64	S6-245572

Orlando, US, 18th – 22nd November 2024	(revision of S6-245295)

Source:	Ericsson

Title:	Correction of the AIMLE Client selection

Spec:	3GPP TS 23.482 V0.3.0

Agenda item:	9.10

Document for:	Approval

1. Introduction

AIMLE client discovery request in table 8.8.3.1-2 already includes the QoS parameters; thus, the alignment in the related clauses is needed.

2. Reason for Change

The following corrections are needed:

-	Table 8.8.3.1-1, the number of the required AIMLE client shall be dependant on AIMLE client discovery criteria, i.e., without the indication it is not clear how many clients are needed to be discovered that leads to increase in signalling and discovery complexity.

-	Table 8.8.3.1-2, QoS requirements require clarification with addition of threshold definition and matching direction.

-	Table 8.9.3.1-1, the service ID shall be clarified as VAL service ID to avoid confusion.

-	Table 8.8.3.1-1, the number of the required AIMLE client shall be dependant on AIMLE client discovery criteria, i.e., without the indication it is not clear how many clients are needed to be selected that leads to increase in signalling and selection complexity.

-	Clause 8.9.2.1, it is specified that AIMLE server may use SEAL, NEF, and NWDAF capabilities to assist the AIMLE client selection. It is clarified how the QoS requirements are determined and the EN is resolved.

-	Clause 8.13.2, it is clarified how the QoS parameters are determined and the EN is resolved.

3. Conclusions

This pCR proposes re-use the provisioning in clause 8.8.3.1 for QoS determination.

4. Proposal

It is proposed to agree the following changes to 3GPP 23.482 V0.3.0.

* * * First Change * * * *

#### 8.8.3.1	AIMLE client discovery request

Table 8.8.3.1-1 shows the request sent by a VAL server to an AIMLE server for the AIMLE client discovery procedure.

Table 8.8.3.1-1: AIMLE client discovery request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identifier of the requestor (e.g., VAL server). |
| AIMLE client discovery criteria | M | Discovery criteria for finding suitable AIMLE clients for AI/ML operations as detailed in Table 8.8.3.1-2. |
| Number of the required AIMLE clients | O | Indicates the requested number of AIMLE clients to be discovered based on discovery criteria. |

Table 8.8.3.1-2: AIMLE client discovery criteria

| Information element | Status | Description |
| --- | --- | --- |
| Service requirement | M | Information about the required service |
| > Service ID | M | Required service ID that the client supports. This identifies the service associated with the requester. |
| > Service permission level | O | Required corresponding service permission level (e.g., premium resource usage standard resource usage, limited resource usage). |
| Requested ML model types | O | Requested ML model types (decision trees, linear regression, neural networks). |
| Requested AIML operations | M | Requested role for AI/ML operations such as model training, model transfer, model inference, model offload, model split. |
| Application layer AIMLE client capabilities | M | Application layer AIMLE client capability information (e.g., ML application type like FL/TL/SL, client availability to support AIML operations at the UE, AIMLE drop off rate). |
| Dataset requirements | O | Information about dataset. |
| > Dataset availability | O | Dataset availability, including dataset identifiers, dataset size, age, list of dataset features. |
| > Data capabilities | O | A list of data capabilities such as the type of data that can be collected (e.g., raw data), supported data processing capabilities (e.g., processed data) and supported exploratory data analysis functions. |
| AIML client task capability requirements | O | Indicates the AIML task requirements to discover the AIML clients for performing AIML tasks. It includes compute capabilities (e.g., high, low), task performance preference capabilities (e.g. Green task, Energy-efficient, low costs) |
| AIMLE client velocity | O | Indicates the AIMLE client velocity. It includes mobile (e.g., high, low), static. |
| Location information | O | Indicates the location information (e.g., Cell Identity, Tracking Area Identity, GPS Coordinates or civic addresses, VAL service area ID) to discover the AIMLE clients. |
| AIMLE client QoS requirements | O | Indicates the AIMLE client QoS information (like PLR, bandwidth, latency, jitter) with the corresponding threshold(s) and threshold matching direction(s) (e.g., above or below) to discover the AIMLE clients. |

Editor’s Note: Whether target area for data collection is included in the AIMLE client discovery criteria is FFS.

# *** Next Change ***

#### 8.9.3.1	AIMLE client selection request

Table 8.9.3.1-1 shows the request sent by a VAL server to an AIMLE server for the AIMLE client selection procedure.

Table 8.9.3.1-1: AIMLE client selection request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identifier of the requestor (e.g., VAL server). |
| VAL service identifier | O | An identifier for the VAL service associated with the requestor. |
| List of AIMLE client IDs | O (NOTE 1) (NOTE 3) | A list of AIMLE client IDs that was previously discovered for inclusion into an AIMLE client set. |
| AIMLE client selection criteria | O (NOTE 2) | Selection criteria for finding suitable AIMLE clients for AI/ML operations as detailed in Table 8.8.3.1-2. |
| Number of the required AIMLE clients | O (NOTE 2) (NOTE 3) | Indicates the requested number of AIMLE clients to be selected based on selection criteria. |
| NOTE 1: This information element is only present for VAL server selection. NOTE 2: These information elements are only present for AIMLE server selection. NOTE 3: At least one of the information elements are present. |  |  |

# *** Next Change ***

#### 8.9.2.1	AIMLE client selection

Pre-conditions:

1.	AIMLE clients that support AI/ML operations have registered with the AIMLE server and included their AIMLE client profiles and optionally a list of supported services.

2.	The AIMLE server can access a ML repository to obtain AIMLE client profiles and supported services associated with the AIMLE clients.

Figure 8.9.2.1-1: AIMLE client selection

1.	A VAL server sends an AIMLE client selection request to an AIMLE server to select AIMLE clients available for participation in AI/ML operations (e.g., is available and have required data to train an ML model). The AIMLE client selection request includes information as described in Table 8.9.3.1-1.

2.	The AIMLE server performs authentication and authorization checks to determine if the requestor is able to select AIMLE clients.

3.	If the requestor is authorized, the AIMLE server performs AIMLE client selection based on the provided inputs as described below.

For VAL server selection, the AIMLE server receives a list of AIMLE client IDs in the request and selects the AIMLE clients in the list as candidate AIMLE clients.

For AIMLE server selection, the AIMLE server retrieves a list of clients with AI/ML capabilities from the ML repository. From the list of clients, the AIMLE server selects a list of candidate AIMLE clients, whose client profiles fulfil the AIMLE client selection criteria. The AIMLE server may use SEAL (LM service), NEF (e.g. MonitoringEvent API), and NWDAF (e.g. UE mobility analytics as defined in clause 6.7.2 of 3GPP TS 23.288 [2]) capabilities to assist the AIMLE client selection.

The AIMLE server may then determines the QoS parameters for the AIML traffic session between the requestor and the candidate AIMLE client(s) and configure the AI/ML traffic session(s) via SEALDD (Sdd_RegularTransmission API) or NEF services (AfSessionWithQoS API).

NOTE 1:	If the available AIMLE clients (as determined by the AIMLE server) that fulfil discovery criteria are less than the required minimum number of AIMLE clients for AI/ML operation (e.g., split AI/ML), then the AIMLE server can discover the remaining required AIMLE clients from other AIMLE servers over AIML-E reference point and include them in the response message.

NOTE 2: The AIMLE server can reuse SEAL group management for any necessary group management.

Editor’s Note: Whether and how the AIMLE server determines the QoS parameters is FFS.

4.	The AIMLE server performs AIMLE client participation procedure with each candidate AIMLE client as described in clause 8.10. For all candidate AIMLE clients that agreed to participate in AI/ML operations, the AIMLE server selects the AIMLE clients and assigns an AIMLE client set identifier for the selected clients. The AIMLE client set identifier may then be used for training a ML model.

5.	The AIMLE server sends an AIMLE client selection response that includes information in Table 8.9.3.2-1.

# *** End of Changes ***