3GPP TSG-SA WG6 Meeting #64	S6-245585

Orlando, USA, 18th – 22nd November 2024	(revision of S6-245253)

Source:	Samsung, Convida Wireless LLC

Title:	Pseudo-CR on updates to the VFL training procedures

Spec:	3GPP TS 23.482 V0.3.0

Agenda item:	9.10

Document for:	Approval

Contact:	Arunprasath R, arun.prasath@samsung.com

1. Introduction

This pCR proposes to update the VFL procedure.

2. Reason for Change

VFL procedure is not complete and details about the continuous monitor and selection of AIMLE clients , retrieval of ML model and mechanism to check the AIMLE clients of their willingness to participate is missing.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.482 V0.3.0

* * * First Change * * * *

### 8.18.2	Procedure for supporting VFL

Pre-conditions:

1.	VFL members registered their VFL profile to the AIMLE Server. The VFL members may update their status or information in their VFL profile to the AIMLE Server.

2.	The datasets of each of the UEs (where the AIMLE Clients (as VFL members) are deployed on) belong to more than one different data domains.

3.	The VAL server has successfully subscribed/registered with the AIMLE server for model training notifications.

Figure 8.18.2-1: Procedure for supporting VFL

1.	An AIMLE server receives an ML model training request from a VAL server as described in clause 8.3.2. If ML model training request is received and the AIMLE server determines to use VFL training (VFL training between different data domains), the procedure continues to step 2.

2.	The AIMLE server retrieves the indicated ML model in step 1 using the ML model retrieval procedure as described in clause 8.1.

3.	If AIMLE client selection criteria are provided in step 1, then the AIMLE server continuously monitors and selects AIMLE clients for the VFL training as described step 4 of the clause 8.13.2. If a list of AIMLE clients is provided in step 1, the AIMLE server selects the provided AIMLE clients for the VFL training.

4.	The AIMLE Server gets VFL members information (the VFL members information may be included in the request in step 1 (Table 8.3.3.1-1) or be obtained through step 3). The AIMLE Server interacts with the VFL members (AIMLE Clients which are deployed on UEs) for each data domain for ML model training capability evaluation as described in clause 8.19.2.

5.	The AIMLE Server determines the VFL members for this VFL training process based on the information received in step 4 and parameters received in step 1 (Table 8.3.3.1-1). The AIMLE Server determines VFL members for each data domain. The criteria used by the AIMLE Server include:

-	Available data and minimum number of data samples for the same sample among the VFL members.

-	Feature alignment of the sample/datasets with data labels among the VFL members.

-	Available time of the VFL members for support the VFL training operations.

-	Capability and minimum number of the VFL members for the VFL training operations.

-	AIML model information for the VFL members and for the AIMLE Server.

6.	The AIMLE Server coordinates the selected VFL members for VFL training. During VFL training process, the VFL members send intermediate results to the AIMLE Server, and the AIMLE Server responds to the VFL members with the updated information (e.g. gradients). The information from the AIMLE server can be used to update the model parameters maintained at each VFL member for the different data domains..

6a.	The AIMLE server may report to the VAL server with the training status, that includes intermediate training results,.The VAL server may adjust its request on the ML model training. If the VAL Server is providing data labels to complete the training, the VAL Server sends updated training parameters for the AIMLE Server to distribute to the VFL members. The updated training parameters apply for models of VFL members associated with each data domain.

Editor’s Note 2:	The procedure for AIMLE Server coordinates the selected VFL members to perform VFL training is FFS.

7.	The AIMLE server sends a ML model training notification to the VAL server as described in clause 8.3.2. If the training schedule is not complete (e.g., there are remaining training rounds), the AIMLE server configures the next set of training schedules and steps 3 to 6 are repeated for the next training round.

* * * Next Change * * * *

### 8.19.3	Information flows

#### 8.19.3.1	ML model training capability evaluation request

Table 8.19.3.1-1 shows the request sent by AIMLE server to FL (e.g., HFL, VFL) members (AIMLE Clients which are deployed on UEs) for ML model training capability evaluation.

Table 8.19.3.1-1: ML model training capability evaluation request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Available time | O | Requirement on available time for supporting FL operations. |
| Test task | O | The task for test ML model training capability. |
| AI/ML models and model parameter(s) | O | Information about the AI/ML models and model parameters for use in FL training. In VFL, AI/ML models for different data domains are provided. |
| Requirement on dataset | O | Requirements on dataset for FL training. |
| > Common feature ID(s) | O (NOTE 1) | Identifier(s) of the required features common to the dataset of the different data domains. |
| >Data domain feature ID lists | O (NOTE 1) | List of features for each data domain(s) of the datasets at the UE. |
| >Data source | O (NOTE 2) | Data source for the FL training. |
| NOTE 1: If Requirement on dataset is provided, at least one of these IEs shall be present when for VFL. NOTE 2: If Requirement on dataset is provided, the IE shall be present when for HFL. |  |  |

NOTE:	The detail content of the Test task and AI/ML model is up to implementation.

* * * End Change * * * *