3GPP TSG-SA WG6 Meeting #64	S6-245735

Orlando, USA, 18th – 22nd November 2024	(revision of S6-245522, S6-245121)

Source:	Samsung

Title:	Pseudo-CR on spatial anchor usage information update

Spec:	3GPP TS 23.437 (0.3.0)

Agenda item:	9.11

Document for:	Approval

Contact:	Sapan Shah (sapan.shah@samsung.com)

1. Introduction

This pCR provides corrections to Sptail anchor usage infomration procedure.

2. Reason for Change

Alinging name of the procedure according to its infomration flows.

- IE table rquest name is “Spatial anchor usage information subscription request”

- Procedure name is “Spatial anchor usage analytics information”

Also, based on details provided in the notification message, the procedure is to provide usage details and not analytics details.

So, chaning the procedure name accordingly.

Also IE tables for request-response model is missing, so adding the IE tables.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.437 (0.3.0).

* * * First Change * * * *

## 8.4	Spatial anchor usage information

### 8.4.1	General

The spatial anchor usage information procedure allows consumer (i.e. VAL server) to get usage information about the spatial anchor.

### 8.4.2	Spatial anchor usage information (Sub-Notify)

#### 8.4.2.1	General

This clause enables VAL server to request spatial anchor usage information from the SEAL SAn server.

#### 8.4.2.2	Procedure

Figure 8.4.2.2-1 depicts the procedure for subscribing to receive information related to spatial anchor usage for metaverse applications. The service is provided by SAn server.

Figure 8.4.2.2-1: Spatial anchor usage information subscription

1.	A consumer (e.g. VAL server) sends a spatial anchor usage information subscription request to the SA server. The request can include the VAL server identifier, security credentials, a notification endpoint (e.g., IP address, FQDN, URI), and spatial anchor filters like spatial anchor identifiers, spatial anchor location information, position of the user, proximity distance, and application service identifier. It may also include the periodicity for reporting notification and sampling interval.

2.	Upon receiving the request, the MMES authorizes the VAL server subscription request and sends a spatial anchor usage subscription response to the consumer.

3.	The SAn server samples the information related to spatial anchor usage related infomration based on the obtained data and the stored spatial anchor data as per the sampling interval. It provides response to the consumer as per the reporting interval. The response message includes spatial anchor ID, number of times spatial anchor discovered and accessed, spatial anchor density per location, user density per spatial anchor, spatial anchor services accessed information.

### 8.4.2.3	Information flows

##### 8.4.3.3.1	Spatial anchor usage information subscription request

Table 8.4.3.3.1-1 describes information elements for the spatial anchor usage information subscription request from the consumer to the SAn server.

Table 8.4.4.1-1: Spatial anchor usage information subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., VAL server). |
| Security credentials | M | The security credentials of the requestor. |
| Spatial anchor filters | M | Indicates the spatial anchor filter for the information |
| > Spatial anchor identifiers | O | Indicates the spatial anchor identities |
| > Spatial anchor location information | O | Indicates the spatial anchor location information |
| > Area information | O | Indicates the area information (e.g., location information) |
| > Application service information | O | Indicates the application service information(e.g., application identifier) |
| sampling interval | O | Indicates the interval to sample the spatial anchor(s) related information |
| Reporting interval | O | Indicates the periodic reporting interval of information |
| notification endpoint | O | Indicates the endpoint(e.g., IP address, FQDN, URI) to send the information notification |
| NOTE: Spatial anchor filters IE must include at least one optional IE. |  |  |

##### 8.4.3.3.2	Spatial anchor usage information subscription response

Table 8.4.3.3.2-1 describes information elements for the spatial anchor usage information subscription response from the SAn server to the consumer.

Table 8.4.3.3.2-1: Spatial anchor usage information subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the subscription request was successful. |
| > subscription ID | M | Identity for the spatial anchor subscription |
| Failure response | O | Indicates that the create spatial anchor group request has failed. |
| > Cause | O | The cause for the request failure. |

##### 8.4.3.3.3	Spatial anchor usage information notification

Table 8.4.3.3.3-1 describes information elements for the spatial anchor usage information notification from the SAn server to the consumer.

Table 8.4.3.3.3-1: Spatial anchor usage information notification

| Information element | Status | Description |
| --- | --- | --- |
| subscription ID | M | Identity for the spatial anchor subscription |
| > Spatial anchor identifier | M | Indicates the spatial anchor identity |
| > Spatial anchor usage information | M | Indicates spatial anchor usage information. It includes number of times spatial anchor discovered and accessed, spatial anchor services accessed information. |
| > Spatial anchor density per location | M | Indicates the density of the spatial anchor in the given area. |
| > User density per spatial anchor | M | Indicates the density of users within proximity to a specific spatial anchor. |

### 8.4.3	Spatial anchor usage information (Request-Response)

#### 8.4.3.1	General

This clause enables VAL server to subscribe spatial anchor usage information from the SEAL SAn server.

#### 8.4.3.2	Procedure

Figure 8.4.3.2-1 depicts the procedure to receive information related to spatial anchor usage for metaverse applications. The service is provided by SAn server.

Figure 8.4.3.2-1 Spatial anchor usage information

1)	The consumer (e.g. VAL server) sends a request message to the SAn server to get the spatial anchor usage information as per Table 8.4.4.1-1.

2)	The SAn server authorizes the consumer. If the requestor is authorized, then the SAn server server provides the spatial anchor usage information. The response message includes spatial anchor ID, number of times spatial anchor discovered and accessed, spatial anchor density per location, user density per spatial anchor, spatial anchor services accessed information.

#### 8.4.3.3	Information flows

##### 8.4.3.1	Spatial anchor usage information request

Table 8.4.3.1-1 describes information elements for the spatial anchor usage information request from the consumer to the SAn server.

Table 8.4.3.1-1: Spatial anchor usage information subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., VAL server). |
| Security credentials | M | The security credentials of the requestor. |
| Spatial anchor filters | M | Indicates the spatial anchor filter for the information |
| > Spatial anchor identifiers | O | Indicates the spatial anchor identities |
| > Spatial anchor location information | O | Indicates the spatial anchor location information |
| > Area information | O | Indicates the area information (e.g., location information) |
| > Application service information | O | Indicates the application service information(e.g., application identifier) |
| NOTE: Spatial anchor filters IE must include at least one optional IE. |  |  |

##### 8.4.3.2	Spatial anchor usage information response

Table 8.4.3.2-1 describes information elements for the spatial anchor usage information subscription response from the SAn server to the consumer.

Table 8.4.3.2-1: Spatial anchor usage information subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the subscription request was successful. |
| > Spatial anchor identifiers | M | Indicates the spatial anchor identities |
| > Spatial anchor usage information | M | Indicates spatial anchor usage information. It includes number of times spatial anchor discovered and accessed, spatial anchor services accessed information. |
| > Spatial anchor density per location | M | Indicates the density of the spatial anchor in the given area. |
| > User density per spatial anchor | M | Indicates the density of users within proximity to a specific spatial anchor. |
| Failure response | O | Indicates that the create spatial anchor group request has failed. |
| > Cause | O | The cause for the request failure. |

* * * End of Change * * * *

<Proposed change in revision marks>