3GPP TSG-CT Meeting #106	CP-243238

# Madrid, Spain; 9th – 10th December 2024											(revision of C1-247154)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.301 | CR | 4124 | rev | 8 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | RAT utilization control in GUTI reallocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Ericsson, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei, HiSilicon, Ericsson, ZTE, Vodafone |  |  |  |  |  |  |  |  |  |
| Work item code: | ECRATU |  |  |  |  |  | Date: |  |  | 2024-12-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Support for RAT utilization control has been added to EPS and 5GS. In 5GS it is supported for the network to initiate providing new RAT utilization control information using the generic UE configuration update procedure. This functionality can be used both to inform of restricted RATs, but also to remove previously indicated RAT restrictions, if needed. To allow the same feature support level in EPS it is proposed to duplicate RACS functionality and add support of RAT utilization control information in the GUTI reallocation procedure |  |  |  |  |  |  |  |  |
| B |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | GUTI reallocation procedure is updated to support RAT utilization control information An Editor’s note is added for further study how to remove RAT restrictions from the UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RAT utilization control support level differ between EPS and 5GS and providing RAT utilization control information cannot be initiated by the UE in EPS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.23, 5.4.1.1, 5.4.1.2, 5.4.1.3, 5.4.1.4, 8.2.16.1, 8.2.16.x (new), 9.9.3.3x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev5: Editor’s Note in 5.4.1.3, from a previous revision, is deleted and a UE requirement is added to delete any entry for the RPLMN upon receiving RAT restriction information without restricted RATs. Addition of RAT restriction information is stored with PLMN id of current PLMN, aligned to C1-245681 and C1-245789. Rev8: Revision to CP-106; in clause 6.4.1.4 incorrect message name has been corrected from “GUTI REALLOCATION COMPLETE” to “GUTI REALLOCATION COMMAND message” |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 5.3.23	Control of UE radio access technology (RAT) utilization by EPS

Network operator may restrict subscriber's access to certain RAT. For this purpose, the network may send the RAT utilization control IE to the UE via the ATTACH ACCEPT message, the TRACKING AREA UPDATE ACCEPT message or the GUTI REALLOCATION COMMAND message (see clause 5.5.1.2, clause 5.5.3.2 and clause 5.4.1.2). Additionally, the UE shall not consider any PLMN and RAT combination with RAT restriction as a candidate PLMN for PLMN selection purpose, as specified in 3GPP TS 23.122 [6].

NOTE:	RAT is defined in 3GPP TS 36.304 [21] and 3GPP TS 38.304 [21A], respectively. In this specification RAT is used to refer to GERAN, UTRAN, E-UTRAN or NG-RAN.

Editor's note: How does the UE handle the received information of restricted RAT indicated in the RAT utilization control IE is FFS.

* * * Next Change * * * *

#### 5.4.1.1	General

The purpose of the GUTI reallocation procedure is to allocate a GUTI and optionally to provide one or more of the following to a particular UE:

-	a new TAI list;

-	a new DCN-ID;

-	in WB-S1 mode, if the UE supports RACS, either a UE radio capability ID deletion indication or a UE radio capability ID; and

-	if the UE supports RAT utilization control, new RAT utilization control information.

The reallocation of a GUTI is performed by the unique procedure defined in this clause. This procedure can only be initiated by the MME in state EMM-REGISTERED.

The GUTI can also be implicitly reallocated at attach or tracking area updating procedures. The implicit reallocation of a GUTI is described in the clauses which specify these procedures (see clause 5.5.1 and 5.5.3).

The PLMN identity in the GUTI indicates the current registered PLMN.

NOTE 1:	The GUTI reallocation procedure is usually performed in ciphered mode.

NOTE 2:	Normally, the GUTI reallocation will take place in conjunction with another mobility management procedure, e.g. as part of tracking area updating.

* * * Next Change * * * *

#### 5.4.1.2	GUTI reallocation initiation by the network

The MME shall initiate the GUTI reallocation procedure by sending a GUTI REALLOCATION COMMAND message to the UE and starting the timer T3450 (see example in figure 5.4.1.2.1).

The GUTI REALLOCATION COMMAND message shall include a GUTI and may include one or more of the following:

-	a TAI list;

-	a DCN-ID;

-	in WB-S1 mode, if the UE supports RACS, either a UE radio capability ID deletion indication or a UE radio capability ID; and

-	if the UE supports RAT utilization control, new RAT utilization control information.

If the RAT utilization control IE is included in the GUTI REALLOCATION COMMAND message, the MME shall not indicate that the access technology of the E-UTRAN cell on which the GUTI REALLOCATION COMMAND message is sent (i.e. E-UTRAN or satellite E-UTRAN) is restricted. The MME can include the RAT utilization control IE with the length of RAT utilization control contents field set to zero for the UE to remove the previously stored RAT utilization control information associated with the current PLMN, if any.

Figure 5.4.1.2.1: GUTI reallocation procedure

* * * Next Change * * * *

#### 5.4.1.3	GUTI reallocation completion by the UE

Upon receipt of the GUTI REALLOCATION COMMAND message, the UE shall:

-	store the GUTI;

-	store the TAI list, if provided;

-	store the DCN-ID, if provided;

-	in WB-S1 mode, if the UE supports RACS and the GUTI REALLOCATION COMMAND message includes:

a)	a UE radio capability ID deletion indication IE set to "Network-assigned UE radio capability IDs deletion requested", delete any network-assigned UE radio capability IDs associated with the registered PLMN stored at the UE, then the UE shall, after the completion of the ongoing GUTI reallocation procedure, initiate a tracking area updating procedure as specified in clause 5.5.3. If the UE has an applicable manufacturer-assigned UE radio capability ID for the current UE radio configuration, the UE shall include the UE radio capability ID availability IE set to "UE radio capability ID available" in the TRACKING AREA UPDATE REQUEST message; or

b)	a UE radio capability ID IE, store the UE radio capability ID as specified in annex C;

-	if the UE supports RAT utilization control and the GUTI REALLOCATION COMMAND message includes a RAT utilization control IE with the length of RAT utilization control contents field set to a non-zero value, store the received RAT utilization control information together with the PLMN identity of the current PLMN in the list of "PLMNs with associated RAT restrictions" and replace the previously stored one associated with the current PLMN, if any, with the newly received RAT utilization control information. If the UE supports RAT utilization control and the GUTI REALLOCATION COMMAND message contains the RAT utilization control IE with the length of RAT utilization control contents field set to zero, the UE shall remove the previously stored RAT utilization control information associated with the current PLMN, if any; and

-	send a GUTI REALLOCATION COMPLETE message to the MME.

The UE considers the new GUTI as valid and the old GUTI as invalid. If the UE receives a new TAI list in the GUTI REALLOCATION COMMAND message, the UE shall consider the new TAI list as valid and the old TAI list as invalid; otherwise, the UE shall consider the old TAI list as valid.

If the GUTI REALLOCATION COMMAND message contains the DCN-ID IE, then the UE shall store the included DCN-ID value together with the PLMN code of the registered PLMN in a DCN-ID list in a non-volatile memory in the ME as specified in annex C.

If the GUTI REALLOCATION COMMAND message does not contain the RAT utilization control IE, the UE shall maintain the stored RAT utilization control information associated with the current PLMN if any.

* * * Next Change * * * *

#### 5.4.1.4	GUTI reallocation completion by the network

Upon receipt of the GUTI REALLOCATION COMPLETE message, the MME shall stop the timer T3450 and consider the new GUTI as valid and the old GUTI as invalid. If a new TAI list is provided in the GUTI REALLOCATION COMMAND message, the MME shall consider the new TAI list as valid and the old TAI list as invalid.

If a RAT utilization control IE was included in the GUTI REALLOCATION COMMAND message, the MME shall consider the new "RAT utilization control information" as valid and the old "RAT utilization control information" as invalid.

* * * Next Change * * * *

#### 8.2.16.1	Message definition

This message is sent by the network to the UE to reallocate a GUTI and optionally to provide a new TAI list or a new DCN-ID or both. See table 8.2.16.1.

Message type:	GUTI REALLOCATION COMMAND

Significance:	dual

Direction:	network to UE

Table 8.2.16.1: GUTI REALLOCATION COMMAND message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Protocol discriminator | Protocol discriminator 9.2 | M | V | 1/2 |
|  | Security header type | Security header type 9.3.1 | M | V | 1/2 |
|  | GUTI reallocation command message identity | Message type 9.8 | M | V | 1 |
|  | GUTI | EPS mobile identity 9.9.3.12 | M | LV | 12 |
| 54 | TAI list | Tracking area identity list 9.9.3.33 | O | TLV | 8-98 |
| 65 | DCN-ID | DCN-ID 9.9.3.48 | O | TLV | 4 |
| 66 | UE radio capability ID | UE radio capability ID 9.9.3.60 | O | TLV | 3-n |
| B- | UE radio capability ID deletion indication | UE radio capability ID deletion indication 9.9.3.61 | O | TV | 1 |
| xx | RAT utilization control | RAT utilization control 9.9.3.3x | O | TLV | 2-n |

* * * Next Change * * * *

#### 8.2.16.xx	RAT utilization control

This IE is included to indicate the restricted RAT(s).

* * * Next Change * * * *

#### 9.9.3.3x	RAT utilization control

The purpose of the RAT utilization control information element is for the network to communicate the restricted RAT(s) to the UE.

The RAT utilization control information element is coded as shown in figure 9.9.3.3x.1 and table 9.9.3.3x.1.

The RAT utilization control is a type 4 information element.

Editor's note:	The specification of encoding RAT utilization control for equivalent PLMNs or roaming partner PLMNs is FFS. This decision depends on whether the registered PLMN and its equivalent PLMNs will share the same RAT utilization control information or have different.

| 8 |  | 7 |  | 6 |  | 5 |  | 4 | 3 |  |  |  | 2 |  | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RAT utilization control IEI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet 1 |
| Length of RAT utilization control contents |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet 2 |
| 0 spare | 0 spare |  | 0 Spare |  | 0 spare |  | 0 spare |  |  |  | 0 spare | Type of RAT utilization control |  |  |  | octet 3* |
| 0 spare | 0 spare |  | 0 Spare |  | 0 spare |  | NG-RAN |  |  | E-UTRAN |  | UTRAN |  | GERAN |  | octet 4* |

Figure 9.9.3.3x.1: RAT utilization control information element

Table 9.9.3.3x.1: RAT utilization control information element

| Type of RAT utilization control (octet 3, bit 1 to 2) |  |  |  |
| --- | --- | --- | --- |
| Bits |  |  |  |
| 2 | 1 |  |  |
| 0 | 0 | current PLMN |  |
| 0 | 1 | Unused, shall be interpreted as "current PLMN" if received by the UE |  |
| 1 | 0 | Unused, shall be interpreted as "current PLMN" if received by the UE |  |
| 1 | 1 | Unused, shall be interpreted as "current PLMN" if received by the UE |  |
| GERAN (octet 4, bit 1) |  |  |
| Bit |  |  |
| 1 |  |  |
| 0 |  | GERAN is not restricted |
| 1 |  | GERAN is restricted |
| UTRAN (octet 4, bit 2) |  |  |
| Bit |  |  |
| 2 |  |  |
| 0 |  | UTRAN is not restricted |
| 1 |  | UTRAN is restricted |
| E-UTRAN (octet 4, bit 3) |  |  |
| Bit |  |  |
| 3 |  |  |
| 0 |  | E-UTRAN is not restricted |
| 1 |  | E-UTRAN is restricted |
| NG-RAN (octet 4, bit 4) |  |  |
| Bit 4 |  |  |
| 0 |  | NG-RAN is not restricted |
| 1 |  | NG-RAN is restricted |

* * * End of Changes * * * *