3GPP TSG-CT Meeting #106	CP-243243

# Madrid, Spain; 9th – 10th December 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.549 | CR | 0041 | rev | 2 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the ETC_Configuration API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Work item code: | NSCALE |  |  |  |  |  | Date: |  |  | 2024-11-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The CR0024 (in C1-242795) added in the specification the ETC_Configuration API. However, the path informaiton of the API is incorrect in the specification because of the CR0024, quote of clause C.2: paths: /configurations/{configurationId}: put: description: > Note that the path information has to be "/val-services/{valServiceId}/configurations/{configurationId}". Quote: ###### 8.1.1.3.2.2 Resource definition Resource URI: {apiRoot}/su_nsc/<apiVersion>/val-services/{valServiceId}/configurations/{configurationId} Additionally, the "valServiceId" (resource URI variable) is not defined in the ETC_Configuration API though described in the procedural part of the specification (see clause 7.2.2.2.2) and also it is part of the resource definition (see clause 8.1.1.3.2.2). |  |  |  |  |  |  |  |  |
| Summary of change: |  | The path of the ETC_Configuration API (under clause C.2) is corrected, i.e., "/val-services/{valServiceId}/configurations/{configurationId}". Also, the "valServiceId" is define in the in the ETC_Configuration API. Backwards compatibility analysis The CR is backwards compatible as the OpenAPI is corrected by indicating the correct path information. Note that currently the API resources structure is completely broken as is wrongly defined by the TS so it cannot be used as currently stated in TS 24.549. Changes from agreed version (v1) of the CR C1-246661 was agreed at CT1#152 (in Orlando). However, an error in the ETC_Configuration API has been found which needs correction, i.e., the "valServiceId" (resource URI variable) is not defined in the ETC_Configuration API. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect path of the ETC_Configuration API which does not correspond to the definition of the API in TS 24.549. Also, the "valServiceId" (resource URI variable) fails to be defined in the API. Wrong API defined in OpenAPI. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | C.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | V1: release version field of the cover sheet corrected. V2: C1-246661 (v1 of the CR) agreed at CT1#152 meeting is revised to correct a further error in the OpenAPI file, i.e., "valServiceId", added to the definition of the ETC_Configuration API: - name: valServiceId description: String identifying the resource. in: path required: true schema: type: string |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# C.2	ETC_Configuration API

openapi: 3.0.0

info:

title: ETC_Configuration

version: 1.0.1

description: |

API for event triggered network slice adaptation configuration.

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 24.549 V18.3.0 Application Data Analytics Enablement Service; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/24_series/24.549/

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/su_nsc/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122.

paths:

/val-services/{valServiceId}/configurations/{configurationId}:

put:

description: >

Perfomrs event triggered network slice adaptation.

operationId: EventTriggeredNetworkAdaptation

tags:

- Event triggered network adaptation (Document)

parameters:

- name: valServiceId

description: String identifying the resource.

in: path

required: true

schema:

type: string

- name: configurationId

description: String identifying the resource.

in: path

required: true

schema:

type: string

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NwSliceAdptEvent'

responses:

'204':

description: >

No Content. The requested network slice adaptation is successfullyprocessed.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

NwSliceAdptEvent:

description: >

Represents the event associated with triggered network slice adaptation

with the underlying network.

type: object

properties:

valUeIds:

type: array

minItems: 1

items:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

sliceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

appReqs:

$ref: '#/components/schemas/AppReqs'

required:

- valUeIds

- sliceId

AppReqs:

description: Represents requirements for the requested application.

type: object

properties:

timeIntervals:

type: array

minItems: 1

items:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

area:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea'

ratType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

* * * End of Changes * * * *