3GPP TSG-CT Meeting #106	CP-243272

# Madrid, Spain; 9th – 10th December 2024                             (is revision of C4-244540)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.572 | CR | 0285 | rev | 4 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support multiple LCS contexts transfer while modifying the UP connection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19, 5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2024-11-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the recent SA2 meeting, clarification on location contexts transfer during modification of user plane connection between UE and LMF was approved in S2-2409391. One UP connection is maintained between UE and LMF for location service for a specific UE. When user plane based periodic and triggered location event report contexts are generated, the source LMF transfers the current location context(s) to the target LMF. Therefore, the multiple LCS session contexts transfer for the UE with LCS UP connection should be supported while modifying the UP connection defined in clause 6.18.3, TS 23.273 . |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new attributes of LCS contexts list to support multiple LCS context transfer while modifying the UP connection defined in clause 6.18.3, TS 23.273 . |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The modification procedure for user plane connection between UE and LMF, as defined in SA2, will not be fully supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.5.X(new), 6.1.6.2.30, 6.1.6.1, 6.1.6.2.X(new), 6.1.9, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible corrections to the OpenAPI files of Nlmf_location. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev: Correct formatting and spelling errors in OpenAPI and the descriptions for attribute addLocContextList. |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.2.2.5.X Multiple Location Contexts Transfer

If there are multiple LCS sessions for the UE with LCS UP connection, and if the source and target LMF support the MCTX feature, the source LMF shall send the additional LCS contexts of the UE to the target LMF in the addLocContextList (see clause 6.18.3 of 3GPP TS 23.273 [19]).

The requirement in clause 5.2.2.5.2 shall be applied with following addition:

Same as step1 of clause 5.2.2.5.2, the HTTP content of the request shall additionally contain:

-	addLocationContext attribute.

# * * * * Second change * * * *

6.1.6.2.30	Type: LocContextData

Table 6.1.6.2.30-1: Definition of type LocContextData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| amfId | NfInstanceId | M | 1 | Indicates the AMF Instance serving the UE. LMF shall use the AMF Instance to forward LCS related N1/N2 messages to the UE/RAN. |  |
| locationQoS | LocationQoS | C | 0..1 | This IE shall contain the location QoS if available. |  |
| supportedGADShapes | array(SupportedGADShapes) | C | 1..N | This IE shall contain the supported GAD shapes if available. |  |
| supi | Supi | C | 0..1 | This IE shall contain the SUPI if available. |  |
| gpsi | Gpsi | C | 0..1 | This IE shall contain the GPSI if available. |  |
| addLocContextList | array(AddLocationContext) | O | 1..N | When present, this IE shall contain a list of locationContext(s) to be transferred from source LMF to target LMF. (NOTE X) | MCTX |
| ldrType | LdrType | M | 1 | The type of LDR |  |
| hgmlcCallBackURI | Uri | M | 1 | Callback URI of the H-GMLC |  |
| ldrReference | LdrReference | M | 1 | LDR Reference |  |
| periodicEventInfo | PeriodicEventInfo | C | 0..1 | Information for periodic event reporting. (NOTE 1) |  |
| areaEventInfo | AreaEventInfo | C | 0..1 | Information for area event reporting. (NOTE 1) |  |
| motionEventInfo | MotionEventInfo | C | 0..1 | Information for motion event reporting. (NOTE 1) |  |
| eventReportMessage | EventReportMessage | M | 1 | Contains an embedded event report |  |
| eventReportingStatus | EventReportingStatus | O | 0..1 | Status of event reporting |  |
| ueLocationInfo | UELocationInfo | O | 0..1 | Location information for the target UE |  |
| cIoT5GSOptimisation | boolean | C | 0..1 | This IE shall be present if it was received from AMF. When present, it shall be set as follows: - true: Control Plane CIoT 5GS Optimisation was used and no signalling or data is currently pending for the UE at the AMF. - false (default): Control Plane CIoT 5GS Optimisation was not used or signalling or data is currently pending for the UE at the AMF. |  |
| ecgi | Ecgi | C | 0..1 | When present, this IE shall indicate the identifier of the E-UTRAN cell serving the UE. This IE shall be present if it was received from AMF. |  |
| ncgi | Ncgi | C | 0..1 | When present, this IE shall indicate the identifier of the NR cell serving the UE. This IE shall be present if it was received from AMF |  |
| guami | Guami | C | 0..1 | This IE shall be present if it was received from AMF. When present, it shall contain the GUAMI serving the UE. |  |
| supportedFeatures | SupportedFeatures | C | 0..1 | This IE shall be present if at least one feature defined in clause 6.1.9 is supported. |  |
| uePositioningCap | UePositioningCapabilities | O | 0..1 | When present, this IE shall indicate the positioning capabilities supported by the UE. |  |
| scheduledLocTime | DateTime | O | 0..1 | When present, this IE shall contain the scheduled time (in UTC) that the UE needs to be located. |  |
| indoorOutdoorInd | IndoorOutdoorInd | O | 0..1 | When present, this IE shall indicate whether the UE is indoor or outdoor. |  |
| losNlosMeasureInd | LosNlosMeasureInd | O | 0..1 | When present, this IE shall indicate whether LOS measurement or NLOS measurement is used. |  |
| upCumEvtRptCriteria | UpCumEvtRptCriteria | O | 0..1 | When present, this IE shall include the cumulative event report timer or the maximum number of location reporting over user plane or both. |  |
| NOTE 1: At least one of periodicEventInfo, areaEventInfo or motionEventInfo shall be present in the LocContextData structure. NOTE X: If the source and target LMF support MCTX feature and the source LMF contains more than one LCS context for the UE with LCS UP connection, this attribute shall include the additional location context(s) of the UE. The first LCS context shall be in the LocContextData data type and shall not be repeated in the addLocContextList attribute. |  |  |  |  |  |

# * * * *Third change * * * *

6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Nlmf_Location service based interface protocol.

Table 6.1.6.1-1: Nlmf_Location specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| InputData | 6.1.6.2.2 | Information within Determine Location Request |
| LocationData | 6.1.6.2.3 | Information within Determine Location Response |
| GeographicalCoordinates | 6.1.6.2.4 | Geographical coordinates |
| GeographicArea | 6.1.6.2.5 | Geographic area specified by different shape |
| Point | 6.1.6.2.6 | Ellipsoid Point |
| PointUncertaintyCircle | 6.1.6.2.7 | Ellipsoid point with uncertainty circle |
| PointUncertaintyEllipse | 6.1.6.2.8 | Ellipsoid point with uncertainty ellipse |
| Polygon | 6.1.6.2.9 | Polygon |
| PointAltitude | 6.1.6.2.10 | Ellipsoid point with altitude |
| PointAltitudeUncertainty | 6.1.6.2.11 | Ellipsoid point with altitude and uncertainty ellipsoid |
| EllipsoidArc | 6.1.6.2.12 | Ellipsoid Arc |
| LocationQoS | 6.1.6.2.13 | QoS of Location request |
| CivicAddress | 6.1.6.2.14 | Indicates a Civic address |
| PositioningMethodAndUsage | 6.1.6.2.15 | Indicates the usage of a positioning method |
| GnssPositioningMethodAndUsage | 6.1.6.2.16 | Indicates the usage of a Global Navigation Satellite System (GNSS) positioning method |
| VelocityEstimate | 6.1.6.2.17 | Velocity estimate |
| HorizontalVelocity | 6.1.6.2.18 | Horizontal velocity |
| HorizontalWithVerticalVelocity | 6.1.6.2.19 | Horizontal and vertical velocity |
| HorizontalVelocityWithUncertainty | 6.1.6.2.20 | Horizontal velocity with speed uncertainty |
| HorizontalWithVerticalVelocityAndUncertainty | 6.1.6.2.21 | Horizontal and vertical velocity with speed uncertainty |
| UncertaintyEllipse | 6.1.6.2.22 | Ellipse with uncertainty |
| UeLcsCapability | 6.1.6.2.23 | Indicates the LCS capability supported by the UE. |
| PeriodicEventInfo | 6.1.6.2.24 | Indicates the information of periodic event reporting |
| AreaEventInfo | 6.1.6.2.25 | Indicates the information of area based event reporting |
| ReportingArea | 6.1.6.2.26 | Indicates an area for event reporting |
| MotionEventInfo | 6.1.6.2.27 | Indicates the information of motion based event reporting |
| CancelLocData | 6.1.6.2.29 | Information within Cancel Location Request |
| LocContextData | 6.1.6.2.30 | Information within Transfer Location Context Request |
| EventReportMessage | 6.1.6.2.31 | Indicates an event report message |
| EventReportingStatus | 6.1.6.2.32 | Indicates the status of event reporting |
| UELocationInfo | 6.1.6.2.33 | Indicates location information of a UE |
| EventNotifyData | 6.1.6.2.34 | Information within Event Notify Request |
| UeConnectivityState | 6.1.6.2.35 | Indicates the connectivity state of a UE |
| LocalOrigin | 6.1.6.2.36 | Indicates a Local origin in a reference system. |
| RelativeCartesianLocation | 6.1.6.2.37 | Relative Cartesian Location |
| Local2dPointUncertaintyEllipse | 6.1.6.2.38 | Local 2D point with uncertainty ellipse |
| Local3dPointUncertaintyEllipsoid | 6.1.6.2.39 | Local 3D point with uncertainty ellipsoid |
| UncertaintyEllipsoid | 6.1.6.2.40 | Ellipsoid with uncertainty |
| LocalArea | 6.1.6.2.41 | Local area specified by different shape |
| UeAreaIndication | 6.1.6.2.42 | UE area Indication |
| MinorLocationQoS | 6.1.6.2.43 | Minor Location QoS |
| MbsrInfo | 6.1.6.2.44 | MBSR Information |
| LocMeasurementReq | 6.1.6.2.45 | Location Measurement Request |
| LocMeasurementResp | 6.1.6.2.46 | Location Measurement Response |
| LocMeasurements | 6.1.6.2.47 | Location Measurements result |
| HighAccuracyGnssMetrics | 6.1.6.2.48 | High Accuracy GNSS Metrics |
| UpNotifyData | 6.1.6.2.49 | UP Notify Data |
| UpSubscription | 6.1.6.2.50 | UP Subscription |
| RelatedUe | 6.1.6.2.51 | Specifies information for related UE for ranging and sidelink positioning |
| UpConfig | 6.1.6.2.52 | UP Config |
| RangeDirection | 6.1.6.2.53 | Represents the distance and direction between two points. |
| 2DRelativeLocation | 6.1.6.2.54 | Represents 2D local co-ordinates with origin corresponding to another known point. |
| 3DRelativeLocation | 6.1.6.2.55 | Represents 3D local co-ordinates with origin corresponding to another known point. |
| AddLocationDatas | 6.1.6.2.56 | Contains one or more LocationData. |
| AddEventNotifyDatas | 6.1.6.2.57 | Contains one or more EventNotifyData. |
| LocationDataExt | 6.1.6.2.58 | Represents location data with one or more corresponding UEs. |
| EventNotifyDataExt | 6.1.6.2.59 | Represents notified data with one or more corresponding UEs, containing one or more EventNotifyData. |
| MappedLocationQoSEps | 6.1.6.2.60 | Mapped Location QoS for EPS |
| AdditionalUeInfo | 6.1.6.2.61 | MBSR UE Information |
| AddLocationContext | 6.1.6.2.X | LCS context information |
| Altitude | 6.1.6.3.2 | Indicates value of altitude |
| Angle | 6.1.6.3.2 | Indicates value of angle |
| Uncertainty | 6.1.6.3.2 | Indicates value of uncertainty |
| Orientation | 6.1.6.3.2 | Indicates value of orientation angle |
| HorizAxesOrientation | 6.1.6.3.2 | Orientation of Horizontal Coordinate System |
| Confidence | 6.1.6.3.2 | Indicates value of confidence |
| Accuracy | 6.1.6.3.2 | Indicates value of accuracy |
| InnerRadius | 6.1.6.3.2 | Indicates value of the inner radius |
| CorrelationID | 6.1.6.3.2 | LCS Correlation ID |
| AgeOfLocationEstimate | 6.1.6.3.2 | Indicates value of the age of the location estimate |
| HorizontalSpeed | 6.1.6.3.2 | Indicates value of horizontal speed |
| VerticalSpeed | 6.1.6.3.2 | Indicates value of vertical speed |
| SpeedUncertainty | 6.1.6.3.2 | Indicates value of speed uncertainty |
| BarometricPressure | 6.1.6.3.2 | Specifies the measured uncompensated atmospheric pressure |
| LcsServiceType | 6.1.6.3.2 | LCS service type |
| LdrReference | 6.1.6.3.2 | LDR Reference |
| LirReference | 6.1.6.3.2 | LIR Reference |
| ReportingAmount | 6.1.6.3.2 | Number of required periodic event reports |
| ReportingInterval | 6.1.6.3.2 | Event reporting periodic interval |
| MinimumInterval | 6.1.6.3.2 | Minimum interval between event reports |
| MaximumInterval | 6.1.6.3.2 | Maximum interval between event reports |
| SamplingInterval | 6.1.6.3.2 | Maximum time interval between consecutive evaluations by a UE of a trigger event |
| ReportingDuration | 6.1.6.3.2 | Maximum duration of event reporting |
| LinearDistance | 6.1.6.3.2 | Minimum straight line distance moved by a UE to trigger a motion event report |
| LMFIdentification | 6.1.6.3.2 | LMF identification |
| EventReportCounter | 6.1.6.3.2 | Number of event reports received from the target UE |
| EventReportDuration | 6.1.6.3.2 | Duration of event reporting |
| UePositioningCapabilities | 6.1.6.3.2 | Indicates the positioning capabilities supported by the UE. |
| TimeWindow | 6.1.6.3.2 | Indicates the Time Window |
| ExternalClientType | 6.1.6.3.3 | Indicates types of External Clients |
| SupportedGADShapes | 6.1.6.3.4 | Indicates supported GAD shapes |
| ResponseTime | 6.1.6.3.5 | Indicates acceptable delay of location request |
| PositioningMethod | 6.1.6.3.6 | Indicates supported positioning methods |
| PositioningMode | 6.1.6.3.7 | Indicates supported modes used for positioning method |
| GnssId | 6.1.6.3.8 | Global Navigation Satellite System (GNSS) ID |
| Usage | 6.1.6.3.9 | Indicates usage made of the location measurement |
| LcsPriority | 6.1.6.3.10 | Indicates priority of the LCS client |
| VelocityRequested | 6.1.6.3.11 | Indicates velocity requirement |
| AccuracyFulfilmentIndicator | 6.1.6.3.12 | Indicates fulfilment of requested accuracy |
| VerticalDirection | 6.1.6.3.13 | Indicates direction of vertical speed |
| LdrType | 6.1.6.3.14 | Indicates LDR types |
| ReportingAreaType | 6.1.6.3.15 | Indicates type of event reporting area |
| OccurrenceInfo | 6.1.6.3.16 | Specifies occurrence of event reporting |
| ReportingAccessType | 6.1.6.3.17 | Specifies access types of event reporting |
| EventClass | 6.1.6.3.18 | Specifies event classes |
| ReportedEventType | 6.1.6.3.19 | Specifies type of event reporting |
| TerminationCause | 6.1.6.3.20 | Specifies causes of event reporting termination |
| LcsQosClass | 6.1.6.3.21 | Specifies LCS QoS class |
| UeLocationServiceInd | 6.1.6.3.22 | Specifies location service types requested by UE |
| IndoorOutdoorInd | 6.1.6.3.23 | Indoor Outdoor Indication |
| FixType | 6.1.6.3.24 | Fix Type |
| LosNlosMeasureInd | 6.1.6.3.25 | LOS/NLOS measurement indication |
| UpConnectionStatus | 6.1.6.3.26 | UP Connection Status |
| RangingSlResult | 6.1.6.3.27 | Specifies result type for ranging and sidelink positioning |
| RelatedUeType | 6.1.6.3.28 | Specifies type of related UE for ranging and sidelink positioning |
| LcsUpConnectionInd | 6.1.6.3.29 | LCS UP Connection Indication |
| UeUpPositioningCapabilities | 6.1.6.3.30 | Indicates the user plane positioning capabilities supported by the UE. |
| SlPositioningCapability | 6.1.6.3.31 | Indicates the Ranging and sidelink positioning capabilities supported by the UE. |

Table 6.1.6.1-2 specifies data types re-used by the Nlmf_Location service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nlmf_Location service based interface.

Table 6.1.6.1-2: Nlmf_Location re-used Data Types

| Data type | Reference | Comments |  |
| --- | --- | --- | --- |
| Supi | 3GPP TS 29.571 [8] | Subscription Permanent Identifier |  |
| Pei | 3GPP TS 29.571 [8] | Permanent Equipment Identifier |  |
| Gpsi | 3GPP TS 29.571 [8] | Generic Public Subscription Identifier |  |
| Ecgi | 3GPP TS 29.571 [8] | E-UTRA Cell Identity |  |
| Ncgi | 3GPP TS 29.571 [8] | NR Cell Identity |  |
| NfInstanceId | 3GPP TS 29.571 [8] | Network Function Instance ID |  |
| Uri | 3GPP TS 29.571 [8] | Uniform Resource Identifier |  |
| RefToBinaryData | 3GPP TS 29.571 [8] | Reference to binary data |  |
| AccessType | 3GPP TS 29.571 [8] | Access type |  |
| CmState | 3GPP TS 29.518 [23] | Connection Management State |  |
| Guami | 3GPP TS 29.571 [8] | GUAMI |  |
| SupportedFeatures | 3GPP TS 29.571 [8] | Supported Features |  |
| RedirectResponse | 3GPP TS 29.571 [8] | Redirect Response |  |
| TwapId | 3GPP TS 29.571 [8] | TWAP identifier |  |
| TnapId | 3GPP TS 29.571 [8] | TNAP identifier |  |
| DateTime | 3GPP TS 29.571 [8] | Date and Time |  |
| DurationSec | 3GPP TS 29.571 [8] |  | Duration Second |
| LpHapType | 3GPP TS 29.518 [23] |  | Type of Low Power and/or High Accuracy Positioning |
| ReportingInd | 3GPP TS 29.515 [27] |  | Reporting indication |
| IntegrityRequirements | 3GPP TS 29.515 [27] |  | Integrity Requirements |
| UpLocRepAddrAfRm | 3GPP TS 29.122 [29] |  | Endpoint address for location reporting over user plane |
| UpCumEvtRptCriteria | 3GPP TS 29.515 [27] |  | Criteria for sending cumulative events reports over control plane |
| IntegrityResult | 3GPP TS 29.515 [27] |  | Integrity Result |

# * * * *Fourth change * * * *

6.1.6.2.X	Type: AddLocationContext

Table 6.1.6.2.X-1: Definition of type AddLocationContext

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| locationQoS | LocationQoS | C | 0..1 | This IE shall contain the location QoS if available. |
| ldrType | LdrType | M | 1 | The type of LDR |
| hgmlcCallBackURI | Uri | M | 1 | Callback URI of the H-GMLC |
| ldrReference | LdrReference | M | 1 | LDR Reference |
| periodicEventInfo | PeriodicEventInfo | C | 0..1 | Information for periodic event reporting. (NOTE 1) |
| areaEventInfo | AreaEventInfo | C | 0..1 | Information for area event reporting. (NOTE 1) |
| motionEventInfo | MotionEventInfo | C | 0..1 | Information for motion event reporting. (NOTE 1) |
| eventReportMessage | EventReportMessage | M | 1 | Contains an embedded event report |
| eventReportingStatus | EventReportingStatus | O | 0..1 | Status of event reporting |
| ueLocationInfo | UELocationInfo | O | 0..1 | Location information for the target UE |
| scheduledLocTime | DateTime | O | 0..1 | When present, this IE shall contain the scheduled time (in UTC) that the UE needs to be located. |
| indoorOutdoorInd | IndoorOutdoorInd | O | 0..1 | When present, this IE shall indicate whether the UE is indoor or outdoor. |
| losNlosMeasureInd | LosNlosMeasureInd | O | 0..1 | When present, this IE shall indicate whether LOS measurement or NLOS measurement is used. |
| upCumEvtRptCriteria | UpCumEvtRptCriteria | O | 0..1 | When present, this IE shall include the cumulative event report timer or the maximum number of location reporting over user plane or both. |
| NOTE 1: At least one of periodicEventInfo, areaEventInfo or motionEventInfo shall be present in the LocContextList structure. |  |  |  |  |

# * * * * Fifth change * * * *

6.1.9	Feature Negotiation

The features in table 6.1.9-1 are defined for the Nlmf_Location API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [4].

Table 6.1.9-1: Supported Features

| Feature number | Feature Name | M/O | Description |
| --- | --- | --- | --- |
| 1 | ES3XX | M | Extended Support of HTTP 307/308 redirection An NF Service Consumer (e.g. AMF) that supports this feature shall support handling of HTTP 307/308 redirection for any service operation of the Location service. An NF Service Consumer that does not support this feature does only support HTTP redirection as specified for 3GPP Release 15. |
| 2 | SAT | O | Satellite Access Support of this feature implies the LMF shall be able to determine the geographical area identified by a country, area of a country or International area where UE is located for PLMN selection verification. |
| 3 | MUTIQOS | O | Support of Mlutiple Location QoSes. This feature bit indicates the support of more than one Location QoSes during consuming location service. |
| 4 | MBSR | O | Support of Mobile Base Station Relay. This feature indicates the support of location service capability for MBSR. |
| 5 | Ranging_SL | O | Support of Ranging and Sidelink Positioning This feature indicates the support of Ranging and Sidelink Positioning. |
| 6 | INTRES | O | Integrity Result Support of this feature implies the LMF shall be able to provide the integrity result for location estimates when integrity requirement was received in the location request. |
| X | MCTX | O | Support of Multiple LCS contexts transfer. This feature is supported if both the source LMF and target LMF are capable of transferring multiple LCS contexts. Otherwise, the Transfer location context procedures are executed multiple times to transfer each LCS context, the target LMF maintains an LCS contexts container where it adds each newly transferred LCS context. |

# * * * * Sixth change * * * *

A.2	Nlmf_Location API

[…]

LocContextData:

description: Information within Transfer Location Context Request.

type: object

required:

- amfId

- ldrType

- hgmlcCallBackURI

- ldrReference

- eventReportMessage

properties:

amfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

locationQoS:

$ref: '#/components/schemas/LocationQoS'

supportedGADShapes:

type: array

items:

$ref: '#/components/schemas/SupportedGADShapes'

minItems: 1

addLocContextList:

type: array

items:

$ref: '#/components/schemas/AddLocationContext'

minItems: 1

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

ldrType:

$ref: '#/components/schemas/LdrType'

hgmlcCallBackURI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

ldrReference:

$ref: '#/components/schemas/LdrReference'

periodicEventInfo:

$ref: '#/components/schemas/PeriodicEventInfo'

areaEventInfo:

$ref: '#/components/schemas/AreaEventInfo'

motionEventInfo:

$ref: '#/components/schemas/MotionEventInfo'

eventReportMessage:

$ref: '#/components/schemas/EventReportMessage'

eventReportingStatus:

$ref: '#/components/schemas/EventReportingStatus'

ueLocationInfo:

$ref: '#/components/schemas/UELocationInfo'

cIoT5GSOptimisation:

type: boolean

default: false

ecgi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ecgi'

ncgi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ncgi'

guami:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Guami'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

uePositioningCap:

$ref: '#/components/schemas/UePositioningCapabilities'

scheduledLocTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

indoorOutdoorInd:

$ref: '#/components/schemas/IndoorOutdoorInd'

losNlosMeasureInd:

$ref: '#/components/schemas/LosNlosMeasureInd'

upCumEvtRptCriteria:

$ref: 'TS29515_Ngmlc_Location.yaml#/components/schemas/UpCumEvtRptCriteria'

AddLocationContext:

description: Information within Location context(s).

type: object

required:

- ldrType

- hgmlcCallBackURI

- ldrReference

- eventReportMessage

properties:

locationQoS:

$ref: '#/components/schemas/LocationQoS'

ldrType:

$ref: '#/components/schemas/LdrType'

hgmlcCallBackURI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

ldrReference:

$ref: '#/components/schemas/LdrReference'

periodicEventInfo:

$ref: '#/components/schemas/PeriodicEventInfo'

areaEventInfo:

$ref: '#/components/schemas/AreaEventInfo'

motionEventInfo:

$ref: '#/components/schemas/MotionEventInfo'

eventReportMessage:

$ref: '#/components/schemas/EventReportMessage'

eventReportingStatus:

$ref: '#/components/schemas/EventReportingStatus'

ueLocationInfo:

$ref: '#/components/schemas/UELocationInfo'

scheduledLocTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

indoorOutdoorInd:

$ref: '#/components/schemas/IndoorOutdoorInd'

losNlosMeasureInd:

$ref: '#/components/schemas/LosNlosMeasureInd'

upCumEvtRptCriteria:

$ref: 'TS29515_Ngmlc_Location.yaml#/components/schemas/UpCumEvtRptCriteria'

EventReportMessage:

description: Indicates an event report message.

type: object

required:

- eventClass

- eventContent

properties:

eventClass:

$ref: '#/components/schemas/EventClass'

eventContent:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

[…]

# * * * * End of changes * * * *