3GPP TSG-CT WG4 Meeting #126	C4-245039

# Orlando, U.S; 18th – 22nd November 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.573 | CR | 0217 | rev | - | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Keepalive of TLS Session for N32-f |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | SBIProtoc18 |  |  |  |  |  | Date: |  |  | 2024-11-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR intends to propose a mechanism for keepalive of TLS session for N32-f. --- GSMA has requested in their LS (see C4-244012) that TCP connection for a long-lived N32-f session needs to be maintained regardless of active traffic exchanged between SEPPs. In order to address this request, it is proposed that explicit HTTP OPTIONS request/response transaction over N32-f is initiated to maintain keepalive of TLS session for N32-f. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - New feature KPALV is introduced to support this functionality. - Feature negotiation for the new feature is added in N32-c Security Capability exchange, to allow both ends to negotiate the use of keepalive. - Timer value used for the keepalive can be exchanged during the N32-c Security Capability exchange. - Define procedure to start the timer and send HTTP OPTIONS request over estabished N32-f when the timer expires. The timer is reset in case the message is sent over N32-f. - Two datatypes (SecNegotiateReqData and SecNegotiateRspData) are updated to include the exchange of timer value for keepalive - Open API is updated to supported the newly added datatypes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements from GSMA N32 lifetime/keepalive is not met. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2, 5.3.3.2.z (new), 6.1.5.2.2, 6.1.5.2.3, 6.1.7, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backwards compatible changes to N32 Handshake API. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** First Change ***

5.2.2	Security Capability Negotiation Procedure

The initiating SEPP shall initiate a Security Capability Negotiation procedure towards the responding SEPP to agree on a security mechanism to use for protecting NF service related signalling over N32-f. An end to end TLS connection shall be setup between the SEPPs before the initiation of this procedure. This procedure may also be used to tear down the N32-f TLS connection if the remote SEPP indicated support of the feature NFTLST during the setup of the N32-c connection. The procedure is described in Figure 5.2.2-1 below.

Figure 5.2.2-1: Security Capability Negotiation Procedure

1.	The initiating SEPP issues a HTTP POST request towards the responding SEPP with the request body containing the "SecNegotiateReqData" IE carrying the following information:

-	Supported security capabilities (i.e., PRINS and/or TLS);

-	Whether the 3gpp-Sbi-Target-apiRoot HTTP header is supported, if TLS security is supported;

-	Sender PLMN ID(s) or SNPN ID(s);

-	Target PLMN ID or SNPN ID;

-	Purpose of the intended usage of N32 connection;

-	The senderN32fFqdn IE, if both the initiating SEPP and the responding SEPP support the SNDN32F feature and the initiating SEPP wishes the responding SEPP to establish the N32-f connection towards a specific FQDN (of the initiating SEPP);

-	The senderN32fPortList IE, if both the initiating SEPP and the responding SEPP support the SNDN32F feature and the initiating SEPP wishes the responding SEPP to establish the N32-f connection using a specific port number. When present, the list shall contain one port number per supported security capability (i.e., PRINS and/or TLS);

-	The n32HandshakeId IE, if the N32 handshake identifier is supported by the initiating SEPP, that may be used to correlate the N32-f connection to the parent N32-c context for TLS security (see clause 5.3.3.2). If the responding SEPP supports the N32 handshake identifier, the responding SEPP shall include the received N32 handshake identifier in TLS protected message forwarding procedure via correlated N32-f connection(s) towards the initiating SEPP and the subsequent N32-c signaling request related to this N32-c context (e.g. to request to tear down the N32-f TLS connection).

-	The n32KeepaliveTimer IE, if the initiating SEPP supports KPALV and wishes to use specific timer value for keepalive.

If different PLMNs or SNPNs are represented by different PLMN IDs or SNPN IDs (respectively) supported by a SEPP, then the SEPP shall use separate N32-connections for each pair of local and remote PLMN or SNPN. Both SEPPs shall store the mapping between the N32 connections and their pair of PLMN IDs or SNPN IDs.

NOTE 1:	If SEPPs support separate FQDN per PLMN or SNPN, then Target PLMN Id or Target SNPN Id is not required as target PLMN or SNPN can be selected by the FQDN.

To tear down the N32-f connection when negotiated security scheme is TLS, the "SecNegotiateReqData" IE shall contain:

-	Supported security capability set to "NONE"

2a.	On successful processing of the request, the responding SEPP shall respond to the initiating SEPP with a "200 OK" status code and a POST response body that contains "SecNegotiateRspData" IE carrying the following information:

-	Selected security capability (i.e., PRINS or TLS);

-	Whether the 3gpp-Sbi-Target-apiRoot HTTP header is supported, if TLS security is selected;

-	Sender PLMN ID(s) or SNPN ID(s).

-	Purpose of the accepted usage of N32 connection.

-	The senderN32fFqdn IE, if the responding SEPP wishes the initiating SEPP to establish the N32-f connection towards a specific FQDN (of the responding SEPP).

-	The senderN32fPort IE, if the responding SEPP wishes the initiating SEPP to establish the N32-f connection using a specific port number.

-	The n32HandshakeId IE, if the N32 handshake identifier was received in the request and the responding SEPP supports the N32 handshake identifier. The initiating SEPP shall include the received N32 handshake identifier in TLS protected message forwarding procedure via correlated N32-f connection(s) towards the responding SEPP and the subsequent N32-c signaling request related to this N32-c context (e.g. to request to tear down the N32-f TLS connection).

-	The n32KeepaliveTimer IE, if the responding SEPP supports KPALV and wishes to use specific timer value for keepalive.

NOTE 2:	Same SEPP endpoints can serve all accepted purposes over the same N32-f connection established as the result of request/response messages.

The responding SEPP compares the initiating SEPP's supported security capabilities to its own supported security capabilities and selects, based on its local policy, a security mechanism, which is supported by both the SEPPs. If the selected security capability indicates any other capability other than PRINS, then the HTTP/2 connection initiated between the two SEPPs for the N32 handshake procedures shall be terminated. The negotiated security capability shall be applicable on both the directions. If the selected security capability is PRINS, then the two SEPPs may decide to create (if not available) / maintain HTTP/2 connection(s) where each SEPP acts as a client towards the other (which acts as a server). This may be used for later signalling of N32-f error reporting procedure (see clause 5.2.5) and N32-f context termination procedure (see clause 5.2.4).

If different PLMNs or SNPNs are represented by different PLMN IDs or SNPN IDs (respectively) supported by a SEPP, then the SEPP shall use separate N32-connections for each pair of local and remote PLMN or SNPN. Both SEPPs shall store the mapping between the N32 connections and their pair of PLMN IDs or SNPN IDs.

The SEPP shall select the PLMN or SNPN from the list of supported PLMN(s) or SNPN(s) based on the received Target PLMN ID or SNPN ID, or based on PLMN or SNPN specific FQDN used in the request, and provide the selected PLMN's PLMN Id(s) in the plmnIdList or the selected SNPN's SNPN Id(s) in the snpnIdList.

In case no purposes are exchanged, the receiving SEPP shall assume by default that purposes are for Roaming and inter-PLMN mobility as described in clause 6.1.5.3.9.

The initiating SEPP and/or responding SEPP may enable the establishment of an N32 connection for the purpose of Disaster Roaming only during disaster conditions.

When the request is for tearing down the existing N32-f TLS connection, the "SecNegotiateRspData" IE shall contain:

-	Supported security capability set to "NONE"

and, subsequently, both SEPP shall terminate the N32-c and N32-f TLS connection.

If the initiating SEPP receives the indication that the responding SEPP supports the feature SNDN32F in the response, the initiating SEPP shall consider that the responding SEPP supports and has accepted the senderN32fFqdn IE and/or senderN32fPortList sent by the initiating SEPP in the request message. Otherwise, the initiating SEPP shall consider that the responding SEPP has ignored the senderN32fFqdn IE and/or senderN32fPortList sent by the initiating SEPP. If the initiating SEPP supports the feature SNDN32F and receives the senderN32fFqdn IE and/or the senderN32fPort IE from the responding SEPP, the initiating SEPP shall establish the N32-f connection towards the responding SEPP using the received N32-f FQDN and/or the senderN32fPort IE.

If the responding SEPP receives the indication that the initiating SEPP supports the feature SNDN32F in the request, the responding SEPP shall consider that the initiating SEPP supports receiving the senderN32fFqdn IE and/or senderN32fPort from the responding SEPP in the response message. Otherwise, the responding SEPP shall consider that the initiating SEPP does not support the senderN32fFqdn IE and/or senderN32fPort, and the responding SEPP shall not send these attributes to initiating SEPP. If the responding SEPP supports the feature SNDN32F and receives the senderN32fFqdn IE and/or the senderN32fPortList IE from the initiating SEPP, the responding SEPP shall establish the N32-f connection towards the initiating SEPP using the received N32-f FQDN and/or the N32-f port number received in the senderN32fPortList IE corresponding to the selected security capability (i.e., TLS or PRINS).

If the N32-f context exists between the peer SEPPs, and the N32 exchange capability request is not for tearing down the N32-f connections, the responding SEPP shall:

-	stop sending any further messages over the N32-f towards the initiating SEPP;

-	delete the current N32-f context and terminate any N32-f connection with the initiating SEPP; and

-	process the received exchange capability request.

If the initiating SEPP wishes to request keepalive of TLS session for N32-f connection, the initiating SEPP shall include the support of the feature KPALV in the N32-c Security Capability Negotiation request. The initiating SEPP may also include the timer value, within the range as indicated in clause 6.1.5.2.2, the initiating SEPP wishes to use to maintain the keepalive, otherwise default value as indicated in clause 6.1.5.2.2 shall be assumed that initiating SEPP wishes to use for the timer value.

If the responding SEPP receives the feature KPALV in the N32-c Security Capability Negotiation request, the responding SEPP may decide whether to accept the keepalive of TLS session for N32-f connection. If the responding SEPP decides to use the keepalive, the support of feature KPALV shall be included in the N32-c Security Capability Negotiation response. The responding SEPP may also include the timer value, within the range as indicated in clause 6.1.5.2.3, the responding SEPP accepts to use to maintain the keepalive, otherwise the time value provided in the request shall apply.

2b.	On failure or redirection, the responding SEPP shall respond to the initiating SEPP with an appropriate status code as specified in clause 6.1.4.2.

If the responding SEPP has sent an outgoing Security Capability Negotiation request to the initiating SEPP, the responding SEPP shall compare the FQDN of the initiating SEPP that has been received in the incoming Security Capability Negotiation request message with the FQDN of the responding SEPP that has been sent in the outgoing Security Capability Negotiation request. If the responding SEPP's FQDN lexicographically precedes, it shall reject the incoming HTTP request message with the cause "N32C_EXCHANGE_CAPABILITY_ONGOING" and it shall continue with its initiated procedure and vice versa.

EXAMPLE: assuming SEPP A's FQDN is "sepp.5gc.mnc345.mcc012.3gppnetwork.org" and SEPP B's FQDN is "sepp.5gc.mnc346.mcc012.3gppnetwork.org", then SEPP A's FQDN precedes SEPP B's FQDN and SEPP A proceeds with its exchange capability procedure.

A SEPP may be configured to accept an HTTP request from a given PLMN and not to send an HTTP request for exchange capability towards that PLMN.

*** Next Change ***

5.3.3	Message Forwarding to Peer SEPP when TLS is used

5.3.3.1	General

When the negotiated security policy between the SEPPs is TLS, then the procedures described in clause 5.3.2 shall not be applied. Messages shall be forwarded to the peer SEPP as specified in clause 6.1.4.3.4 of 3GPP TS 29.500 [4].

5.3.3.2	Correlation of N32-c context and N32-f Connection for TLS Security

This clause addresses the correlation between a N32-f connection and its parent N32-c context when the negotiated security mode is TLS. When TLS Security is used, the correlation between a N32-f connection to its parent N32-c context shall be identified with following mechanism:

-	When there is only one N32-c context successfully negotiated between a pair of SEPPs, one SEPP shall correlate a N32-f connection to its N32-c context by matching the peer SEPP's identifier (i.e. the FQDN of the peer SEPP) in the received TLS certificate with the FQDN(s) of the peer SEPP in the corresponding N32-c context (i.e. the FQDNs carried in the "sender" IE and/or the "senderN32fFqdn" IE); or

NOTE: 	If the received certificate contains FQDNs for different SEPPs (e.g. one common certificate used for all the SEPPs in the whole network), one SEPP can use the peer SEPP FQDN in the Via header (see clause 6.10.10.3 of 3GPP TS 29.500 [4]) of the incoming N32-f HTTP message to perform the matching with the FQDN in the corresponding N32-c context.

-	When multiple N32-c contexts were successfully negotiated between a pair of SEPPs, then

-	if the N32 Handshake Ids were exchanged during the N32-c negotiation, one SEPP shall correlate the N32-f connection to its parent N32-c context by matching the N32 Handshake Id in the incoming N32-f HTTP messages (carried in the "3gpp-Sbi-N32-Handshake-Id" header) with the received N32 Handshake Id (carried in the "n32HandshakeId" IE) in corresponding N32-c context; or

-	if the N32 Handshake Ids were not successfully exchanged during the N32-c negotiation (i.e. if at least one SEPP did not signal its N32 Handshake Id during the Security Capability Negotiation Procedure) and if different N32 purposes were successfully negotiated for N32-c contexts, one SEPP shall correlate the N32-f connection to its parent N32-c context by matching the N32 purpose of the incoming N32-f HTTP messages (as stated in the "3gpp-Sbi-Interplmn-Purpose" HTTP header if present or as "ROAMING" if the "3gpp-Sbi-Interplmn-Purpose" HTTP header is not present, see clause 6.14 of 3GPP TS 29.500 [4]) with the supported N32 purpose(s) in the corresponding N32-c context.

5.3.3.2.z	Keepalive of TLS Session for N32-f Connection

When the two PLMNs require keepalive of TLS session for N32-f connection, the SEPPs may negotiate the feature KPALV when initiating N32-c Security Capability Negotiation to achieve this as described in clause 5.2.2.

If SEPPs have successfully negotiated the feature KPALV, both SEPP shall start the timer with the value as negotiated. The timer shall restart in case the SEPP sends message over N32-f based on request received. If the timer expires, the SEPP shall send a HTTP OPTIONS request towards the Authority of the other SEPP over the established TLS session for N32-f. In case the SEPP does not receive response to the HTTP OPTIONS request, the SEPP shall regard the N32-f connection to be no longer valid and shall release the corresponding N32 connection.

*** Next Change ***

6.1.5.2.2	Type: SecNegotiateReqData

Table 6.1.5.2.2-1: Definition of type SecNegotiateReqData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| sender | Fqdn | M | 1 | This IE shall uniquely identify the SEPP that is sending the request. This IE is used to identify and store the negotiated security capability against the right SEPP. |  |
| n32HandshakeId | string | C | 0..1 | This IE shall be present if the initiating SEPP support N32 handshake identifier and the N32 handshake identifier may be used to correlate the N32-f connection to the parent N32-c context for TLS security (see clause 5.3.3.2). When present, this IE shall contain the N32 handshake identifier to be used by the responding SEPP, if TLS security is negotiated, for: - TLS protected message forwarding procedure over N32-f towards the initiating SEPP - subsequent N32-c signalling request related to this N32-c connection (e.g. to request to tear down the N32-f TLS connection) The n32HandshakeId shall encode a 64-bit integer in hexadecimal representation. Each character in the string shall take a value of "0" to "9" or "A" to "F" and shall represent 4 bits. The most significant character representing the 4 most significant bits of the N32 handshake Id shall appear first in the string, and the character representing the 4 least significant bit of the N32 handshake Id shall appear last in the string. Pattern: '^[A-Fa-f0-9]{16}$' Example: "0600AD1855BD6007". |  |
| supportedSecCapabilityList | array(SecurityCapability) | M | 1..N | This IE shall contain the list of security capabilities that the requesting SEPP supports. To tear down the N32-f TLS connection and to reset N32-c context (as identified by the n32HandshakeId IE, if present), this IE shall set SecurityCapability as "NONE". |  |
| 3GppSbiTargetApiRootSupported | boolean | C | 0..1 | This IE should be present and indicate that the 3gpp-Sbi-Target-apiRoot HTTP header is supported, if TLS security is supported for N32f message forwarding. When present, it shall indicate if TLS security using the 3gpp-Sbi-Target-apiRoot HTTP header is supported: - true: supported - false (default): not supported (NOTE 1) |  |
| plmnIdList | array(PlmnId) | O | 1..N | A list of PLMN IDs associated with the SEPP, which is sending the request. The list to be stored by the receiving SEPP in a N32-f Context (see clause 5.9.3 in 3GPP TS 33.501 [6]) |  |
| snpnIdList | array(PlmnIdNid) | O | 1..N | A list of SNPN IDs associated with the SEPP, which is sending the request. The list to be stored by the receiving SEPP in a N32-f Context (see clause 5.9.3 in 3GPP TS 33.501 [6]) |  |
| targetPlmnId | PlmnId | O | 1 | When present, this IE shall contain a PLMN ID of the target SEPP. See clause 5.2.2 step 1. |  |
| targetSnpnId | PlmnIdNid | O | 0..1 | When present, this IE shall contain a SNPN ID of the target SEPP. See clause 5.2.2 step 1. |  |
| intendedUsagePurpose | array(IntendedN32Purpose) | O | 1..N | This attribute notifies the list of requested usage purpose the N32 is established for. |  |
| supportedFeatures | SupportedFeatures | C | 0..1 | This IE shall be present if at least one feature defined in clause 6.1.7 is supported |  |
| senderN32fFqdn | Fqdn | O | 0..1 | This IE may be present if the sending SEPP wishes the receiving SEPP to establish the N32-f connection towards a specific FQDN. (NOTE 2) | SNDN32F |
| senderN32fPortList | array(Uinteger) | O | 1..N | This IE may be present if the sending SEPP wishes the receiving SEPP to establish the N32-f connection using a specific port number. The N32-f ports list shall contain one port number per security capability encoded in the supportedSecCapabilityList IE and it shall be ordered in the same order as the security capabilities list. For example, if TLS is the first security capability in the supportedSecCapabilityList, then the first N32-f port in the senderN32fPortList shall be for TLS. (NOTE 3) | SNDN32F |
| n32KeepaliveTimer | integer | O | 0..1 | This IE may be present if the initiating SEPP wishes to use for keepalive of TLS session for N32-f connection. The value shall be represented in seconds and between 300 and 86400, i.e. between 5 minutes and 1 day. If the value is absent, 3600 (seconds) is assumed to be used. | KPALV |
| NOTE 1: The attribute name does not follow the naming conventions specified in 3GPP TS 29.501 [5]. The attribute name is kept though as defined in the current specification for backward compatibility reason. NOTE 2: If the senderN32fFqdn IE is absent, the receiving SEPP establishes the N32-f connection towards the sending SEPP using the N32-c FQDN and/or local configuration. NOTE 3: If the senderN32fPortList IE is absent, the receiving SEPP shall use a locally configured port if any, otherwise the default HTTPs port number, i.e., TCP port 443 for "https" URIs as specified in IETF RFC 9113 [7]. |  |  |  |  |  |

*** Next Change ***

6.1.5.2.3	Type: SecNegotiateRspData

Table 6.1.5.2.3-1: Definition of type SecNegotiateRspData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| sender | Fqdn | M | 1 | This IE shall uniquely identify the SEPP that is sending the response. This IE is used to identify and store the negotiated security capability against the right SEPP. |  |
| selectedSecCapability | SecurityCapability | M | 1 | This IE shall contain the security capability selected by the responding SEPP. When the request is for tearing down the N32-f TLS connection, the responding SEPP shall add SecurityCapability as "NONE". |  |
| n32HandshakeId | string | C | 0..1 | This IE shall be present, if the N32 handshake identifier to was received in the request and the responding SEPP supports the N32 handshake identifier. When present, this IE shall contain the N32 handshake identifier to be used by the initiating SEPP for: - TLS protected message forwarding procedure over N32-f towards the responding SEPP - subsequent N32-c signalling request related to this N32-c connection (e.g. to request to tear down the N32-f TLS connection) The n32HandshakeId in the response may have a different value than the n32HandshakeId received in the request. The n32HandshakeId shall encode a 64-bit integer in hexadecimal representation. Each character in the string shall take a value of "0" to "9" or "A" to "F" and shall represent 4 bits. The most significant character representing the 4 most significant bits of the N32 handshake Id shall appear first in the string, and the character representing the 4 least significant bit of the N32 handshake Id shall appear last in the string. Pattern: '^[A-Fa-f0-9]{16}$' Example: "0600AD1855BD6007". |  |
| 3GppSbiTargetApiRootSupported | boolean | C | 0..1 | This IE should be present and indicate that the 3gpp-Sbi-Target-apiRoot HTTP header is supported, if TLS security is negotiated for N32f message forwarding and the initiating SEPP indicated support of this header. When present, it shall indicate if TLS security using the 3gpp-Sbi-Target-apiRoot HTTP header is supported: - true: supported - false (default): not supported (NOTE 1) |  |
| plmnIdList | array(PlmnId) | O | 1..N | A list of PLMN IDs of a single PLMN associated with the SEPP, which is sending the response. The list to be stored by the receiving SEPP in a N32-f Context (see clause 5.9.3 in 3GPP TS 33.501 [6]). If different PLMNs are represented by different PLMN IDs supported by a SEPP, then the SEPP shall select the PLMN as specified in clause 5.2.2 step 2a. |  |
| snpnIdList | array(PlmnIdNid) | O | 1..N | A list of SNPN IDs of a single SNPN associated with the SEPP, which is sending the response. The list to be stored by the receiving SEPP in a N32-f Context (see clause 5.9.3 in 3GPP TS 33.501 [6]). If different SNPNs are represented by different SNPN IDs supported by a SEPP, then the SEPP shall select the SNPN as specified in clause 5.2.2 step 2a. |  |
| allowedUsagePurpose | array(IntendedN32Purpose) | O | 1..N | This attribute notifies the list of allowed usage purpose the N32 is established for. IntendedN32Purpose shall not include attribute "cause". |  |
| rejectedUsagePurpose | array(IntendedN32Purpose) | O | 1..N | This attribute notifies the list of rejected usage purpose the N32 is established for. Shall only be present if any of the requested usage purpose is rejected. |  |
| supportedFeatures | SupportedFeatures | C | 0..1 | This IE shall be present if at least one feature defined in clause 6.1.7 is supported |  |
| senderN32fFqdn | Fqdn | O | 0..1 | This IE may be present if the sending SEPP wishes the receiving SEPP to establish the N32-f connection towards a specific FQDN. (NOTE 2) | SNDN32F |
| senderN32fPort | Uinteger | O | 0..1 | This IE may be present if the sending SEPP wishes the receiving SEPP to establish the N32-f connection using a specific port number. (NOTE 3) | SNDN32F |
| n32KeepaliveTimer | integer | O | 0..1 | This IE may be present if the initiating SEPP wishes to use for keepalive of TLS session for N32-f connection. The value shall be represented in seconds and with the same range as n32KeepaliveTimer attribute in SecNegotiateReqData datatype as specified in clause 6.1.5.2.2. If the value is absent, the value as received in SecNegotiateReqData datatype as specified in clause 6.1.5.2.2 is assumed to be used. | KPALV |
| NOTE 1: The attribute name does not follow the naming conventions specified in 3GPP TS 29.501 [5]. The attribute name is kept though as defined in the current specification for backward compatibility reason. NOTE 2: If the senderN32fFqdn IE is absent, the receiving SEPP establishes the N32-f connection towards the sending SEPP using the N32-c FQDN and/or local configuration. NOTE 3: If the senderN32fPort number is absent, the receiving SEPP shall use a locally configured port, if any, otherwise the default HTTPs port number, i.e., TCP port 443 for "https" URIs as specified in IETF RFC 9113 [7]. |  |  |  |  |  |

*** Next Change ***

6.1.7	Feature Negotiation

The feature negotiation mechanism specified in clause 6.6 of 3GPP TS 29.500 [4] shall be used to negotiate the features applicable between the c-SEPP and the p-SEPP, for the N32 Handshake service, if any.

The c-SEPP shall indicate the features it supports for the N32 Handshake service, if any, by including the supportedFeatures attribute in the HTTP POST request message for following service operations:

-	Security Capability Negotiation procedure, as specified in clause 5.2.2 to negotiate the security capability;

The p-SEPP shall determine the supported features for the requested network as specified in clause 6.6 of 3GPP TS 29.500 [4] and shall indicate the supported features by including the supportedFeatures attribute in content of the HTTP response for the service operation.

The syntax of the supportedFeatures attribute is defined in clause 5.2.2 of 3GPP TS 29.571 [12].

The following features are defined for the N32 Handshake service.

Table 6.1.7-1: Features of supportedFeatures attribute used by N32 Handshake service

| Feature Number | Feature | M/O | Description |
| --- | --- | --- | --- |
| 1 | NFTLST | O | N32-f TLS Connection Termination Support A SEPP that supports this feature shall support handling of Security Capability Negotiation procedure to tear down the N32-f TLS connection as specified in clause 5.2.2). |
| 2 | PSEPRO | O | PRINS Security Profiles Support A SEPP that supports this feature shall support the negotiation of security profiles as specified in clause 5.2.3.3. |
| 3 | PSIU | M | Protection of Sensitive Information in URI (Path and Query Parameters) A SEPP that complies with this release of the specification shall support this feature, i.e. the protection of sensitive information in URI path and query parameters in HTTP messages to be forwarded. |
| 4 | SNDN32F | O | Support of N32-f FQDN and N32-f Port. A SEPP that supports this feature: - may signal an N32-f FQDN and Port within the Security Capability Negotiation towards the remote SEPP; - shall support receiving a N32-f FQDN and Port(s) within the Security Capability Negotiation from the remote SEPP - shall support forwarding the N32-f traffic towards the N32-f FQDN and Port received from the peer SEPP. |
| Y | KPALV | O | Supports and use of Keepalive of TLS session for N32-f A SEPP that supports this feature may indicates the support and the intends to use Keepalive of TLS session for N32-f. |
| Feature number: The order number of the feature within the supportedFeatures attribute (starting with 1). Feature: A short name that can be used to refer to the bit and to the feature. M/O: Defines if the implementation of the feature is mandatory ("M") or optional ("O"). Description: A clear textual description of the feature. |  |  |  |

*** Next Change ***

A.2	N32 Handshake API

openapi: 3.0.0

info:

version: '1.3.1'

title: 'N32 Handshake API'

description: |

N32-c Handshake Service.

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

servers:

- url: '{apiRoot}/n32c-handshake/v1'

variables:

apiRoot:

default: https://example.com

description:  apiRoot as defined in clause 4.4 of 3GPP TS 29.501.

externalDocs:

description: 3GPP TS 29.573 V18.8.0; 5G System; Public Land Mobile Network (PLMN) Interconnection; Stage 3

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.573/

*** skipped for clarity ***

SecNegotiateReqData:

description: Defines the security capabilities of a SEPP sent to a receiving SEPP

type: object

required:

- sender

- supportedSecCapabilityList

properties:

sender:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Fqdn'

n32HandshakeId:

type: string

pattern: '^[A-Fa-f0-9]{16}$'

supportedSecCapabilityList:

type: array

items:

$ref: '#/components/schemas/SecurityCapability'

minItems: 1

3GppSbiTargetApiRootSupported:

type: boolean

default: false

# The attribute name does not follow the naming conventions specified in 3GPP TS 29.501. The attribute name is kept though as defined in the current specification for backward compatibility reason.

plmnIdList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

minItems: 1

snpnIdList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnIdNid'

minItems: 1

targetPlmnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

targetSnpnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnIdNid'

intendedUsagePurpose:

type: array

items:

$ref: '#/components/schemas/IntendedN32Purpose'

minItems: 1

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

senderN32fFqdn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Fqdn'

senderN32fPort:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

n32KeepaliveTimer:

type: intger

SecNegotiateRspData:

description: Defines the selected security capabilities by a SEPP

type: object

required:

- sender

- selectedSecCapability

properties:

sender:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Fqdn'

selectedSecCapability:

$ref: '#/components/schemas/SecurityCapability'

n32HandshakeId:

type: string

pattern: '^[A-Fa-f0-9]{16}$'

3GppSbiTargetApiRootSupported:

type: boolean

default: false

# The attribute name does not follow the naming conventions specified in 3GPP TS 29.501. The attribute name is kept though as defined in the current specification for backward compatibility reason.

plmnIdList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

minItems: 1

snpnIdList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnIdNid'

minItems: 1

allowedUsagePurpose:

type: array

items:

$ref: '#/components/schemas/IntendedN32Purpose'

minItems: 1

rejectedUsagePurpose:

type: array

items:

$ref: '#/components/schemas/IntendedN32Purpose'

minItems: 1

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

senderN32fFqdn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Fqdn'

senderN32fPortList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

minItems: 1

n32KeepaliveTimer:

type: intger

*** skipped for clarity ***

*** End of Change ***