3GPP TSG-CT WG4 Meeting #126	C4-245127

# Orlando, U.S; 18th – 22nd November 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.175 | CR | 0026 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of PS Data off feature for data channel |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2024-11-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to the agreed SA2 CR#1421 of TS23.228, a new optional parameter of PS Data Off Status is added in Nimsas_SessionEventControl API. This parameter is used to support the "Handling of PS data off exemption for services over IMS DC" based on the conclusions for KI#5 in TR 23.700-77. This CR propose to add the new optional parameter of PS Data Off Status in Nimsas_SessionEventControl API. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new optional parameter of PS Data Off Status in Nimsas_SessionEventControl API. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 2 solution is not implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.2.2, 6.1.6.3.3, 6.1.6.3.x, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR .23.228.. CR 1421 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible corrections to TS29175_Nimsas_SessionEventControl.yaml. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

6.1.6.2.2	Type: SessionEventNotification

Table 6.1.6.2.2-1: Definition of type SessionEventNotification

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| notificationEvent | NotificationEvent | M | 1 | The NotificationEvent corresponds to a report on one subscribed event. |
| sessionId | SessionId | M | 1 | The session ID is the identity of the IMS session for which the event relates to. This IE contains the information in the Call-ID header which is the typical header of SIP message. |
| sessionInfo | SessionInfo | C | 0..1 | The sessionInfo is the IMS session related information, e.g. the calling and called identity. It shall be contained when the attribute eventyType of the notificationEvent is one of the "SESSION_ESTABLISHMENT_REQUEST", "MEDIA_CHANGE_REQUEST". |
| mediaInfoList | map(MediaInfo) | C | 1..N | Media info list includes a list of media related information, e.g. media identity, media type and media specifications. The mediaId attribute within the MediaInfo data type is the key of the map. It shall be contained when the attribute eventyType of the notificationEvent is one of the "SESSION_ESTABLISHMENT_REQUEST", "SESSION_ESTABLISHMENT_PROGRESS", "SESSION_ESTABLISHMENT_ALERTING", "SESSION_ESTABLISHMENT_SUCCESS", "MEDIA_CHANGE_REQUEST", "MEDIA_CHANGE_SUCCESS", or "MEDIA_CHANGE_FAILURE". |
| psDataOffStatus | PsDataOffStatus | O | 0..1 | The psDataOffStatus indicate the active or inactive status of 3GPP PS Data Off of the UE the network is serving. It shall be contained when the attribute eventyType of the notificationEvent is "PS_DATA_OFF_STATUS_CHANGE". |

* * * Next Changes * * * *

##### 6.1.6.3.3	Enumeration: EventType

The enumeration EventType represents the type of the event for which the notification is generated. It shall comply with the provisions defined in table 6.1.6.3.3-1.

Table 6.1.6.3.3-1: Enumeration EventType

| Enumeration value | Description |
| --- | --- |
| "SESSION_ESTABLISHMENT_REQUEST" | Session establishment request event. |
| "SESSION_ESTABLISHMENT_PROGRESS" | Session establishment progress event. |
| "SESSION_ESTABLISHMENT_ALERTING" | Session establishment alerting event. |
| "SESSION_ESTABLISHMENT_SUCCESS" | Session establishment success event. |
| "SESSION_ESTABLISHMENT_FAILURE" | Session establishment failure event. |
| "MEDIA_CHANGE_REQUEST" | Media change request event. |
| "MEDIA_CHANGE_SUCCESS" | Media change success event. |
| "MEDIA_CHANGE_FAILURE" | Media change failure event. |
| "SESSION_TERMINATION" | Session termination event. |
| "PS_DATA_OFF_STATUS_CHANGE" | PS Data Off Status change event. |

* * * Next Changes * * * *

6.1.6.3.x	Enumeration: PsDataOffStatus

The enumeration PsDataOffStatus represents the active or inactive status of 3GPP PS Data Off of the UE the network is serving. It shall comply with the provisions defined in table 6.1.6.3.x-1.

Table 6.1.6.3.x-1: Enumeration PsDataOffStatus

| Enumeration value | Description |
| --- | --- |
| "ACTIVE" | The status of 3GPP PS Data Off of the UE is active. |
| "INACTIVE" | The status of 3GPP PS Data Off of the UE is inactive. |

* * * Next Changes * * * *

A.2	Nimsas_SessionEventControl API

openapi: 3.0.0

info:

title: 'IMS AS SessionEventControl Service'

version: 1.0.0

description: |

Nimsas_SessionEventControl Service.

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.175 V18.1.0; IP Multimedia Subsystem; IP Multimedia Subsystem (IMS) Application

Server (AS) Services; Stage 3

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.175/'

[…]

# STRUCTURED DATA TYPES

SessionEventNotification:

description: The session event notification item.

type: object

required:

- notificationEvent

- sessionId

properties:

notificationEvent:

$ref: '#/components/schemas/NotificationEvent'

sessionId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SessionId'

sessionInfo:

$ref: '#/components/schemas/SessionInfo'

mediaInfoList:

description: >

Contains a media related information. The mediaId attribute within the MediaInfo

data type is the key of the map.

type: object

additionalProperties:

$ref: '#/components/schemas/MediaInfo'

minItems: 1

psDataOffStatus:

$ref: '#/components/schemas/PsDataOffStatus'

[…]

EventInitiator:

description: The initiator of the event for which the notification is generated.

anyOf:

- type: string

enum:

- SERVED_IMS_SUBSCRIBER

- REMOTE_IMS_SUBSCRIBER

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

# ENUMS

EventType:

description: The type of the event for which the notification is generated.

anyOf:

- type: string

enum:

- SESSION_ESTABLISHMENT_REQUEST

- SESSION_ESTABLISHMENT_PROGRESS

- SESSION_ESTABLISHMENT_ALERTING

- SESSION_ESTABLISHMENT_SUCCESS

- SESSION_ESTABLISHMENT_FAILURE

- MEDIA_CHANGE_REQUEST

- MEDIA_CHANGE_SUCCESS

- MEDIA_CHANGE_FAILURE

- SESSION_TERMINATION

- PS_DATA_OFF_STATUS_CHANGE

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

PsDataOffStatus:

description: The active or inactive status of 3GPP PS Data Off of the UE the network is serving.

anyOf:

- type: string

enum:

- ACTIVE

- INACTIVE

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

* * * End of Changes * * * *