3GPP TSG-CT WG4 Meeting #126	C4-245137

Orlando, U.S; 18th – 22nd November 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.564 | CR | 0120 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UPF notifying the termination of the UPF event subscription |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | UPEAS_Ph2 |  |  |  |  |  | Date: |  |  | 2024-11-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | CR 23.501-5782 introduces new requirement to require the UPF to send a report to the NF service consumer that the subscription has been implicitly terminated e.g. when the corresponding PFCP session has been released i.e. the PDU session is not served by the UPF anymore. CR 23.502-5004 removes the UPF relocation indication in the notification report as the UPF has no idea if the PFCP session is released due to a relocation of the UPF, or a deactivation of the PDU session. It can only report the subscription shall be terminated since the PDU session is not served by the UPF anymore and include any event report which has not yet reported to the NF service consumer. It is proposed to introduce a new Event Type "Subscription Termination" similar to what is specified for AMF Event Exposure service. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The UPF may send a notification to report to the NF service consumer that the subscription has been terminated e.g. when the PDU session is not served by the UPF anymore, i.e. the corresponding PFCP session has been released. Introduce a new Event Type for Subscription Termination and a new data type SubTerminationReason. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 2 requirements are not aligned. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.3.1, 6.1.6.1, 6.1.6.2.3, 6.1.6.3.3, 6.2.6.3.x, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.501 CR 5782 TS 23.502 CR 5004 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The contribution introduces a new backwards compatible feature to Nupf_EventExposure API. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** First Change ***

#### 5.2.2.3	Notify

##### 5.2.2.3.1	General

The Notify service operation is invoked by the UPF, to send a notification, towards the notification URI, when certain event included in the subscription has taken place. See Figure 5.2.2.3.2-1.

For the events "USER_DATA_USAGE_MEASURES" and "USER_DATA_USAGE_TRENDS", the UPF shall use the HTTP method POST, using the notification URI received in the subscription creation as specified in clause 5.2.2.2.2, including e.g. the subscription ID, Event ID(s) for which event has happened, notification correlation ID provided by the NF service consumer at the time of event subscription, to send a notification.

If the subscription is targeting PDU sessions of any UE, i.e. the "anyUe" is set to true in the subscription creation request, the UPF shall perform the requested measurementsfor every PDU session that matches the event filter information (i.e. S-NSSAIs, DNNs, either Application ID(s) or traffic filters) and send notification(s) with multiple NotificationItem IEs within the NotificationData wherein each NotificationItem shall correspond to a report on one subscribed event per PDU session. If the subscription request included a sampling ratio, the notification may include the sampling ratio achieved by the UPF.

For the events "QOS_MONITORING" and "TSC_MNGT_INFO", the UPF shall use the HTTP method POST, using the notification URI received from the SMF via N4 interface, see clause 5.33.5 of 3GPP TS 29.244 [15].

For the event "USER_DATA_USAGE_MEASURES", the event notification may contain following information:

-	Volume Measurement: measurements of data volume exchanged (UL, DL and/or overall) and/or number of packets exchanged (UL, DL and/or overall) determined for the requested Granularity of Measurements.

-	Throughput Measurement: measurements of data throughput (UL and DL) determined for the requested Granularity of Measurements.

-	Application related Information: URLs and/or Domain information (Domain name and protocol) detected in the target traffic identified by the information included in the subscription request, e.g. an application id.

When the granularity of the measurement is per data flow, the notification shall include the packet filter set and the Applications Identifier if available.

For the event "USER_DATA_USAGE_TRENDS", the event notification may contain following information:

-	Throughput Statistic Measurement (average and/or peak throughput) over the measurement determined for the requested Granularity of Measurements.

When the granularity of the measurement is per data flow, the notification shall include the packet filter set and the Applications Identifier if available.

For the event "QOS_MONITORING", this service operation is used by the UPF to send the following types of event notifications:

-	Periodic notification on the downlink packet delay, uplink packet delay, and/or the round trip packet delay between the UPF (PSA) and UE;

-	Event triggered notification on the downlink packet delay, uplink packet delay, and/or the round trip packet delay between the UPF (PSA) and UE, i.e. when the packet delay exceeds a defined threshold;

-	Notification on the downlink packet delay, uplink packet delay, and/or the round trip packet delay between the UPF (PSA) and UE when the PDU session is released.

-	Event triggered notification of congestion information of the QoS flow on the UL and/or DL directions received from the NG-RAN, upon a change of the congestion information.

For the event "TSC_MNGT_INFO", the event notification may contain the following information:

-	Port Management Information Container(s) for one or more NW-TT ports and/or

-	a User Plane Node Management Information Container.

The event notification shall also contain the following information:

-	the related NW-TT port number(s), if Port Management Information Container(s) is present; and

-	the notification correlation ID received from the SMF, if any.

The event "SUBSCRIPTION_TERMINATION" may be implicitly subscribed in the UPF; and if so, the UPF shall send a notification to the NF service consumer to report the subscription has been terminated in the UPF with a termination cause, e.g. when the corresponding PFCP session is released by the SMF, i.e., the PDU session is not served by the UPF anymore. In the notification request message, the UPF shall include any event report that has been collected but not sent to the NF service consumer yet. See also clause 5.8.2.17 of 3GPP TS 23.501 [2] and clause 4.15.4.5 of 3GPP TS 23.502 [3].

# *** Next Change ***

#### 6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Nupf_EventExposure service.

Table 6.1.6.1-1: Nupf_EventExposure specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| NotificationData | 6.1.6.2.2 | The list of NotificationItems |  |
| NotificationItem | 6.1.6.2.3 | Represents a report on one subscribed event |  |
| QosMonitoringMeasurement | 6.1.6.2.4 | QoS Monitoring Measurement information |  |
| UserDataUsageMeasurements | 6.1.6.2.5 | User Data Usage Measurements |  |
| VolumeMeasurement | 6.1.6.2.6 | Volume Measurement |  |
| ThroughputMeasurement | 6.1.6.2.7 | Throughput Measurement |  |
| ApplicationRelatedInformation | 6.1.6.2.8 | Application Related Information |  |
| ThroughputStatisticMeasurement | 6.1.6.2.9 | Throughput Statistic Measurement |  |
| DomainInformation | 6.1.6.2.10 | Domain Name and Domain Name Protocol |  |
| UpfEventSubscription | 6.1.6.2.11 | Represents an individual event subscription resource on UPF |  |
| UpfEventMode | 6.1.6.2.12 | Describes how the reports shall be generated for a subscribed event |  |
| UpfEvent | 6.1.6.2.13 | Describes an event to be subscribed |  |
| CreateEventSubscription | 6.1.6.2.14 | Data within a create UPF event subscription request |  |
| CreatedEventSubscription | 6.1.6.2.15 | Data within a create UPF event subscription response |  |
| ReportingSuggestionInformation | 6.1.6.2.16 | Reporting Suggestion Information |  |
| TscManagementInfo | 6.1.6.2.17 | TSC Management Information |  |
| EventType | 6.1.6.3.3 | Event Type |  |
| UpfEventTrigger | 6.1.6.3.4 | Describes how the UPF generates the report for the event |  |
| MeasurementType | 6.1.6.3.5 | Type of Measurement |  |
| GranularityOfMeasurement | 6.1.6.3.6 | Granularity Of Measurement |  |
| DnProtocol | 6.1.6.3.7 | Domain Name Protocol |  |
| ReportingUrgency | 6.1.6.3.8 | Reporting Urgency |  |
| SubTerminationReason | 6.1.6.3.x | Subscription Termination Reason |  |

Table 6.1.6.1-2 specifies data types re-used by the Nupf_EventExposure service from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nupf_EventExposure service.

Table 6.1.6.1-2: Nupf_EventExposure re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| DateTime | 3GPP TS 29.571 [16] | Date time |  |
| Dnn | 3GPP TS 29.571 [16] | DNN |  |
| Gpsi | 3GPP TS 29.571 [16] | GPSI |  |
| Snssai | 3GPP TS 29.571 [16] | S-NSSAI |  |
| Uint32 | 3GPP TS 29.571 [16] | Uint32 |  |
| MacAddr48 | 3GPP TS 29.571 [16] | MAC Address |  |
| Ipv4Addr | 3GPP TS 29.571 [16] | IPv4 address |  |
| Ipv6Prefix | 3GPP TS 29.571 [16] | IPv6 address prefix |  |
| Uint64 | 3GPP TS 29.571 [16] | Unsigned 64-bit integer |  |
| BitRate | 3GPP TS 29.571 [16] | Bit rate |  |
| PacketRate | 3GPP TS 29.571 [16] | Packet rate |  |
| TrafficVolume | 3GPP TS 29.571 [16] | Traffic Volume |  |
| ApplicationId | 3GPP TS 29.571 [16] | The application identifier. |  |
| DurationSec | 3GPP TS 29.571 [16] |  |  |
| NotificationFlag | 3GPP TS 29.571 [16] | Notification flag. |  |
| PartitioningCriteria | 3GPP TS 29.571 [16] | Used to partition UEs before applying sampling. |  |
| ProblemDetails | 3GPP TS 29.571 [16] |  |  |
| SamplingRatio | 3GPP TS 29.571 [16] | Sampling Ratio. |  |
| Uri | 3GPP TS 29.571 [16] |  |  |
| IpAddr | 3GPP TS 29.571 [16] |  |  |
| SupportedFeatures | 3GPP TS 29.571 [16] |  |  |
| Supi | 3GPP TS 29.571 [16] |  |  |
| Pei | 3GPP TS 29.571 [16] |  |  |
| Uinteger | 3GPP TS 29.571 [16] | Unsigned Integer |  |
| PortManagementContainer | 3GPP TS 29.512 [19] | PMIC |  |
| BridgeManagementContainer | 3GPP TS 29.512 [19] | UMIC |  |
| FlowInformation | 3GPP TS 29.512 [19] | IP or Ethernet Flow Information |  |
| PatchItem | 3GPP TS 29.571 [16] | Patch item of JSON PATCH |  |
| PatchResult | 3GPP TS 29.571 [16] | Patch result of JSON PATCH |  |
| MutingExceptionInstructions | 3GPP TS 29.571 [16] | Muting exception instructions. |  |
| MutingNotificationsSettings | 3GPP TS 29.571 [16] | Muting notifications settings. |  |

# *** Next Change ***

##### 6.1.6.2.3	Type: NotificationItem

Table 6.1.6.2.3-1: Definition of type NotificationItem

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| eventType | EventType | M | 1 | The event type of the event for which the notification is generated. |  |
| ueIpv4Addr | Ipv4Addr | C | 0..1 | IPv4 address of the UE (NOTE 1, NOTE 3) |  |
| ueIpv6Prefix | Ipv6Prefix | C | 0..1 | IPv6 address prefix of the UE (NOTE 1, NOTE 3) |  |
| ueMacAddr | MacAddr48 | O | 0..1 | MAC address of the UE. (NOTE 2, NOTE 3) |  |
| dnn | Dnn | O | 0..1 | When present, this attribute indicates the DNN of the PDU session for which the notification is generated. |  |
| snssai | Snssai | O | 0..1 | When present, this attribute indicates the S-NSSAI of the PDU session for which the notification is generated. |  |
| gpsi | Gpsi | O | 0..1 | When present, this attribute indicates the GPSI of the UE for which the notification is generated. |  |
| supi | Supi | O | 0..1 | Subscription Permanent Identifier |  |
| timeStamp | DateTime | M | 1 | The value represents the UTC time when the information in this report was generated. |  |
| startTime | DateTime | O | 0..1 | When present, this attribute shall provide the timestamp when the information measured for generating this report was started. |  |
| qosMonitoringMeasurement | QosMonitoringMeasurement | C | 0..1 | This attribute shall be present if eventType is set to "QOS_MONITORING". |  |
| userDataUsageMeasurements | array(UserDataUsageMeasurements) | C | 1..N | This IE shall be present if eventType is set to "USER_DATA_USAGE_MEASURES" or "USER_DATA_USAGE_TRENDS". |  |
| tscMngtInfo | TscManagementInfo | C | 0..1 | This attribute shall be present if eventType is set to "TSC_MNGT_INFO". |  |
| termReason | SubTerminationReason | C | 0..1 | This IE shall be present when the event type is SUBSCRIPTION_TERMINATION. When present, this IE shall indicate the reason for the subscription termination. |  |
| NOTE 1: At least one of ueIpv4Addr and ueIpv6Prefix shall be present if the subscription applies to an IP PDU session. NOTE 2: An NF service consumer subscribing to receive QoS Monitoring Measurement report for an ethernet PDU session shall accept the NotificationItem having neither ueIpv4Addr nor ueIpv6Prefix. NOTE 3: At least one of ueIpv4Addr, ueIpv6Prefix and ueMacAddr shall be present. |  |  |  |  |  |

# *** Next Change ***

##### 6.1.6.3.3	Enumeration: EventType

The enumeration EventType represents the type of event to which the NF service consumer may subscribe to and for which the notification is generated. It shall comply with the provisions defined in table 6.1.5.3.3-1.

Table 6.1.6.3.3-1: Enumeration EventType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| "QOS_MONITORING" | QoS Monitoring Measurement (see clause 5.2.1.3.2) |  |
| "USER_DATA_USAGE_MEASURES" | User Data Usage Measures (see clause 5.2.1.3.3) |  |
| "USER_DATA_USAGE_TRENDS" | User Data Usage Trends (see clause 5.2.1.3.4) |  |
| "TSC_MNGT_INFO" | TSC Management Information |  |
| "SUBSCRIPTION_TERMINATION" | This event type is used by the UPF to inform the NF service consumer that the subscription is terminated at the UPF, e.g. when the PDU session is not served by the UPF anymore, i.e. the corresponding PFCP session has been released. |  |

# *** Next Change ***

##### 6.2.6.3.X	Enumeration: SubTerminationReason

Table 6.2.6.3.x-1: Enumeration SubTerminationReason

| Enumeration value | Description |
| --- | --- |
| "PDU_SESSION_NOT_SERVED" | Indicates that the subscription is terminated because the PDU session corresponding to the subscription is not anymore served by the UPF, i.e. the corresponding PFCP session has been released. |
| "SUB_TERMINATION" | The subscription is terminated due to unscribing triggered by the SMF |  |

# *** Next Change ***

# A.2	Nupf_EventExposure API

openapi: 3.0.0

info:

title: 'UPF Event Exposure Service'

version: 1.1.1

description: |

UPF Event Exposure Service.

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.564 V18.6.0; 5G System; User Plane Function Services; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.564/

servers:

- url: '{apiRoot}/nupf-ee/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

******skipped for clarity******

NotificationItem:

description: represents a report on one subscribed event

type: object

required:

- eventType

- timeStamp

anyOf:

- required: [ ueIpv4Addr ]

- required: [ ueIpv6Prefix ]

- required: [ ueMacAddr ]

properties:

eventType:

$ref: '#/components/schemas/EventType'

ueIpv4Addr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

ueIpv6Prefix:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Prefix'

ueMacAddr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MacAddr48'

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

timeStamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

startTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

qosMonitoringMeasurement:

$ref: '#/components/schemas/QosMonitoringMeasurement'

tscMngtInfo:

$ref: '#/components/schemas/TscManagementInfo'

userDataUsageMeasurements:

type: array

items:

$ref: '#/components/schemas/UserDataUsageMeasurements'

minItems: 1

termReason:

$ref: '#/components/schemas/SubTerminationReason'

******skipped for clarity******

EventType:

description: Event Type

anyOf:

- type: string

enum:

- QOS_MONITORING

- USER_DATA_USAGE_MEASURES

- USER_DATA_USAGE_TRENDS

- TSC_MNGT_INFO

- SUBSCRIPTION_TERMINATION

- type: string

******skipped for clarity******

SubTerminationReason:

description: Subscription Termination Reason.

anyOf:

- type: string

enum:

- PDU_SESSION_NOT_SERVED

- SUB_TERMINATION

- type: string

******skipped for clarity******

# *** End of Changes ***