# 3GPP TSG-CT WG4 Meeting #126	C4-245141

# Orlando, U.S; 18th – 22nd November 2024	(revision of C4-24xxxx)

# Source:	Samsung

# Title:	New WID on Reducing Information Exposure over SBI

# Document for:	Approval

# Agenda Item:	19.2.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: 	 Reducing Information Exposure over SBI

Acronym:	RedInfExp_SBI

Unique identifier:	xxxxxxx

Potential target Release:	Rel-19

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | x |  |
| No | x | x | x |  |  |
| Don't know |  |  |  |  | x |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
| X | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A | N/A | N/A | N/A |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 1000001 | Study on Reducing Information Exposure over SBI | CT4 study on Reducing Information Exposure over SBI |

# 3	Justification

APIs defined by SBI currently does not limit the amount of information exposed to different NF-Service Consumers. The result of this is that NF receives excessive data, even if it does not require it and there is no mechanism to reduce or avoid communication of excessive data that is not required by the consumer.Therefore, a mechanism is needed to strike a balance between data exposure aspects and efficiency/performance between the NFs.

Against this background, CP#100 has approved a SID to study on Reducing Information Exposure over SBI (FS_RedInfExp_SBI), please see CP-231350. In this SID, the following three key issues are studied:

- Key Issue #1: Excessive Data Exposure over SBI;

- Key Issue #2: Access to dis-allowed Data;

- Key Issue #3: Access to dis-allowed Resource Segments Access to dis-allowed Resource Segments ;

Based on various discussions in CT4, now the SID has reached completion. Conclusions for all of the 3 key issues have been agreed. As documented in TR 29.857, only KI#1 & 3 have normative impacts.

Considering the progress of the SID, a WID on Reducing Information Exposure over SBI is needed.

# 4	Objective

The aim of this work item is to specify the solutions that concluded to be standardized in TR 29.857 Clause 8 Conclusions.

Stage 3 objectives:

CT4:

- Changes to NRF APIs to allow NF-Producers register resource content filter definitions into NRF.

- Changes to NRF APIs to allow inclusion of resource content filter IDs in access-tokens, which is only applicable for the particular token type, captured in the 3GPP TS 33.501 clause 13.4.1.1.2. Access token request for accessing services of a specific NF Service Producer instance / NF Service producer service instance.

- Inclusion of one or more filters specifying information to be excluded in the resource retrieval requests by NF-Consumers

- Extend the NF profile by introducing a service scenario indication as new attribute in each NF information data type (e.g. UpfInfo, SmfInfo, BsfInfo) to identify the specific service scenario. The service scenario indication can be flexible specified by operators depending on different implementation purposes.

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 29.510 | - Changes to NRF APIs to allow NF-Producers register resource content filter definitions into NRF. - Changes to NRF APIs to allow inclusion of resource content filter IDs in access-tokens. - Extending xxxInfo attributes with service scenario indication, wherever applicable | TSG#110 (Sep. 2025) | CT4 responsibility (stage 3) |
| 29.500 | - Inclusion of one or more filter specifying information to be excluded in the resource retrieval requests by NF-Consumers | TSG#110 (Sep. 2025) | CT4 responsibility (stage 3) |

# 6	Work item Rapporteur(s)

Nivedya P S, Samsung, nivedya.ps@samsung.com

# 7	Work item leadership

CT4

# 8	Aspects that involve other WGs

# 9	Supporting Individual Members

Supporting IM name

Samsung

Nokia

CMCC

CATT

Intel