3GPP TSG-CT WG4 Meeting #126	C4-245309

# Orlando, U.S; 18th – 22nd November 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.501 | CR | 0175 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Query parameter clarification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-CT |  |  |  |  |  | Date: |  |  | 2024-11-19 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Clause 5.3.13 specifies the format of structured query parameters, mandating to include a "content:" block, and specifying the "application/json" media type. However, some 5GC SBI APIs fail to follow this guideline (see e.g. ip-addrs in 29.522), while implementations of these APIs assume content: application/json as default. Similarly for omission of the "style" keyword set to "form" and the "explode" keyword set to "false". |  |  |  |  |  |  |  |  |
| Summary of change: |  | In the absence of explicit media type specification for structured query parameters, “application/json” shall be assumed as default. Similarly "style: form" and "explode: false" shall be assumed as default for arrays of simple type. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear stage 3 specification may result in interoperability problems. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 5.3.13	Formatting of structured data types in query parameters

Structured data types shall represent JSON objects or arrays.

When used in query parameters of a URI, the following formatting shall be used:

-	JSON objects and arrays of JSON objects: they shall be formatted using the JSON syntax, which is specified in OpenAPI Specification [4] by including a "content:" block, and specifying the "application/json" media type, followed by the OpenAPI definition of the object.

EXAMPLE:

- name: plmn-id

in: query

content:

application/json:

schema:

type: object

properties:

mcc:

type: string

mnc:

type: string

This results in the following formatting:

.../resource?plmn-id={"mcc":"123","mnc":"456"}

When an 5GC SBI API omits the inclusion of a "content:" block, a "content:" block specifying the "application/json" media type shall be assumed as default.

-	Arrays of simple types: they shall be formatted using the OpenAPI "style" keyword set to "form" and the "explode" keyword set to "false".

EXAMPLE:

- name: service-names

in: query

style: form

explode: false

schema:

type: array

items:

type: string

This results in the following formatting:

.../resource?service-names=service1,service2,service3

When an 5GC SBI API omits the inclusion of the "style" keyword set to "form" and the "explode" keyword set to "false", "style: form" and "explode: false" shall be assumed as default.

As described in 3GPP TS 29.500 [2], clause 5.2.10, when the URI is composed, it shall escape (percent-encode) certain "reserved" characters. When an array of strings is formatted with "explode: false" keyword, the COMMA (U+002C) character is used as separator of the different string values; this implies that this character (the value separator) shall not be escaped, while any other comma used inside the different string values shall be escaped (percent-encoded).

* * * End of Change * * * *