3GPP TSG-CT WG4 Meeting #126	C4-245358

# Orlando, U.S; 18th – 22nd November 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.336 | CR | 0187 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 29.336 MPS for Messaging Indicator on S6t |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Peraton Labs, CISA ECD, AT&T, T-Mobile USA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | MPS4msg |  |  |  |  |  | Date: |  |  | 2024-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | An indicator is required to inform IMS and core network functional entities whether or not MPS is active for text messaging. The S6t interface needs to convey the indicator from the SCEF to the HSS for reference by both IMS messaging as well as SMS over NAS involved functional entities. TS 23.228 § 5.21 IMS Multimedia Priority Services Procedures: "The HSS shall store the IMS Priority Indication, the Priority Level, and the MPS for Messaging indication as part of the subscription information/user profile." TS 23.682 § 5.18 Procedure for Network Parameter Configuration via SCEF: "The SCEF sends a Set Suggested Network Configuration Request (… MPS for Messaging indication) message to the HSS" |  |  |  |  |  |  |  |  |
| Summary of change: |  | First change: The HSS shall store the MPS for Messaging indication only if the UE has an MPS subscription in the HSS. Second change: Added the MPS-Messaging-Indication AVP to the list of AVPs. Third change: Added the MPS-Messaging-Indication AVP to the Suggested-Network-Configuration AVP. Fourth change: Created a new MPS-Messaging-Indication AVP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MPS users will not have priority for SMS messaging. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.1.2, 8.4.1, 8.4.66, 8.4.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# ***** First change *****

#### 7.2.1.2	Detailed Behaviour of the HSS

When the Configuration Information Request is received from the SCEF, the HSS shall, in the following order:

1.	Check that the User Identity for whom data is asked exists in HSS. If not, Experimental-Result shall be set to DIAMETER_ERROR_USER_UNKNOWN (5001) in the Configuration Information Answer.

2.	Check whether the requesting SCEF is authorized to request the specified service (e.g. presence of Monitoring Event Configuration AVPs indicates the MONTE service). If not, Experimental-Result shall be set to DIAMETER_ERROR_UNAUTHORIZED_REQUESTING_ENTITY (5510) in the Configuration Information Answer.

2a.	If the MTC Provider Information is received, check if the MTC Provider ID is authorized for the UE or the group of UEs. If not, Experimental-Result shall be set to DIAMETER_ERROR_UNAUTHORIZED_MTC_PROVIDER (5516) in the Configuration Information Answer.

3.	Check that the requested service (e.g. MONTE, indicated by the presence of Monitoring-Event-Configuration AVPs, or Communication Pattern, indicated by the presence of AESE-Communication-Pattern AVP) is authorized for the UE or the group of UEs. If not, Experimental-Result shall be set to DIAMETER_ERROR_UNAUTHORIZED_SERVICE (5511) in the Configuration Information Answer.

3a.	If the request is IMSI Group Id Retrieval (as indicated by the CIR-Flags AVP), the HSS shall return the IMSI Group Id(s) in the order of sequence that corresponds to the order of sequence of the received External Group Identifier(s) received in the User-Identifier AVP and AdditionalIdentifiers AVP. The Result Code shall be set to DIAMETER_SUCCESS. Stop processing.

Otherwise, continue to step 4.

NOTE:	Applicable steps 4-8 are performed for each monitoring event type or communication pattern requested.

4.	Check whether the limits on number of monitoring events that can be requested per monitoring type and SCEF-ID is reached. If so, Result-Code in Monitoring-Event-Config-Status for the affected service(s) shall be set to DIAMETER_RESOURCES_EXCEEDED (5006). The Result-Code shall be set to DIAMETER_SUCCESS in the Configuration Information Answer.

4a.	Check whether the requesting SCEF is authorized to request the specific service configuration(s) (e.g. Monitoring Event Type indicates a specific monitoring event configuration). If not, Experimental-Result-Code in Monitoring-Event-Config-Status for the affected service configuration(s) shall be set to DIAMETER_ERROR_UNAUTHORIZED_REQUESTING_ENTITY (5510). The Result-Code shall be set to DIAMETER_SUCCESS in the Configuration Information Answer.

4b.	Check that the requested specific service configuration(s) (e.g. Monitoring Event Type) is authorized for the UE or the group of UEs. If not, Experimental-Result-Code in Monitoring-Event-Config-Status for the affected service(s) shall be set to DIAMETER_ERROR_UNAUTHORIZED_SERVICE (5511). The Result-Code shall be set to DIAMETER_SUCCESS in the Configuration Information Answer.

5.	When the request is for a group, i.e. because the External-Identifier AVP of the User-Identifier AVP contain an External Group Identifier, the HSS applies the Monitoring-Event-Configuration AVP to each UE of the Group and includes the CIA-Flags AVP with the Group-Configuration-In-Progress bit set in the Configuration Information Answer. The Result-Code shall be set to DIAMETER_SUCCESS in the Configuration Information Answer.

If the CIR command is to delete configured monitoring event(s) for certain UE(s) in a group of UEs, i.e. Monitoring-Event-Configuration AVP(s) containing Exclude-Identifier AVP, and the HSS supports Dynamic-Group-Event-Monitoring feature, the HSS shall cancel the monitoring event(s) for the UE(s) indicated by Exclude-Identifier AVP and keep monitoring events(s) for other UEs in the group, as specified in clause 5.6.1 of 3GPP TS 23.682 [2].

If the CIR command is to add configured monitoring event(s) for certain UE(s) in a group of UEs, i.e. Monitoring-Event-Configuration AVP(s) containing Include-Identifier AVP, and the HSS supports Dynamic-Group-Event-Monitoring feature, the HSS shall create the monitoring event(s) for the UE(s) indicated by Include-Identifier AVP, as specified in clause 5.6.1 of 3GPP TS 23.682 [2].

6.	If a serving node is registered and is involved in the reporting of the configured monitoring event, the HSS shall forward the monitoring event configuration to the serving node and wait for the answer before sending the Configuration Information Answer to the SCEF. The monitoring event configuration status from the serving node for each event shall be conveyed by the HSS to the SCEF. The Result-Code shall be set to DIAMETER_SUCCESS in the Configuration Information Answer.

7.	If the user is not registered in any serving node or the user is purged in the registered serving node(s), the HSS shall store the configuration data related to the service; also, it shall indicate to the SCEF that the user is absent, in the Configuration Information Answer, by setting the relevant bit in the S6t-HSS-Cause IE. The Result-Code shall be set to DIAMETER_SUCCESS in the Configuration Information Answer.

8.	For Monitoring, if the data related to an immediate reporting is available in the HSS, the HSS (e.g. as being received from the MME/SGSN in the Insert Subscriber Data answer) shall include this data in the Configuration Information Answer.For Monitoring events that can be answered by the HSS without requiring interation with the serving node (e.g. roaming status, IMEI-Change), the HSS shall respond immediately, either in the CIA or in an immediate RIR.

If the HSS is aware that the UE is registered in an MME and an SGSN and the services supported by the MME and SGSN are different, the HSS shall additionally report the capabilities of the different nodes to the SCEF by including Supported-Services AVP(s) with the subset of services supported by the serving node(s) among those supported by the HSS, as indicated by the Node-Type AVP (e.g. HSS, MME). If the capabilities are the same reported from the MME, the SGSN and the HSS, the HSS shall report the service capabilities without Node-Type to the SCEF. If the Supported-Services of the SGSN and MME are different and the UE is purged in SGSN or MME the HSS shall report the Supported-Services to the SCEF excluding the Supported-Services from the purged node.

If the HSS receives CIR command from SCEF and has sent IDR commands over S6a/S6d to an MME and an SGSN, the HSS shall check the Result-Codes in Monitoring-Event-Config-Status AVPs reported by the MME and the SGSN:

-	if they are different, the HSS shall include both Service-Report AVPs with the respective Node-Type (indicating the type of serving node) in the Monitoring-Event-Config-Status AVP to the SCEF in the CIA command;

-	otherwise, the HSS shall include one Service-Report AVP without Node-Type in the Monitoring-Event-Config-Status AVP to the SCEF in the CIA command.

If the configuration data in the CIR command are out of the allowed range, the HSS shall set the Experimental-Result-Code in Monitoring-Event-Config-Status for the affected service(s) to DIAMETER_ERROR_REQUESTED_RANGE_IS_NOT ALLOWED (5512).

If the received SCEF Reference ID for Deletion does not exist, the HSS shall set the Experimental-Result-Code in Monitoring-Event-Config-Status for the affected service(s) to DIAMETER_ERROR_CONFIGURATION_EVENT_NON_EXISTANT (5514).

If the SCEF Reference ID exists and the old configuration data could not be replaced by new Configuration event data, the HSS shall set the Experimental-Result-Code in Monitoring-Event-Config-Status for the affected service(s) to DIAMETER_ERROR_CONFIGURATION_EVENT_STORAGE_NOT_SUCCESSFUL (5513).

If the HSS cannot fulfil the received request for reasons not stated in the above steps (e.g. due to a database error), it shall stop processing the request and set Result-Code in Configuration Information Answer to DIAMETER_UNABLE_TO_COMPLY.

If the HSS detects that configuration data provided by a Monitoring-Event-Configuration are overwritten by a CIR command and both configurations were initiated by the same SCEF, the HSS shall include in the CIA command the Monitoring Event Report AVP including the SCEF Reference ID and SCEF Reference ID for Deletion of the overwritten configuration data.

If the HSS needs to report loss of connectivity it shall include the Monitoring-Type AVP set to "LOSS_OF_CONNECTIVITY" in the Monitoring Event Report. In addition, the HSS may also include the Loss-Of-Connectivity-Reason AVP in the Monitoring Event Report.

For the "CHANGE_OF_IMSI_IMEI(SV)_ASSOCIATION" event type, if there was no previous IMEI(SV) information stored in the HSS, and a new IMEI(SV) is received from the MME/SGSN, this is equivalent to a change of IMEI(SV) and the corresponding event shall be reported, by setting the "IMEI" flag (bit 0) in the IMEI-Change AVP, as described in clause 8.4.22.

For the "ROAMING_STATUS" event type, if there was no previous roaming status information stored in the HSS, and a certain PLMN ID value is received from the MME/SGSN, this is equivalent to a change of roaming status and the corresponding event shall be reported.

If the SCEF indicates the support of Monitoring event feature to the HSS and the HSS supports Monitoring. The HSS shall include the Supported-Services AVP with the Supported-Monitoring-Events AVP indicating the authorized monitoring events for the requesting SCEF in the CIA command.

If CIR message includes multiple SCEF Reference ID and one, several or all SCEF Reference ID Monitoring events cannot be handled, or fail to be configured at the registered serving node(s), the HSS shall report all SCEF Reference ID requested to the SCEF with an appropriate Experimental-Result-Code or Result-Code in the Monitoring-Event-Config-Status AVP(s).

NOTE:	In order to avoid reattempts from the SCS/AS when some configuration failed at the serving node(s), based on operator's choice, the HSS can still store the configuration data, return a successful response to SCEF and report the SCEF with monitoring event(s) suspended. The configuration data can then be forwarded whenever traffic is received from MME/SGSN and event(s) can be reported to SCEF as resumed, as described in clause 7.2.2.

If a CIR message includes multiple SCEF Reference ID and for a SCEF Reference ID at least one CP parameter set cannot be handled, the HSS shall reply within the AESE-Communication-Pattern-Config-Status all SCEF Reference ID requested to the SCEF with an appropriate Experimental-Result-Code or Result-Code.

If an SCEF Reference ID received in a CIR command match with an SCEF Reference ID stored in the HSS and both SCEF Reference ID are provided by the same SCEF ID, the HSS shall delete the stored CP sets associated with the SCEF reference Id and store the new CP set(s).

If the CIR message contains combinations of monitoring events and CP parameter set it shall handle each set belonging to an SCEF Reference ID separately and shall send a combined answer to the SCEF with all SCEF Reference IDs requested.

If the SCEF Reference ID for Deletion is present, the receiving node shall delete the corresponding configuration, if stored.

If the SCEF Reference ID is present, the receiving node shall store the configuration event.

IF the CIR message contains the CIR-Flags with delete all monitoring events, the HSS shall delete all Monitoring events configured by the SCEF for the subscriber. This includes forwarding the deletion to involved serving nodes. In this case the CIR message shall not contain new monitoring events.

If the CIR message contains the CIR-Flags with delete all communication pattern, and the feature Config-Eff-CP is supported, the HSS shall delete all Communication Pattern configured by the SCEF for the subscriber. This includes forwarding the deletion to involved serving nodes. In this case the CIR message shall not contain new communication pattern.

If the CIR message contains the CIR-Flags with delete all network parameter configurations, and the feature Config-Eff-NP is supported, the HSS shall delete all Suggested Network Configurations configured by the SCEF for the subscriber. This includes the updating of involved serving nodes. In this case the CIR message shall not contain new network parameter configurations.

If the CIR command contains the CIR-Flags AVP with the bit for Enhanced Coverage Query set, the HSS shall return the current settings of Enhanced Coverage together with the current Serving PLMN-ID (if any) in the CIA command.

If the CIR command contains Enhanced-Coverage-Restriction AVP, the HSS shall update the subscription data for Enhanced Coverage; the update shall be a complete replacement of any stored information with the received information. This may result in the need to update the MME/SGSN via S6a/d/MAP-Gr with the new value for access restriction; there is however no need for the HSS to delay sending of CIA until updates of serving nodes are confirmed.

If the CIR command contains Suggested-Network-Configuration AVP to update a previous Suggested Network Configuration (i.e. the SCEF Reference ID matches with an SCEF Reference ID stored in the HSS and both SCEF Reference IDs are provided by the same SCEF ID), the HSS shall check whether the suggested parameters are acceptable, and if so, update the stored UE's subscription parameters accordingly. The update shall be a complete replacement of any stored information with the received information. This may result in Insert Subscriber Data Signalling towards the UE's serving node. If the HSS modifies parameters received in Suggested-Network-Configuration AVP, the HSS shall inform the SCEF about the active parameters. When the HSS did not modify parameters received in the Suggested-Network-Configuration AVP, the HSS shall return a Suggested-Network-Configuration AVP to the SCEF in CIA containing only SCEF Reference ID and SCEF-ID.

If a Monitoring-Event-Report is included in a CIA command and the Monitoring-Event-Report is related to a One-time Monitoring event request, the HSS shall delete the related Monitoring event for the UE or the individual group member UE locally after sending the Monitoring-Event-Report.

If the subscribed periodic RAU/TAU Timer, Active Time, Suggested number of downlink packets, or any combination was previously set by a different SCEF requests including the Monitoring-Event-Configuration AVP(s) or Suggested-Network-Configuration AVP identified by a different SCEF Reference ID for the same UE, the HSS shall accept the request as follows:

-	If the newly received the value of the Maximum-Detection-Time AVP or the Maximum-Latency AVP within the Monitoring-Event-Configuration AVP(s) or the value of the Subscribed-Periodic-RAU-TAU-Timer AVP within the Suggested-Network-Configuration AVP is lower than the provided subscribed periodic RAU/TAU timer, the HSS shall set the subscribed periodic RAU/TAU timer using the newly received value.

-	If the newly received the value of Maximum-Response-Time AVP within the Monitoring-Event-Configuration AVP(s) or the value of the Active-Time AVP within the Suggested-Network-Configuration AVP is higher than the provided subscribed Active Time (i.e. previously provided Maximum Response Time), the HSS shall set the subscribed Active Time using the newly received value.

-	If the DL-Buffering-Suggested-Packet-Count AVP is newly received within the Monitoring-Event-Configuration AVP(s) or Suggested-Network-Configuration AVP, the HSS shall set the subscribed Suggested number of downlink packets by adding the newly received value to the currently used value of Suggested number of downlink packets if the aggregated value is within the operator defined range. If the aggregated value is not within the operator defined range, the HSS shall set the subscribed Suggested number of downlink packets according to operator defined range.

If the MPS-Messaging-Indication is present, and if allowed by local policy, the HSS shall store the MPS for Messaging indication only if the UE has an MPS subscription in the HSS.

# ***** Second change *****

### 8.4.1	General

The following table specifies the Diameter AVPs defined for the S6t interface protocol, their AVP Code values, types, possible flag values and whether or not the AVP may be encrypted. The Vendor-ID header of all AVPs defined in this specification shall be set to 3GPP (10415).

For all AVPs which contain bit masks and are of the type Unsigned32, bit 0 shall be the least significant bit. For example, to get the value of bit 0, a bit mask of 0x00000001 should be used.

Table 8.4.1-1: S6t specific Diameter AVPs

|  |  |  |  | AVP Flag rules |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Attribute Name | AVP Code | Clause defined | Value Type | Must | May | Should not | Must not | May Encr. |
| AESE-Communication-Pattern | 3113 | 8.4.25 | Grouped | M,V |  |  |  | No |
| Communication-Pattern-Set | 3114 | 8.4.26 | Grouped | M,V |  |  |  | No |
| Periodic-Communication-Indicator | 3115 | 8.4.27 | Unsigned32 | M,V |  |  |  | No |
| Communication-Duration-Time | 3116 | 8.4.28 | Unsigned32 | M,V |  |  |  | No |
| Periodic-time | 3117 | 8.4.29 | Unsigned32 | M,V |  |  |  | No |
| Scheduled-Communication-Time | 3118 | 8.4.30 | Grouped | M,V |  |  |  | No |
| Stationary-Indication | 3119 | 8.4.31 | Unsigned32 | M,V |  |  |  | No |
| AESE-Communication-Pattern-Config-Status | 3120 | 8.4.32 | Grouped | M,V |  |  |  | No |
| AESE-Error-Report | 3121 | 8.4.33 | Grouped | M,V |  |  |  | No |
| Monitoring-Event-Configuration | 3122 | 8.4.2 | Grouped | M,V |  |  |  | No |
| Monitoring-Event-Report | 3123 | 8.4.3 | Grouped | M,V |  |  |  | No |
| SCEF-Reference-ID | 3124 | 8.4.4 | Unsigned32 | M,V |  |  |  | No |
| SCEF-ID | 3125 | 8.4.5 | DiameterIdentity | M,V |  |  |  | No |
| SCEF-Reference-ID-for-Deletion | 3126 | 8.4.6 | Unsigned32 | M,V |  |  |  | No |
| Monitoring-Type | 3127 | 8.4.7 | Unsigned32 | M,V |  |  |  | No |
| Maximum-Number-of-Reports | 3128 | 8.4.8 | Unsigned32 | M,V |  |  |  | No |
| UE-Reachability-Configuration | 3129 | 8.4.9 | Grouped | M,V |  |  |  | No |
| Monitoring-Duration | 3130 | 8.4.10 | Time | M,V |  |  |  | No |
| Maximum-Detection-Time | 3131 | 8.4.11 | Unsigned32 | M,V |  |  |  | No |
| Reachability-Type | 3132 | 8.4.12 | Unsigned32 | M,V |  |  |  | No |
| Maximum Latency | 3133 | 8.4.13 | Unsigned32 | M,V |  |  |  | No |
| Maximum Response Time | 3134 | 8.4.14 | Unsigned32 | M,V |  |  |  | No |
| Location-Information-Configuration | 3135 | 8.4.15 | Grouped | M,V |  |  |  | No |
| MONTE-Location-Type | 3136 | 8.4.16 | Unsigned32 | M,V |  |  |  | No |
| Accuracy | 3137 | 8.4.17 | Unsigned32 | M,V |  |  |  | No |
| Association-Type | 3138 | 8.4.18 | Unsigned32 | M,V |  |  |  | No |
| Roaming-Information | 3139 | 8.4.19 | Unsigned32 | M,V |  |  |  | No |
| Reachability-Information | 3140 | 8.4.20 | Unsigned32 | M,V |  |  |  | No |
| IMEI-Change | 3141 | 8.4.22 | Unsigned32 | M,V |  |  |  | No |
| Monitoring-Event-Config-Status | 3142 | 8.4.24 | Grouped | M,V |  |  |  | No |
| Supported-Services | 3143 | 8.4.40 | Grouped | M,V |  |  |  | No |
| Supported-Monitoring-Events | 3144 | 8.4.41 | Unsigned64 | M,V |  |  |  | No |
| CIR-Flags | 3145 | 8.4.39 | Unsigned32 | M,V |  |  |  | No |
| Service-Result | 3146 | 8.4.37 | Grouped | M,V |  |  |  | No |
| Service-Result-Code | 3147 | 8.4.38 | Unsigned32 | M,V |  |  |  | No |
| Reference-ID-Validity-Time | 3148 | 8.4.42 | Time | M,V |  |  |  | No |
| Event-Handling | 3149 | 8.4.43 | Unsigned32 | M,V |  |  |  | No |
| NIDD-Authorization-Request | 3150 | 8.4.44 | Grouped | M,V |  |  |  | No |
| NIDD-Authorization-Response | 3151 | 8.4.45 | Grouped | M,V |  |  |  | No |
| Service-Report | 3152 | 8.4.47 | Grouped | M,V |  |  |  | No |
| Node-Type | 3153 | 8.4.48 | Unsigned32 | M,V |  |  |  | No |
| S6t-HSS-Cause | 3154 | 8.4.50 | Unsigned32 | M,V |  |  |  | No |
| Enhanced-Coverage-Restriction | 3155 | 8.4.51 | Grouped | V |  |  | M | No |
| Enhanced-Coverage-Restriction-Data | 3156 | 8.4.52 | Grouped | V |  |  | M | No |
| Restricted-PLMN-List | 3157 | 8.4.53 | Grouped | V |  |  | M | No |
| Allowed-PLMN-List | 3158 | 8.4.54 | Grouped | V |  |  | M | No |
| Requested-Validity-Time | 3159 | 8.4.55 | Time | V |  |  | M | No |
| Granted-Validity-Time | 3160 | 8.4.56 | Time | V |  |  | M | No |
| NIDD-Authorization-Update | 3161 | 8.4.57 | Grouped | V |  |  | M | No |
| Loss-Of-Connectivity-Reason | 3162 | 8.4.58 | Unsigned32 | V |  |  | M | No |
| Group-Reporting-Guard-Timer | 3163 | 8.4.59 | Unsigned32 | V |  |  | M | No |
| CIA-Flags | 3164 | 8.4.60 | Unsigned32 | V |  |  | M | No |
| Group-Report | 3165 | 8.4.61 | Grouped | V |  |  | M | No |
| Group-Report-Item | 3166 | 8.4.62 | Grouped | V |  |  | M | No |
| RIR-Flags | 3167 | 8.4.63 | Unsigned32 | V |  |  | M | No |
| Type-Of-External-Identifier | 3168 | 8.4.64 | Unsigned32 | V |  |  | M | No |
| APN-Validity-Time | 3169 | 8.4.65 | Grouped | V |  |  | M | No |
| Suggested-Network-Configuration | 3170 | 8.4.66 | Grouped | V |  |  | M | No |
| Monitoring-Event-Report-Status | 3171 | 8.4.67 | Grouped | V |  |  | M | No |
| PLMN-ID-Requested | 3172 | 8.4.68 | Enumerated | V |  |  | M | No |
| AdditionalIdentifiers | 3173 | 8.4.69 | Grouped | V |  |  | M | No |
| NIR-Flags | 3174 | 8.4.70 | Unsigned32 | V |  |  | M | No |
| Reporting-Time-Stamp | 3175 | 8.4.71 | Time | V |  |  | M | No |
| NIA-Flags | 3176 | 8.4.72 | Unsigned32 | V |  |  | M | No |
| Group-User-Identifier | 3177 | 8.4.73 | Grouped | V |  |  | M | No |
| MTC-Provider-Info | 3178 | 8.4.74 | Grouped | V |  |  | M | No |
| MTC-Provider-ID | 3179 | 8.4.75 | UTF8String | V |  |  | M | No |
| PDN-Connectivity-Status-Configuration | 3180 | 8.4.76 | Grouped | V |  |  | M | No |
| PDN-Connectivity-Status-Report | 3181 | 8.4.77 | Grouped | V |  |  | M | No |
| PDN-Connectivity-Status-Type | 3182 | 8.4.78 | Unsigned32 | V |  |  | M | No |
| Traffic-Profile | 3183 | 8.4.79 | Unsigned32 | V |  |  | M | No |
| Updated-Network-Configuration | 3184 | 8.4.80 | Grouped | V |  |  | M | No |
| Battery-Indicator | 3185 | 8.4.81 | Unsigned32 | V |  |  | M | No |
| SCEF-Reference-ID-Ext | 3186 | 8.4.82 | Unsigned64 | V |  |  | M | No |
| SCEF-Reference-ID-for-Deletion-Ext | 3187 | 8.4.83 | Unsigned64 | V |  |  | M | No |
| Exclude-Identifiers | 3188 | 8.4.84 | Grouped | V |  |  | M | No |
| Include-Identifiers | 3189 | 8.4.85 | Grouped | V |  |  | M | No |
| NOTE 1: The AVP header bit denoted as "M" indicates whether support of the AVP is required. The AVP header bit denoted as "V" indicates whether the optional Vendor-ID field is present in the AVP header. For further details, see IETF RFC 6733 [23]. NOTE 2: If the M-bit is set for an AVP and the receiver does not understand the AVP, it shall return a rejection. If the M-bit is not set for an AVP, the receiver shall not return a rejection, whether or not it understands the AVP. If the receiver understands the AVP but the M-bit value does not match with the definition in this table, the receiver shall ignore the M-bit. |  |  |  |  |  |  |  |  |

The following table specifies the Diameter AVPs re-used by the S6t interface protocol from existing Diameter Applications, including a reference to their respective specifications and when needed, a short description of their use within S6t.

Any other AVPs from existing Diameter Applications, except for the AVPs from Diameter base protocol specified in IETF RFC 6733 [23], do not need to be supported. The AVPs from Diameter base protocol specified in IETF RFC 6733 [23] are not included in table 8.4.1-2, but they may be re-used for the S6t protocol.

Table 8.4.1-2: S6t re-used Diameter AVPs

| Attribute Name |  | Reference |  | Comments |  | M-bit |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| User-Identifier |  | 6.4.2 |  | see 8.4.36 |  |  |  |
| External-Identifier |  | 6.4.11 |  |  |  |  |  |
| MSISDN |  | 3GPP TS 29.329 [10] |  |  |  |  |  |
| User-Name |  | IETF RFC 6733 [23] |  | This AVP shall contain the IMSI of the UE |  |  |  |
| Supported-Features |  | 3GPP TS 29.229 [7] |  | see 8.4.23 |  |  |  |
| Feature-List-ID |  | 3GPP TS 29.229 [7] |  |  |  |  |  |
| Feature-List |  | 3GPP TS 29.229 [7] |  |  |  |  |  |
| OC-Supported-Features |  | IETF RFC 7683 [15] |  | See 6.4.16 |  | Must not set |  |
| OC-OLR |  | IETF RFC 7683 [15] |  | See 6.4.17 |  | Must not set |  |
| Visited PLMN Id |  | 3GPP TS 29.272 [14] |  | This Information Element shall contain the identity (MCC and MNC) of the serving PLMN, either the VPLMN or the HPLMN. |  |  |  |
| Charged-Party |  | 3GPP TS 32.299 [16] |  |  |  |  |  |
| EPS-Location-Information |  | 3GPP TS 29.272 [14] |  | see 8.4.21 |  |  |  |
| MME-Location-Information |  | 3GPP TS 29.272 [14] |  | see 8.4.34 |  |  |  |
| SGSN-Location-Information |  | 3GPP TS 29.272 [14] |  | see 8.4.35 |  |  |  |
| E-UTRAN-Cell-Global-Identity |  | 3GPP TS 29.272 [14] |  |  |  |  |  |
| Tracking-Area-Identity |  | 3GPP TS 29.272 [14] |  |  |  |  |  |
| Current-Location-Retrieved |  | 3GPP TS 29.272 [14] |  |  |  |  |  |
| Age-Of-Location-Information |  | 3GPP TS 29.272 [14] |  |  |  |  |  |
| User-CSG-Information |  | 3GPP TS 29.272 [14] |  |  |  |  |  |
| Cell-Global-Identity |  | 3GPP TS 29.272 [14] |  |  |  |  |  |
| Service-Area-Identity |  | 3GPP TS 29.272 [14] |  |  |  |  |  |
| Routing-Area-Identity |  | 3GPP TS 29.272 [14] |  |  |  |  |  |
| eNodeB-ID |  | 3GPP TS 29.217 [17] |  |  |  |  |  |
| Day-Of-Week-Mask |  | IETF RFC 5777 [18] |  |  |  |  |  |
| Time-Of-Day-Start |  | IETF RFC 5777 [18] |  |  |  |  |  |
| Time-Of-Day-End |  | IETF RFC 5777 [18] |  |  |  |  |  |
| DRMP |  | IETF RFC 7944 [20] |  | see 8.4.46 |  | Must not set |  |
| Service-Selection |  | IETF RFC 5778 [21] |  | See 8.4.49 |  |  |  |
| Load |  | IETF RFC 8583 [22] |  | See 6.4.20 |  | Must not set |  |
| DL-Buffering-Suggested-Packet-Count |  | 3GPP TS 29.272 [14] |  |  |  |  |  |
| Extended-eNodeB-ID |  | 3GPP TS 29.217 [17] |  |  |  | Must not set |  |
| Maximum-UE-Availability-Time |  | 3GPP TS 29.338 [12] |  |  |  |  |  |
| Idle-Status-Indication |  | 3GPP TS 29.128 [24] |  |  |  | Must not set |  |
| Active-Time |  | 3GPP TS 29.128 [24] |  | When used over S6t, this AVP contains the value of Maximum Response Time (see 3GPP TS 29.122 [26], clause 5.13.2.1.2) which is used to calculate the value of subscribed Active Time as described in 3GPP TS 23.682 [2], clause 4.5.21. |  | Must not set |  |
| DL-Buffering-Suggested-Packet-Count |  | 3GPP TS 29.272 [14] |  |  |  | Must not set |  |
| Subscribed-Periodic-RAU-TAU-Timer |  | 3GPP TS 29.272 [14] |  | When used over S6t, this AVP contains the value of Maximum Latency (see 3GPP TS 29.122 [26], clause 5.13.2.1.2) which is used to calculate the value of subscribed periodic RAU/TAU timer as described in 3GPP TS 23.682 [2], clause 4.5.21. |  | Must not set |  |
| IMSI-Group-Id |  | 3GPP TS 29.272 [14] |  |  |  | Must not set |  |
| Number-of-UEs |  | 3GPP TS 29.154 [25] |  |  |  | Must not set |  |
| Terminal-Information |  | 3GPP TS 29.272 [14] |  |  |  | Must not set |  |
| PDN-Type |  | 3GPP TS 29.272 [14] |  |  |  | Must not set |  |
| Non-IP-PDN-Type-Indicator |  | 3GPP TS 29.272 [14] |  |  |  | Must not set |  |
| Non-IP-Data-Delivery-Mechanism |  | 3GPP TS 29.272 [14] |  |  |  | Must not set |  |
| Served-Party-IP-Address |  | 3GPP TS 32.299 [16] |  |  |  | Must not set |  |
| Reachability-Cause |  | 3GPP TS 29.128 [24] |  |  |  | Must not set |  |
| MPS-Messaging-Indication |  | 3GPP TS 29.272 [14] |  | For use on the S6t interface. |  | Must not set |  |

# ***** Third change *****

### 8.4.66	Suggested-Network-Configuration

The Suggested-Network-Configuration AVP is of type Grouped, and it shall contain the details for configuration for Suggested Network configuration.

AVP format:

Suggested-Network-Configuration::=	<AVP header: 3170 10415>

{ SCEF-Reference-ID }

[ SCEF-Reference-ID-Ext ]

{ SCEF-ID }

[ Subscribed-Periodic-RAU-TAU-Timer ]

[ Active-Time ]

[ DL-Buffering-Suggested-Packet-Count ]

[ MTC-Provider-Info ]

*[ SCEF-Reference-ID-for-Deletion ]

*[ SCEF-Reference-ID-for-Deletion-Ext ]

[ MPS-Messaging-Indication ]

*[AVP]

If the values of SCEF-Reference-ID and SCEF-Reference-ID-for-Deletion are the same, no Network Parameter Configurations AVP(s) shall be present, that is, SCEF-Reference-ID-for-Deletion takes precedence. If the values of SCEF-Reference-ID and SCEF-Reference-ID-for-Deletion are not the same, Network Parameter Configurations AVP(s) are related to SCEF-Reference-ID.

When the "Extended Reference IDs" feature is supported by the HSS and SCEF, the SCEF-Reference-ID-Ext and SCEF-Reference-ID-for-Deletion-Ext AVPs shall be used insted of SCEF-Reference-ID and SCEF-Reference-ID-for-Deletion respectively; in such case, the required AVP "SCEF-Reference-ID" shall be included in the grouped AVP by the sender, but its content shall be discarded by the receiver.

# ***** Fourth change *****

### 8.4.x	MPS-Messaging-Indication

The MPS-Messaging-Indication AVP is of type Enumerated and shall indicate the MPS for Messaging status being  requested. The following values are defined:

CLEAR (0)

The value CLEAR (0) indicates that the request is to have the MPS for Messaging indication for the UE to be clear (disabled).

SET (1)

The value SET (1) indicates that the request is to have the MPS for Messaging indication for the UE to be set (enabled).

The default value, when this AVP is not included, is CLEAR (0).

# ***** End of changes *****