3GPP TSG-CT WG4 Meeting #126	C4-245420

Orlando, U.S; 18th – 22nd November 2024	was C4-245132

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.527 | CR | 0089 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PFCP sessions excluded from the restoration upon a SMF failure with SMF set being deployed |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2024-11-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | When SMF Set feature (i.e. either SSET or MAPS feature as specified in clauses 5.22.2 and 5.22.3) is deployed in the network, upon detecting the failure of the SMF, the UPF shall restore the affected PFCP session towards an alternative SMF pertaining to the same SMF Set. See below relevant requirements: In clause 4.4.3.2 of 3GPP TS 23.527: 4.4.3 Restoration Procedure for SMF Failure without Restart 4.4.3.1 General When a UPF detects that a peer PFCP entity in the SMF is not reachable for a preconfigured time, and the MPAS or SSET procedure specified for SMF set in clause 5.22 of 3GPP TS 29.244 [4]) are not used, the UPF shall delete all the session contexts affected by the peer PFCP entity failure that it may have stored. 4.4.3.2 Failure of an SMF in a SMF set When the MPAS or SSET procedure specified for SMF set in clause 5.22 of 3GPP TS 29.244 [4]) is used, when the UPF detects that a peer PFCP entity in an SMF is not responsive for a preconfigured time, or upon being instructed by SMF(s) of the SMF set to move PFCP sessions, the UPF shall move the PFCP sessions that were served by the failed PFCP entity to another PFCP entity in the same SMF or another SMF, as specified in clause 5.22 of 3GPP TS 29.244 [4] and clause 4.7.3. In 3GPP TS 29.244, it reads in clause 5.22.2 and 5.22.3: 7) The UPF shall trigger the SMF restoration procedures for an SMF in a SMF set specified in 3GPP TS 23.527 [40] for a PFCP session that can be controlled by different SMFs of an SMF set when a heartbeat failure is detected for the IP address of the assigned CP F-SEID. … 7) The UPF shall trigger the SMF restoration procedures for an SMF in a SMF set specified in 3GPP TS 23.527 [40] for a PFCP session that can be controlled by different SMFs of an SMF set when a heartbeat failure is detected. However, NOT all PFCP sessions are subject for such restoration, for example: The PFCP sessions established for transporting packets containing the signaling message between the SMF and the radius server, between the SMF and the DHCP server, as well as ICMP message to detect the liveness of the P-CSCF, and so on; these PFCP sessions are SMF specific and not associated with any PDU session, each SMF will establish these PFCP sessions on its own, therefore there is no point to restore those PFCP sessions established by SMF1 towards the SMF2 since SMF2 has already established those PFCP sessions between the SMF2 and the UPF for the same purposes. (see also clause 5.3.1 as below) " For EPC PFCP session contexts may correspond to individual PDN connections, TDF sessions, or standalone sessions not tied to any PDN connection or TDF session used e.g. for forwarding RADIUS, Diameter or DHCP signalling between the PGW-C and the PDN or for forwarding End Marker packets from the PGW-C or SGW-C to a downstream SGW-U or eNodeB. For 5GC PFCP session contexts may correspond to individual PDU sessions or standalone sessions not tied to any PDU sessions used e.g. for forwarding RADIUS, Diameter or DHCP signalling between the SMF and the DN or for forwarding End Marker packets from the SMF to a downstream UPF or NG-RAN." It is proposed to add an indication to the UPF to indicate whether the PFCP session should be excluded from the restoration procedure when detecting the SMF failure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce an indication in the PFCP Session Establishment Request message to indicate the UPF that the PFCP session shall be excluded from the restoration of the PFCP sessions affected by the SMF failure, i.e. the PFCP session shall be deleted. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Hanging PFCP sessions in the UPF, possible extra signaling for restoring those PFCP sessions which are not subject to be restored at an alternative SMF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: add a clarification for excluding the PFCP sessions from the restoration before the figure. |  |  |  |  |  |  |  |  |

# *** First Change ***

#### 4.4.3.2	Failure of an SMF in a SMF set

When the MPAS or SSET procedure specified for SMF set in clause 5.22 of 3GPP TS 29.244 [4]) is used, when the UPF detects that a peer PFCP entity in an SMF is not responsive for a preconfigured time, or upon being instructed by SMF(s) of the SMF set to move PFCP sessions, the UPF shall move the PFCP sessions (other than those PFCP sessions excluded from the restoration) that were served by the failed PFCP entity to another PFCP entity in the same SMF or another SMF, as specified in clause 5.22 of 3GPP TS 29.244 [4] and clause 4.7.3.

Figure 4.4.3.2-1 depicts an example call flow for a failure (without restart) of an SMF in an SMF set using the MPAS or SSET feature.

Figure 4.4.3.2-1: Failure (without restart) of an SMF in an SMF set using the MPAS or SSET feature

0.	When using the MPAS feature, the SMFs of the SMF set establish PFCP associations with the UPF, including the SMF Set ID IE set to an FQDN representing the SMF Set and optionally Alternative SMF IP Address IE(s) of alternative PFCP entities in the same SMF or different SMFs of the SMF set.

When using the SSET feature, one SMF of the SMF set establishes one PFCP association with the UPF for the SMF set, optionally including Alternative SMF IP Address IE(s) of alternative PFCP entities in the same SMF or different SMFs of the SMF set. In this case, the UPF considers that the SMF1, SMF2 and SMF3 represent different PFCP entities.

The SMFs of the SMF set establish PFCP sessions in the UPF, optionally providing a FQ-CSID and/or a Group ID as described in clause 4.7.2. The SMF may also indicate to the UPF if the PFCP session shall be excluded from the restoration procedure as specified below, i.e. the PFCP session shall be deleted locally, e.g. for a PFCP session established for transporting Radius signaling between the SMF and the Radius server via the UPF.

1.	The SMF1 fails without restart.

2.	The SMFs of the SMF set may request the UPF to move group(s) of PFCP sessions, each identified by a FQ-CSID, Group ID or CP IP Address, to alternative SMF(s) of the SMF set.

3.	Upon being instructed by the SMFs to move PFCP sessions, or upon detecting that the SMF1 is not responsive, the UPF sends subsequent PFCP Session Report Request messages to the alternative SMFs of the SMF set, as specified in clause 5.22 of 3GPP TS 29.244 [4] and clause 4.7.3. The UPF sets the SEID field to zero in the PFCP header of the PFCP Session Report Request and includes the old CP F-SEID that was assigned by the previous SMF in the request.

Upon receiving such requests, the SMF takes over the control of the PFCP sessions from the previous SMF and sends PFCP Session Report Response messages including, for each PFCP session, the CP F-SEID IE with the IPv4 or IPv6 address of the new PFCP entity and the same or a modified SEID, and optionally including the N4-u F-TEID that the UPF shall use for sending data towards the new entity.

Alternatively (not depicted in the figure), the SMF may redirect a PFCP Session Report Request to a different SMF of the same SMF set, either by rejecting the request with the cause "Redirection Requested" and with the IP address of the new entity to contact, or by forwarding the request to another PFCP entity pertaining to the same SMF or another SMF in the SMF set. In the former case, the UPF resends a PFCP Session Report Request to the new PFCP entity to contact. In the latter case, the new PFCP entity sends a PFCP Session Report Response including the CP F-SEID IE with the IPv4 or IPv6 address of the new PFCP entity and the same or a modified SEID, and optionally including the N4-u F-TEID that the UPF shall use for sending data towards the new entity.

When a UPF detects that none of the SMF of the SMF set is responsive for a preconfigured time, the UPF shall delete all the session contexts established by any SMF of the SMF set that it may have stored.

# *** End of Changes ***