3GPP TSG-CT WG4 Meeting #126	C4-245532

# Orlando, U.S; 18th – 22nd November 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.562 | CR | 0157 | rev | 2 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 29.562 MPS for IMS Messaging Indication on Nhss |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Perspecta Labs, CISA ECD, AT&T, T-Mobile USA, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | MPS4msg |  |  |  |  |  | Date: |  |  | 2024-11-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | An indication is required to inform IMS functional entities whether or not MPS is active for text messaging. The indication indicates whether MPS for Messaging is active (invoked) or inactive (revoked). The Cx interface needs to be able to convey the indication. |  |  |  |  |  |  |  |  |
| Summary of change: |  | First change: Adds MessagingPriority to PriorityLevels. This indicates whether MPS for messaging is enabled or disabled. Second change: Adds explanation that PriorityLevels also contains the MessagingPriority indication. Third change: Adds MessagingPriority to the OpenAPI. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MPS users will not have priority for IMS messaging. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.6.2.13, 6.2.6.2.33, A.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backwards-compatible new features, with impacts on the following APIs: - TS29562_Nhss_imsSDM.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# ***** First change *****

##### 6.2.6.2.13	Type: PriorityLevels

Table 6.2.6.2.13-1: Definition of type PriorityLevels

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| servicePriorityLevelList | array(NameSpacePriority) | M | 1..N | List of namespaces and their associated priorities allowed for the user. |
| servicePriorityLevel | integer | O | 0..1 | Possible values: 0 (Highest priority) 1 2 3 4 (Lowest priority) |
| messagingPriority | boolean | O | 0..1 | Priority for MPS for messaging true: enabled false or absent: not enabled May be present only if Service Priority Level include "WPS" namespace |

# ***** Second change *****

##### 6.2.6.2.33	Type: PublicIdentifier

Table 6.2.6.2.33-1: Definition of type PublicIdentifier

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| publicIdentity | PublicIdentity | M | 1 | Public identity information |
| displayName | string | O | 0..1 | Name associated with the public identity |
| imsServicePriority | PriorityLevels | O | 0..1 | Multimedia service priority. Includes MPS for messaging priority indication. |
| serviceLevelTraceInfo | ServiceLevelTraceInformation | O | 0..1 | Service Level Tracing Information |
| barringIndicator | boolean | O | 0..1 | Indicates whether the identity is barred, i.e. it can not be used in any IMS communication except registrations, re-registrations and de-registrations. true: identitiy is barred false or absent: identity is not barred |
| wildcardedImpu | string | O | 0..1 | When identityType is wildcarded IMPU it contains the wildcard id as stored by HSS |

# ***** Third change *****

# A.3	Nhss_imsSDM API

(… text not shown for clarity …)

PriorityLevels:

description: Namespaces and priority levels allowed for the IMS public Identity

type: object

required:

- servicePriorityLevelList

properties:

servicePriorityLevelList:

type: array

items:

$ref: '#/components/schemas/NameSpacePriority'

minItems: 1

uniqueItems: true

servicePriorityLevel:

type: integer

minimum: 0

maximum: 4

messagingPriority:

type: boolean

default: false

(… text not shown for clarity …)

# ***** End of changes ***