3GPP TSG-CT6 Meeting #120-bis	C6-240684

# Orlando, United States, 19th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 31.127 | CR | 0008 | rev | 2 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | X | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TS 31.127 Correction to Test Env in section 4 and Annex A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Technologies Ireland, Apple AB Denmark, Comprion GmbH |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT6 |  |  |  |  |  |  |  |  |  |
| Work item code: | UEConTest_R18 |  |  |  |  |  | Date: |  |  | 2024-11-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Missing information on Test EFs required for the Test Applets used in the present specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add Annex A to include examples of Test EFs in the Test nrUSIM configuration and include refernce to Annex A in the test envirenment 4.1.5. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Information on Test EFs required for the Test Applets used in the present specification will not be available. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.4, 4.1.5, Annex A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Was C6-240674, was C6-240672 |  |  |  |  |  |  |  |  |

[…]

# 4	Test environment

## 4.1	Test environment description

…

***** first change *****

### 4.1.4	Example - test environment for seamless testing

Seamless testing within the scope of the present document relies on monitoring of data transfer between ME and nrUSIM traced by a software interface between nrUSIM and baseband implemented by the ME vendor.

The logged communication can be transferred to the TT and will be used to determine if conformance requirements are met.

NOTE:	A test toolkit applet may be used for testing USAT specific test cases in TS 31.117 [35]. Refer to Annex A.1 for examples of test EFs required when using the test toolkit applet.

Figure 4.3: Test environment for seamless testing

***** next change *****

### 4.1.5	Example – test environment for test toolkit events based testing

Test toolkit events based testing within the scope of the present document is applicable for UEs supporting the required USIM application toolkit functionality. A toolkit applet is installed onto the nrUSIM, capable of handling test events internal to the card runtime environment and the applets to monitor APDUs received at the nrUICC.

NOTE:	The same test toolkit applet may be used for testing USAT specific test cases in TS 31.117 [35]. Refer to Annex A.2 for examples of test EFs required when using the test toolkit applet.

The logged events can be stored in a test EF during the test execution phase and transferred to the TT during the verification phase to determine if conformance requirements are met.

ME may provide an interface for tester to monitor APDU communication during test execution through the TT.

Figure 4.4: Test environment for test toolkit events based testing

Please see the required "Test configuration state" in Annex N and the "Examples of test configuration usage" in Annex O of ETSI TS 102 221 [8]

[…]

***** next change *****

Annex A (informative):
Examples of Test EFs in the Test-nrUICC configuration

The details provided for the Test-SIM (Default UICC) in TS 31.121 [2] clause 4 apply to the Test-nrUICC configuration used with this test specification, with the following addition.

Test Tool or device manufacturer may provide the Test EF structure on the GSMA TS.48 [9] configuration depending on the test applet used for the testing.

## A.1	Test EF structure - 1

DFTEST shall be present at the Master File (MF) level to be used for testing purpose.

The general structure of DFTEST and associated EF is defined below.

This EF structure is suggested to be used with the test environment defined in clause 4.1.4 of the present document in case USAT functionality is required to operate the test case.

|  |  |  |  |  |  |  |  |  |  |  | MF |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  | '3F00' |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | EFDIR |  |  | EFPL |  |  | EFARR |  |  | EFICCID |  |  | EFUMPC |  |  |
|  |  |  |  |  |  |  |  | '2F00' |  |  | '2F05' |  |  | '2F06' |  |  | '2FE2' |  |  | '2F08' |  |  |
|  |  | DFTEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | '7F0A' |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | EFSTK |  |  | EFSETSTK |  |  | EFCC |  |  | EFEVENTLIST |  |  |  |  |  |
|  |  |  |  |  |  |  |  | '2F0A' |  |  | '2F0B' |  |  | '2F0C' |  |  | '2F0D' |  |  |  |  |  |

The information in EFSTK, EFSETSTK, EFCC, EFEVENTLIST shall be used. A Generic STK Applet that utilizes the test data configured by tester in these new EFs to trigger the required commands from the UICC shall be used.

NOTE:	File ID for DFTEST is implementation specific.

## A.1.1	EFSTK (SIMToolkit data)

| Identifier: '2F0A' |  | Structure: linear fixed |  |  | Optional |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Record length: X bytes (see note) |  |  |  | Update activity: low |  |  |  |
| Access Conditions: READ ALW UPDATE ALW DEACTIVATE ALW ACTIVATE ALW |  |  |  |  |  |  |  |
| Bytes | Description |  | Default Value |  |  | M/O | Length |
| 1 to X | Test Data - Record 1 |  | 'FF FF .. FF FF' |  |  | M | X bytes |
| .. | .. |  | .. |  |  | M | X bytes |
| 39*X+1 to 40*X | Test Data - Record 40 |  | 'FF FF .. FF FF' |  |  | M | Xbytes |

Each individual record can hold the tester configured data, specific to the proactive command that needs to be issued by the ME referenced by EFSETSTK, for structure and coding details of the proactive command refer TS 31.111 [27], ETSI TS 102 223 [28].

NOTE:	The record length is implementation specific. It is recommended to configure a record length allowing to entirely store each of the toolkit commands required for testing.

## A.1.2	EFSETSTK (Set SIM Toolkit)

| Identifier: '2F0B' |  | Structure: transparent |  |  | Optional |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| File size: 1 byte |  |  |  | Update activity: low |  |  |  |
| Access Conditions: READ ALW UPDATE ALW DEACTIVATE ALW ACTIVATE ALW |  |  |  |  |  |  |  |
| Bytes | Description |  | Default Value |  |  | M/O | Length |
| 1 | Test Data |  | '00' |  |  | M | 1 byte |

Test Data is either set by the tester to the default value '0x00' or a specific record number of EFSTK. When read whilst set to a specific record number of EFSTK, it will imply to trigger the respective SIM toolkit command.

Example:

EFSTK is configured as '0x01':

the test toolkit applet will issue the command stored at record 01 of EFSTK.

Once the command is issued successfully, the applet will set EFSETSTK to the default value.

EFSTK is configured as '0x80':

the test toolkit applet will issue all available commands stored in records of EFSTK in sequence.

NOTE:	This is helpful for tests where multiple proactive commands are required to be issued in sequence (from record 1 to n of EFSTK) after receiving the Terminal Response from the prior command.

## A.1.3	EFCC (CALL CONTROL)

| Identifier: '2F0C' |  | Structure: transparent |  |  | Optional |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Record length: 255 bytes |  |  |  | Update activity: low |  |  |  |
| Access Conditions: READ ALW UPDATE ALW DEACTIVATE ALW ACTIVATE ALW |  |  |  |  |  |  |  |
| Bytes | Description |  | Default Value |  |  | M/O | Length |
| 1 to 255 | Test Data |  | '00 .. 00' |  |  | M | 255 bytes |

Test Data is configured by the tester with the Call control response (as defined in ETSI TS 102 223 [28], TS 31.111 [27]) to process the Call Control Envelope commands received from the ME based on the Call Control service configured in UST.

For example:

- '00 00' implies allowed not modified.

- '01 00' implies not allowed.

- '02 LL…XX' implies allowed with modifications. (LL denotes the length of the TLV).

## A.1.4	EFEVENTLIST (EVENT LIST)

| Identifier: '2F0D' |  | Structure: transparent |  |  | Optional |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Record length: 255 bytes |  |  |  | Update activity: low |  |  |  |
| Access Conditions: READ ALW UPDATE ALW DEACTIVATE ALW ACTIVATE ALW |  |  |  |  |  |  |  |
| Bytes | Description |  | Default Value |  |  | M/O | Length |
| 1 to 255 | Test Data |  | 'FF .. FF' |  |  | M | 255 bytes |

The Test data is of format <length><event1><event2>..<event n>.. FF FF, where the length indicates the number of configured events as configured by the tester. Supported events are defined in ETSI TS 102 223 [28], TS 31.111 [27].

The data persist even after the card reset. Hence this data needs to be cleared by the tester by reverting the contents to default values (FF..FF)

## A.2	Test EF structure - 2

EFTC_IN, EFTC_OUT, and EFEF_RESET files may be used for testing purpose and if used, shall be present at the Master File (MF) level. A Generic STK Test Applet can make use of the data configured in those files by the TT.

The general structure for the EFs is defined as below.

|  |  |  |  |  |  | MF |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | '3F00' |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | EFDIR |  |  | EFPL |  |  | EFARR |  |  | EFICCID |  |  |  | EFUMPC |  |
|  |  |  | '2F00' |  |  | '2F05' |  |  | '2F06' |  |  | '2FE2' |  |  |  | '2F08' |  |
|  |  |  | EFEF_RESET |  |  | EFTC_IN |  |  | EFTC_OUT |  |
|  |  |  | '2FXX' |  |  | '2FYY' |  |  | '2FZZ' |  |

NOTE:	File IDs for EFTC_IN, EFTC_OUT, and EFEF_RESET files are implementation specific.

This EF structure is suggested to be used with the test environment defined in clause 4.1.5 of the present document in case USAT functionality is required to operate the test case.

EFTC_IN and EFTC_OUT files are used during test case execution, and it shall be updated fresh for each test case. EFEF_RESET file is used for saving EF content required to restore the TS.48 configuration to its original content before initializing test specific data.

## A.2.1	EFTC_IN (Test Case Input)

This file contains data required for a test applet to handle test case execution. Content and format of the data may be changed depending on the Applet implementation.

| Identifier: '2FXX' |  | Structure: transparent |  |  | Optional |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Record length: 255 bytes |  |  |  | Update activity: low |  |  |  |
| Access Conditions: READ ALW UPDATE ALW DEACTIVATE ALW ACTIVATE ALW |  |  |  |  |  |  |  |
| Bytes | Description |  | Default Value |  |  | M/O | Length |
| 1 to 255 | Test Data |  | 'FF .. FF' |  |  | M | 255 bytes |

The EF may include data related to:

- Test step control

  - TT can update this data to direct test applet to move to next step in the test case.

- Toolkit command data

  - This can be a sequence of TLVs related to proactive commands or event registrations or call control response data or any other action required for the test case.

  - Different tags can be defined for this purpose.

Example:

| Bytes | Description | M/O | Length |
| --- | --- | --- | --- |
| 1 | Test step control | M | 1 byte |
| 2 | Tag 1: Wait for new step tag | M | 1 byte |
| 3 | Tag 1 length | M | 1 byte |
| 4 to 4+X-1 | Tag 1 data | M | X bytes |
| 4+X | Tag 2: Proactive command tag | O | 1 byte |
| 5+X | Tag 2 length | O | 1 byte |
| 6+X to 6+X+Y-1 | Tag 2 data | O | Y bytes |
| NOTE: X and Y is greater than zero if the Tag is present. |  |  |  |

## A.2.2	EFTC_OUT (Test Case Output)

This file contains data received from the ME and it can be read by the TT to verify if meeting the acceptance criteria of a test case. Additionally, few leading bytes can be used for tracking EFTC_IN and EFTC_OUT current offsets if required by the applet. Content and format of the data may be changed depending on the Applet implementation.

| Identifier: '2FYY' |  | Structure: transparent |  |  | Optional |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Record length: 255 bytes |  |  |  | Update activity: low |  |  |  |
| Access Conditions: READ ALW UPDATE ALW DEACTIVATE ALW ACTIVATE ALW |  |  |  |  |  |  |  |
| Bytes | Description |  | Default Value |  |  | M/O | Length |
| 1 to 255 | Test Data |  | 'FF .. FF' |  |  | M | 255 bytes |

The EF can include data related to,

- Test EFs offset control,

  - TT can update initial offsets during test case initialization.

  - TT can track and update the test EF offsets as applet executes test steps.

- Toolkit command response data received from the ME,

  - This can be a sequence of TLVs related to proactive command responses (TERMINAL RESPONSE) or envelope commands (EVENTs).

  - Different Tags can be used for this purpose.

Example:

| Bytes | Description | M/O | Length |
| --- | --- | --- | --- |
| 1-2 | Test EFs offset control bytes | M | 2 bytes |
| 3-4 | Length | M | 2 bytes |
| 5 | Tag 1: Proactive command 1 tag (for TERMINAL RESPONSE 1) | O | 1 byte |
| 6 | Tag 1 length | O | 1 byte |
| 7 to 7+X-1 | Tag 1 data | O | X bytes |
| 7+X | Tag 2: Proactive command 2 tag (for TERMINAL RESPONSE 2) | O | 1 byte |
| 8+X | Tag 2 length | O | 1 byte |
| 9+X to 9+X+Y-1 | Tag 2 data | O | Y bytes |
| NOTE: X and Y is greater than zero if the Tag is present. |  |  |  |

## A.2.3	EFEF_RESET (EF_RESET)

This file contains data required for updating EFs in the TS.48 configuration in order to restore the original content of it as part of test case initialization. TT can request the applet through test control byte to read the content from this EF and update required EFs in the TS.48 configuration. Content and format of the data may be changed depending on the Applet implementation.

[…]

| Identifier: '2FZZ' |  | Structure: linear fixed |  |  |  | Optional |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Record length: 128 bytes |  |  | Update activity: low |  |  |  |  |
| File size: 128 * n, (n = 25) |  |  |  |  |  |  |  |
| Access Conditions: READ ALW UPDATE ALW DEACTIVATE ALW ACTIVATE ALW |  |  |  |  |  |  |  |
| Byte | Description |  |  | Default Value | M/O |  | Length |
| 1 to X(1) | EF content read control data |  |  | 'FF FF .. FF FF' | M |  | X bytes |
| X+1 to X+1+128-1 | Sequence of EF content data - 1 |  |  | 'FF FF .. FF FF' | M |  | 128 bytes |
| X+1+128 to X+1+128*2-1 | Sequence of EF content data - 2 |  |  | 'FF FF .. FF FF' | O |  | 128 bytes |
| .. | .. |  |  | .. | O |  | 128 bytes |
| X+1+128*23 to X+1+128*24-1 | Sequence of EF content data - 24 |  |  | 'FF FF .. FF FF' | O |  | 128 bytes |
| NOTE 1: X depends on applet implementation and is greater than zero. |  |  |  |  |  |  |  |

The EF can include data related to,

- EF content read control data,

  - TT can update this record with an instruction on what records to be read to update the EFs in the nrUICC.

- Sequence of EF content data,

  - This can be a sequence of TLVs related to EF content in the TS.48 configuration.

  - Each sequence can include data related to more than one EF in the TS.48 configuration.

  - Applet can choose a format to include number of padding pattern (eg ‘FF’) or unused bytes instead of updating with a sequence of same pattern to save the size of data in a record.

Example format:

| Record number | Data format |
| --- | --- |
| 1 | <instruction to read all none empty records>, <REFRESH flag> OR <read from start record>, <read end record>, <REFRESH flag> |
| 2 | <path length 1>,< EF id 1>,<0 (for Transparent EF)>,<length of data 1>,<data 1>,<no of repeated pattern>,<length of pattern>,<repeated pattern>, <path length 2>,< EF id 2>,<1, (for record# 1)>,<length of actual data 2.1>,<data 2.1>,<no of repeated pattern>, <length of pattern>,<repeated pattern>, <path length 2>,< EF id 2>,<2, (for record# 2)>,<length of actual data 2.2>,<data 2.2>,<no of repeated pattern>, <length of pattern>,<repeated pattern>, ….. |
| 3 | <path length 5>,< EF id 5>,<0 (Transparent EF)>,<length of data 5>,<data 5>,0, <path length 6>,< EF id 6>,<1 (Record# 1)>,<no. of FF>,<length of actual data 6.1>,<data 6.1>,0, <path length 7>,< EF id 7>,<0 (Transparent E)>,<no. of FF>,<length of actual data 7>,<data 7>,0, ….. |
| NOTE: <length of pattern> and <repeated pattern> is not required if <no of repeated pattern> is 0. |  |

Example data:

| Record number | Data format |
| --- | --- |
| 1 | FD,1 OR 2 ,3, 0 |
| 2 | '02 6F C9 01 04 01 02 03 04 00…..' |
| 3 | '02 6F CE 05 01 09 07 01 FF 02 6F D2 00 03 03 03 03 00 61 01 FF…..' |

***** end of changes *****