3GPP TSG-CT WG6 Meeting #119-bis	draft_C6-240461

# Maastricht, Netherlands, 20th Aug 2024 - 23rd Aug 2024

Title:	Availability of 3GPP CT6 Release 18 specifications defining associated elementary files.

Release:	Rel-18

Work Item:	TEI18

Source:	3GPP CT WG6

To:	GSMA TSG

Cc:	GSMA eSIM WG, TCA eUICC WG

Contact person:

Name: 	Stanley Matthew Mayalil

E-mail Address:	smayalil@apple.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:		None

# 1	Overall description

3GPP CT WG6 would like to inform GSMA TSG that 3GPP CT WG6 has concluded on defining 3GPP file structures required for Rel 18 implementations.

Please find in following tables summary of Rel 18 and previous Rel 17 added elements.

Rel 18:

| DF/EF | ADF/DF | Service no | Size (Bytes) | Structure | Specification/Comment |
| --- | --- | --- | --- | --- | --- |
| EFUST (USIM Service Table) | ADF-USIM | UST service from 147 to 150 added |  | - | 31.102/updated EF (Mandatory requirement - service 133 to be available, when 123 is available) |
| EFNASCONFIG (Non Access Stratum Configuration) | ADF-USIM |  | - Variable | Transparent | 31.102/updated EF - Additional NAS configuration parameters added. - Satellite Disabling Allowed for 5GMM cause #15 |
| EFOCST (Operator Controlled Signal threshold per Access Technology) | ADF-USIM | UST service:148 | >=1 | Transparent | 31.102/New EF |
| DF5MBSUECONFIG | ADF-USIM | UST service:147 |  |  | 31.102/New DF |
| EF5MBSUECONFIG (5MBS UE pre-configuration) | ADF-USIM/DF-5MBS | UST service:147 | Variable | Transparent | 31.102/New DF |
| EF5MBSUSD (5MBS User Service Description) | ADF-USIM/DF-5MBS | - | Variable | Transparent | 31.102/New EF |
| DFA2X (Aircraft-to-Everything) | DFTELECOM |  | - | - | 31.102/New DF |
| EFAST (A2X Service Table) | DFA2X | UST service: 149 | >=1 | Transparent | 31.102/New EF |
| EFA2X_CONFIG (A2X configuration data) | DFA2X | AST service: 1 | >=4 | Transparent | 31.102/New EF |
| EFA2XP_PC5 (A2X policy data over PC5) | DFA2X | AST service: 2 | >=11 | Transparent | 31.102/New EF |
| EFA2X_DDAAP_PC5(A2X Direct Detect And Avoid policy data over PC5) | DFA2X | AST service: 3 | >=4 | Transparent | 31.102/New EF |
| EFA2X_DC2P_PC5(A2X Direct C2 communication policy data over PC5) | DFA2X | AST service: 4 | >=6 | Transparent | 31.102/New EF |
| EFA2XP_Uu (A2X policy data over Uu) | DFA2X | AST service: 5 | >=12 | Transparent | 31.102/New EF |
| EFIMSDCI (IMS Data Channel Indication) | Either ADF-USIM or ADF-ISIM | UST service: 150 or IST service: 22 | = 1 | Transparent | 31.102 / Optional New EF 31.103 / New EF Note: refer 31.102 regarding the presence of this file in ADF-USIM. |
| EFAC_GBAUAPI (Access Control to GBA_U_API) | Either ADF-USIM or ADF-ISIM | UST service: 68 or IST service: 2 | >=7 | Linear Fixed | 31.102 / Optional New EF 31.103 / New EF Note: refer 31.102 regarding the presence of this file in ADF-USIM. |
| EFIST (ISIM Service Table) | ADF-ISIM | IST service 22 added | - | - | 31.102/updated EF |
| EF5G_PROSE_U2URU (5G ProSe configuration data for UE-to-UE relay UE) | DF5G_ProSe | UST service: 139 and 5G Prose service: 6 | >=46 | Transparent | 31.102/New EF |
| EF5G_PROSE_EU | DF5G_ProSe | UST service: 139 and 5G Prose service: 7 | >=46 | Transparent | 31.102/New EF |
| EF5G_PROSE_U2NRU (5G ProSe configuration data for UE-to-network relay UE) | DF5G_ProSe | UST service: 139, 5G Prose service: 3 | >=32 | Transparent | UPDATE content only: additional new TAG added in Rel 18. |
| EF5G_PROSE_RU (5G ProSe configuration data for remote UE) | DF5G_ProSe | UST service: 139, 5G Prose service: 4 | >=29 | Transparent | UPDATE content only: additional new TAG added in Rel 18. |
| EFARR | ADF-SSIM |  | Variable | Linear Fixed | 31.105/New EF |
| EFEAPID | ADF-SSIM |  | Variable | Transparent | 31.105/New EF |
| EFNSSAI | ADF-SSIM |  | = 4 | Linear Fixed | 31.105/New EF |
| EFEAPSTATUS | ADF-SSIM |  | = 5 | Linear Fixed | 31.105/New EF |

NOTE: Since 3GPP TS 31.102 version 17.2.0, If Service 136 'Support for multiple records of NAS security context storage for multiple registration' is available in UST, then the EF 5GS3GPPNSC shall have two records instead of one.

Rel 17:

| EF | ADF/DF | Service no | Size (Bytes) | Structure | Specification/Comment |
| --- | --- | --- | --- | --- | --- |
| EFUST (USIM Service Table) | ADF-USIM | UST: service from 136 to 146 added |  | - | 31.102/updated EF |
| EFeAKA (enhanced AKA support) | ADF-USIM | - |  | Transparent | 31.102/New EF |
| DF5G ProSe | ADF-USIM | UST service: 139 | - | - | 31.102/New DF |
| EF5G_PROSE_ST (5G ProSe Service Table) | DF5G_ProSe | UST service: 139 | >=1 | Transparent | 31.102/New EF |
| EF5G_PROSE_DD (5G ProSe configuration data for direct discovery) | DF5G_ProSe | UST service : 139, 5G Prose service: 1 | >=26 | Transparent | 31.102/New EF |
| EF5G_PROSE_DC (5G ProSe configuration data for direct communication) | DF5G_ProSe | UST service: 139, 5G Prose service: 2 | >=12 | Transparent | 31.102/New EF |
| EF5G_PROSE_U2NRU (5G ProSe configuration data for UE-to-network relay UE) | DF5G_ProSe | UST service: 139, 5G Prose service: 3 | >=32 | Transparent | 31.102/New EF |
| EF5G_PROSE_RU (5G ProSe configuration data for remote UE) | DF5G_ProSe | UST service: 139, 5G Prose service: 4 | >=29 | Transparent | 31.102/New EF |
| EF5G_PROSE_UIR (5G ProSe configuration data for usage information reporting) | DF5G_ProSe | UST service: 139, 5G Prose service: 5 | >=32 | Transparent | 31.102/New EF |
| DFSNPN | ADF-USIM | UST service 143 or 146 |  |  | 31.102/New DF |
| EFPWS_SNPN (Public Warning System in SNPNs) | DFSNPN | UST service: 143 | =1 | Transparent | 31.102/New EF |
| EFNID (Network Identifier for SNPN) | DFSNPN | UST service: 146 | =6 | Linear Fixed | 31.102/New EF |
| EFCAG | DF5GS | UST service: 137 | >=2 | Transparent | 31.102/New EF |
| EFSOR-CMCI | DF5GS | UST service: 138 | >=7 | Transparent | 31.102/New EF |
| EFDRI | DF5GS | UST service: 140 | >=7 | Transparent | 31.102/New EF |
| EF5GSEDRX | DF5GS | UST service: 141 | >=2 | Transparent | 31.102/New EF |
| EF5GNSWO_CON | DF5GS | UST service: 142 | >=1 | Transparent | 31.102/New EF |
| EFMCHPPLMN | DF5GS | UST service: 144 | >=1 | Transparent | 31.102/New EF |
| EFKAUSF_DERIV ATION | DF5GS | UST service: 145 | >=1 | Transparent | 31.102/New EF |

References: 3GPP TS 31.102, 3GPP TS 31.103, 3GPP TS 31.105

3GPP CT WG6 looks forward to a fruitful collaboration with GSMA TSG group.

# 2	Actions

To: TCA eUICC WG

ACTION: 	3GPP CT WG6 kindly asks GSMA TSG group to consider the above while defining the new GSMA TS.48 profile version for Rel 17 and Rel 18.

# 3	Dates of next TSG CT WG 6 meetings

CT6#120-bis	19- 22 November 2024		Orlando (US)

CT6#121	18- 21 February 2025		Athens, GR