3GPP TSG-RAN4 Meeting #112	R4-2414073

# Maastricht, Netherlands, 19th – 23rd August, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 4716 | rev | 1 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_NTN_enh-Core) CR to TS 38.133 specification corrections of for NR NTN enh |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh-Core |  |  |  |  |  | Date: |  |  | 2024-07-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Location-based measurement initiation criterion of earth-moving cell is not captured The calculation method of Navailable and Ntotal and applicability rule of kgap for intra-frequency cell identification are missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | For cell reselection, add Location-based measurement initiation criterion of earth-moving cell Re-add the calculation method of Navailable and Ntotal and applicability rule of kgap for intra-frequency cell identification |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The NTN spec will not be accurate and complete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2C.2.3, 4.2C.2.4, 9.2C.6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R4-2411756 |  |  |  |  |  |  |  |  |

# <Start of change>

#### 4.2C.2.3	Measurements of intra-frequency NR cells

The UE shall be able to identify new intra-frequency cells and perform SS-RSRP and SS-RSRQ measurements of the identified intra-frequency cells without an explicit intra-frequency neighbour list containing physical layer cell identities.

If Srxlev > SIntraSearchP and Squal > SIntraSearchQ, and the distance between UE and serving cell reference location or serving cell moving reference location is smaller than distanceThresh if the distanceThresh is configured  (see TS 38.304[1]) and UE has location information, then the UE is not required to perform measurement of intra-frequency.

The UE shall be able to evaluate whether a newly detectable intra-frequency cell meets the reselection criteria defined in TS38.304 [1] within Kmulti_SMTC * Tdetect,NR_Intra when that Treselection= 0 if the UE does not support the feature for enhanced RRM requirements defined in TS38.306 [14] or if the enhancedMeasurementLEO-r17 is not enabled, or within Kmulti_SMTC *  Tdetect,NR_Intra_enh if the UE supports the feature for enhanced RRM requirements defined in TS38.306 [14] and the enhancedMeasurementLEO-r17 is enabled. An intra frequency cell is considered to be detectable according to the conditions defined in Annex B.1.6 for a corresponding Band.

The UE shall measure SS-RSRP and SS-RSRQ at least every Kmulti_SMTC * Tmeasure,NR_Intra (see table 4.2C.2.3-1) if the UE does not support the feature for enhanced RRM requirements defined in TS38.306 [14] or if the enhancedMeasurementLEO-r17 is not enabled, or every Kmulti_SMTC * Tmeasure,NR_Intra_enh (see table 4.2C.2.3-2) if the UE supports the feature for enhanced RRM requirements defined in TS38.306 [14]  and the enhancedMeasurementLEO-r17 is enabled, for intra-frequency cells that are identified and measured according to the measurement rules.

The UE shall filter SS-RSRP and SS-RSRQ measurements of each measured intra-frequency cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Tmeasure,NR_Intra/2.

For UE in FR1-NTN:

If smtcs do not overlap with each other,

-	Kmulti_SMTC=1, if GEO satellites are measured on the carrier;

-	Kmulti_SMTC=NLEO,iNLEO,simul, if LEO satellites are measured on the carrier;

-	If smtcs partially overlap with each other,

-	Kmulti_SMTC=NSMTC,overlap, if only GEO satellites are measured on the carrier;

-	Kmulti_SMTC=i=1NSMTC,overlapNLEO,iNLEO,simul, if only LEO satellites are measured on the carrier;

Where

-	NLEO,i Is the number of LEO satellites to be measured within i-th SMTC,

-	NLEO,simul Is the number of LEO satellites that UE can measure in parallel within an SMTC,

-	NSMTC,overlap Is the number of smtcs that partially overlap with each other.

Note: for deriving Kmulti_SMTC for Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra, two SMTCs are considered as overlapping if they overlap in one or more occasions during a single Tdetect,NR_Intra, Tmeasure,NR_Intra or Tevaluate,NR_Intra.

For UE in FR2-NTN, Kmulti_SMTC = 1.

The parameter Kmulti_SMTC is the scaling factor for measurements of multiple SMTCs which correspond to different satellites.

The UE shall not consider a NR neighbour cell in cell reselection, if it is indicated as not allowed in the measurement control system information of the serving cell.

For an intra-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the intra-frequency cell has met reselection criterion defined in TS38.304 [1] within Kmulti_SMTC * Tevaluate,NR_Intra if the UE does not support the feature for enhanced RRM requirements defined in TS38.306 [14]  or if the enhancedMeasurementLEO-r17 is not enabled, or within Kmulti_SMTC * Tevaluate,NR_Intra_enh if the UE supports the feature for enhanced RRM requirements defined in TS38.306 [14]  and the enhancedMeasurementLEO-r17 is enabled, when Treselection = 0 as specified in table 4.2C.2.3-1 or table 4.2C.2.3-2 provided that:

-	when rangeToBestCell is not configured:

-	the cell is at least 3dB better ranked in FR1 or 4.5dB better ranked in FR2.

-	when rangeToBestCell is configured:

-	the cell has the highest number of beams above the threshold absThreshSS-BlocksConsolidation among all detected cells whose cell-ranking criterion R value in TS38.304 [1] is within rangeToBestCell of the cell-ranking criterion R value of the highest ranked cell.

-	if there are multiple such cells, the cell has the highest rank among them.

-	the cell is at least 3dB better ranked in FR1 or 4.5dB better ranked in FR2 if the current serving cell is among them.

When evaluating cells for reselection, the SSB side conditions apply to both serving and non-serving intra-frequency cells.

If Treselection timer has a nonzero value and the intra-frequency cell is satisfied with the reselection criteria which are defined in TS38.304 [1], the UE shall evaluate this intra-frequency cell for the Treselection time. If this cell remains satisfied with the reselection criteria within this duration, then the UE shall reselect that cell.

Table 4.2C.2.3-1: Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra

| DRX cycle length [s] | Scaling Factor (N1) | Tdetect,NR_Intra [s] (number of DRX cycles) | Tmeasure,NR_Intra [s] (number of DRX cycles) | Tevaluate,NR_Intra [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- |
|  | FR1 |  |  |  |
| 0.32 | 1 | 11.52 x N1 x M2 (36 x N1 x M2) | 1.28 x N1 x M2 (4 x N1 x M2) | 5.12 x N1 x M2 (16 x N1 x M2) |
| 0.64 |  | 17.92 x N1 (28 x N1) | 1.28 x N1 (2 x N1) | 5.12 x N1 (8 x N1) |
| 1.28 |  | 32 x N1 (25 x N1) | 1.28 x N1 (1 x N1) | 6.4 x N1 (5 x N1) |
| 2.56 |  | 58.88 x N1 (23 x N1) | 2.56 x N1 (1 x N1) | 7.68 x N1 (3 x N1) |
| Note 1: M2 = 2 if SMTC periodicity of measured intra-frequency cell > 20 ms and 1<NSMTC 4 upon more than 1 SMTC configured at the UE; M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms and NSMTC=1 upon 1 SMTC configured at the UE; otherwise M2=1. Where, NSMTC is the number of SMTCs configured by SAN If different SMTC periodicities are configured for different cells, the SMTC periodicity in this note is the one used by the cell being identified. During PSS/SSS detection, the periodicity of the SMTC configured for the intra-frequency carrier is assumed, and if the actual SSB transmission periodicity is greater than the SMTC configured for the intra-frequency carrier, longer Tdetect, NR_intra is expected. Note 2: The UE is not required to meet the requirements for 2.56s DRX cycle length for earth-moving LEO deployment. |  |  |  |  |

Table 4.2C.2.3-2: Tdetect,NR_Intra_enh, Tmeasure,NR_Intra_enh and Tevaluate,NR_Intra_enh

| DRX cycle length [s] | Tdetect,NR_Intra_enh [s] (number of DRX cycles) | Tmeasure,NR_Intra_enh [s] (number of DRX cycles) | Tevaluate,NR_Intra_enh [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 2.56 x M2 (8 x M2)Note 1 | 0.32 x M3 (1 x M3) Note 1 | 0.96 x M4 (3 x M4) Note 1 |
| 0.64 | 5.12 (8) | 0.64 (1) | 1.92 (3) |
| 1.28 | 8.96 (7) | 1.28 (1) | 3.84 (3) |
| 2.56 | 58.88 (23) | 2.56 (1) | 7.68 (3) |
| Note 1: When SMTC < = 40 ms, M2 = M3 = M4 = 1; and when SMTC > 40 ms, M2 = 2, M3 = M4 = 2.5 |  |  |  |

If ‘t-Service’ is broadcasted and applicable, UE shall be able to detect, measure, and evaluate neighbour cells before the serving cell stops serving the area regardless of whether the distance condition based on serving cell reference location or serving cell moving reference location is met or the legacy Srxlev/Squal condition are met, and when to start the detection, measurement and evaluation on neighbour cells is up to UE implementation. This requirement does not apply when the time span from the last slot of SI transmission within SI modification period where the broadcasting of the last updated value for t-Service is acquired by the UE for the first time to the first slot when the cell is scheduled to stop serving the area according to the broadcasted information is less than Ttrigger.

Ttrigger = max(Kmulti_SMTC*Tdetect,NR_Intra, Kmulti_SMTC*Kcarrier* Tdetect,NR_Inter),

where

-	Kcarrier is the number of NR inter-frequency carriers indicated by the serving cell,

-	Tdetect,NR_Intra refers to intra-frequency cell detection delay in IDLE/INACTIVE mode defined Table Table 4.2C.2.3-2,

-	Tdetect,NR_Inter refers to inter-frequency cell detection delay in IDLE/INACTIVE mode defined Table 4.2C.2.4-2.

The requirements in this clause apply provided that the number of SMTCs for any inter-frequency carrier does not exceed the parallelSMTC-r17, otherwise UE may select one or subset of all the configured SMTCs sequentially for performing the measurements until all of the SMTCs can be measured. The selection of SMTCs to be used is up to UE implementation, and in this case, measurement period longer than the corresponding measurement period specified in Table 4.2C.2.3-1 and Table 4.2C.2.3-2 is expected.

# <Next change>

#### 4.2C.2.4	Measurements of inter-frequency NR cells

The UE shall be able to identify new inter-frequency cells and perform SS-RSRP or SS-RSRQ measurements of identified inter-frequency cells if carrier frequency information is provided by the serving cell, even if no explicit neighbour list with physical layer cell identities is provided.

If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, and the distance between UE and serving cell reference location or serving cell moving reference location is smaller than distanceThresh if distanceThresh is configured and UE has location information, then the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2C.2.9.

If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, or the distance between UE and serving cell reference location or serving cell moving reference location is larger than distanceThresh if distanceThresh is configured and UE has location information, then the UE shall search for and measure inter-frequency layers of higher, equal or lower priority in preparation for possible reselection. The requirements apply provided that the distance exceeds the distanceThresh by a margin of 50 m when referenceLocation is configured by the network or [80]m when movingReferenceLocation is configured by the network. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority layers shall be the same as that defined below in this clause.

The UE shall be able to evaluate whether a newly detectable inter-frequency cell meets the reselection criteria defined in TS38.304 [1] within i=1KcarrierKmulti_SMTC,i*Tdetect,NR_Inter if the UE does not support the feature for enhanced RRM requirements defined in TS38.306 [14]  or if the enhancedMeasurementLEO-r17 is not enabled, or within i=1KcarrierKmulti_SMTC,i*Tdetect,NR_Inter_enh if the UE supports the feature for enhanced RRM requirements defined in TS38.306 [14]  and the enhancedMeasurementLEO-r17 is enabled, if at least carrier frequency information is provided for inter-frequency neighbour cells by the serving cells when Treselection = 0 provided that the reselection criteria is met by a margin of at least [5]dB in FR1 for reselections based on ranking or [6]dB in FR1 for SS-RSRP reselections based on absolute priorities or [4]dB in FR1 for SS-RSRQ reselections based on absolute priorities. The parameter Kcarrier is the number of NR inter-frequency carriers indicated by the serving cell.

The parameter Kmulti_SMTC,i is the scaling factor for measurement of multiple SMTCs or multiple satellites.

For FR2-NTN, Kmulti_SMTC,i = 1.

For UE in FR1-NTN:

-	If SMTCs do not overlap with each other,

-	KmultiSMTC,i=1, if GEO satellites are measured on the carrier;

-	KmultiSMTC,i=NLEO,iNLEO,simul, if LEO satellites are measured on the carrier;

-	If SMTCs partially overlap with each other,

-	KmultiSMTC,i=NSMTC,overlap, if only GEO satellites are measured on the carrier;

-	KmultiSMTC,i=i=1NSMTC,overlapNLEO,iNLEO,simul, if only LEO satellites are measured on the carrier;

where

NLEO,i is the number of LEO satellites to be measured within i-th SMTC,

NLEO,simul is the number of LEO satellites that UE can measure in parallel within an SMTC,NSMTC,overlap is the number of SMTCs that partially overlap with each other.

Note: 	for deriving Kmulti_SMTC,i for Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter of frequency layer i, two SMTCs are considered as overlapping if they overlap in one or more occasions during a single Tdetect,NR_Inter, Tmeasure,NR_Inter or Tevaluate,NR_Inter.

An inter-frequency cell is considered to be detectable according to the conditions defined in Annex B.1.7 for a corresponding Band.

When higher priority cells are found by the higher priority search, they shall be measured at least every Tmeasure,NR_Inter. If, after detecting a cell in a higher priority search, it is determined that reselection has not occurred then the UE is not required to continuously measure the detected cell to evaluate the ongoing possibility of reselection. However, the minimum measurement filtering requirements specified later in this clause shall still be met by the UE before it makes any determination that it may stop measuring the cell. If the UE detects on a NR carrier a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell.

The UE shall measure SS-RSRP or SS-RSRQ at least every i=1KcarrierKmulti_SMTC,i*Tmeasure,NR_Inter (see table 4.2C.2.4-1) if the UE does not support the feature for enhanced RRM requirements defined in TS38.306 [14]  or if the enhancedMeasurementLEO-r17 is not enabled, or every i=1KcarrierKmulti_SMTC,i*Tmeasure,NR_Inter_enh (see table 4.2C.2.4-2) if the UE supports the feature for enhanced RRM requirements defined in TS38.306 [14]  and the enhancedMeasurementLEO-r17 is enabled, for identified lower or equal priority inter-frequency cells. If the UE detects on a NR carrier a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell.

The UE shall filter SS-RSRP or SS-RSRQ measurements of each measured higher, lower and equal priority inter-frequency cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Tmeasure,NR_Inter/2.

The UE shall not consider a NR neighbour cell in cell reselection, if it is indicated as not allowed in the measurement control system information of the serving cell.

For an inter-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the inter-frequency cell has met reselection criterion defined TS 38.304 [1] within i=1KcarrierKmulti_SMTC,i*Tevaluate,NR_Inter if the UE does not support [capability for enhanced requirements] or if the [NW configuration for enhanced requirements] is not enabled, or within i=1KcarrierKmulti_SMTC,i*Tevaluate,NR_Inter_enh if the UE supports the feature for enhanced RRM requirements defined in TS38.306 [14]  and the enhancedMeasurementLEO-r17 is enabled, when Treselection = 0 as specified in table 4.2C.2.4-1 provided that the reselection criteria is met by

-	the condition when performing equal priority reselection and

when rangeToBestCell is not configured:

-	the cell is at least [5]dB better ranked in FR1 or.

when rangeToBestCell is configured:

-	the cell has the highest number of beams above the threshold absThreshSS-BlocksConsolidation among all detected cells whose cell-ranking criterion R value in TS38.304 [1] is within rangeToBestCell of the cell-ranking criterion R value of the highest ranked cell.

-	if there are multiple such cells, the cell has the highest rank among them

-	the cell is at least [5]dB better ranked in FR1 if the current serving cell is among them. or

-	[6]dB in FR1 for SS-RSRP reselections based on absolute priorities or

-	4]dB in FR1 for SS-RSRQ reselections based on absolute priorities.

When evaluating cells for reselection, the SSB side conditions apply to both serving and inter-frequency cells.

If Treselection timer has a non zero value and the inter-frequency cell is satisfied with the reselection criteria, the UE shall evaluate this inter-frequency cell for the Treselection time. If this cell remains satisfied with the reselection criteria within this duration, then the UE shall reselect that cell.

The UE is not expected to meet the measurement requirements for an inter-frequency carrier under DRX cycle=320 ms defined in Table 4.2C.2.4-1 under the following conditions:

-	TSMTC_intra = TSMTC_inter = 160 ms; where

-	TSMTC_intra is the periodicity of the SMTC configured for the intra-frequency carrier if no identified intra-frequency cell is in the PCI list of smtc2-LP on this intra-frequency carrier; TSMTC_intra is the periodicity of the smtc2-LP configured for the intra-frequency carrier if at least one identified intra-frequency cell is in the PCI list of smtc2-LP on this intra-frequency carrier. During PSS/SSS detection, the periodicity of the SMTC configured for the intra-frequency carrier is assumed for TSMTC_intra. If the actual SSB transmission periodicity is greater than the SMTC configured for the intra-frequency carrier, longer Tdetect, NR_intra is expected.

-	TSMTC_inter is the actual SMTC periodicity used by the inter-frequency cell being identified. During PSS/SSS detection, the periodicity of the SMTC configured for the inter-frequency carrier is assumed for TSMTC_inter. If the actual SSB transmission periodicity is greater than the SMTC configured for the inter-frequency carrier, longer Tdetect, NR_inter is expected.

-	SMTC occasions configured for the inter-frequency carrier occur up to 1 ms before the start or up to 1 ms after the end of the SMTC occasions configured for the intra-frequency carrier, and

-	SMTC occasions configured for the intra-frequency carrier and for the inter-frequency carrier occur up to 1 ms before the start or up to 1 ms after the end of the paging occasion in TS38.304 [1].

Table 4.2C.2.4-1: Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter

| DRX cycle length [s] | Scaling Factor (N1) | Tdetect,NR_Inter [s] (number of DRX cycles) | Tmeasure,NR_Inter [s] (number of DRX cycles) | Tevaluate,NR_Inter [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- |
|  | FR1 |  |  |  |
| 0.32 | 1 | 11.52 x N1 x 1.5 (36 x N1 x 1.5) | 1.28 x N1 x 1.5 (4 x N1 x 1.5) | 5.12 x N1 x 1.5 (16 x N1 x 1.5) |
| 0.64 |  | 17.92x N1 (28 x N1) | 1.28 x N1 (2 x N1) | 5.12 x N1 (8 x N1) |
| 1.28 |  | 32 x N1 (25 x N1) | 1.28 x N1 (1 x N1) | 6.4 x N1 (5 x N1) |
| 2.56 |  | 58.88 x N1 (23 x N1) | 2.56 x N1 (1 x N1) | 7.68 x N1 (3 x N1) |
| Note 1: UE is not required to fulfil the requirements for 2.56s DRX cycle length for earth-moving LEO deployment. |  |  |  |  |

Table 4.2C.2.4-2: Tdetect,NR_Inter_enh, Tmeasure,NR_Inter_enh and Tevaluate,NR_Inter_enh

| DRX cycle length [s] | Tdetect,NR_Inter_enh [s] (number of DRX cycles) | Tmeasure,NR_Inter_enh [s] (number of DRX cycles) | Tevaluate,NR_Inter_enh [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | [3.2 x M2 (10 x M2)] Note 1 | [0.32 x M3 ([1] x M3)] Note 1 | 0.96 x M4 (3 x M4) Note 1 |
| 0.64 | [6.4 (10)] | [0.64 (1)] | 1.92 (3) |
| 1.28 | [10.24 (8)] | 1.28 (1) | 3.84 (3) |
| 2.56 | 58.88 (23) | 2.56 (1) | 7.68 (3) |
| Note 1: When SMTC < = 40 ms, M2 = M3 = M4 = 1; and when SMTC > 40 ms, M2 = 1.5, M3 = M4 = 2 |  |  |  |

If t-Service is broadcasted and applicable, UE shall be able to detect, measure, and evaluate neighbour cells before the serving cell stops serving the area regardless of whether the distance condition based on serving cell reference location or serving cell moving reference location or the legacy Srxlev/Squal condition are met, and when to start detection, measurement, and evaluation is up to UE implementation. This requirement does not apply when the time span from the last slot of SI transmission within SI modification period where the broadcasting of the last updated value for t-Service is acquired by the UE for the first time to the first slot when the cell is scheduled to stop serving the area according to the broadcasted information is less than Ttrigger, and Ttrigger = max(Kmulti_SMTC*Tdetect,NR_Intra, Kmulti_SMTC*Kcarrier* Tdetect,NR_Inter) when serving cell is below the search threshold, and Ttrigger = max(Kmulti_SMTC*Tdetect,NR_Intra, Nlayer* [60s]) when serving cell is above the search threshold, where

-	Kcarrier is the number of NR inter-frequency carriers indicated by the serving cell,

-	Nlayer is the total number of higher priority NR carrier frequencies broadcasted in system information,

-	Tdetect,NR_Intra refers to HST intra-frequency cell detection delay in IDLE/INACTIVE mode defined Table 4.2.2.3-2,

-	Tdetect,NR_Inter refers to HST inter-frequency cell detection delay in IDLE/INACTIVE mode defined Table 4.2.2.4-2.

The requirements in this clause apply provided that the number of SMTCs for any inter-frequency carrier does not exceed the [UE capability], otherwise UE may select one or subset of all the configured SMTCs sequentially until all of the SMTCs can be measured, the selection of SMTCs to be used is up to UE implementation, and longer measurement delay than the corresponding measurement period specified in Table 4.2C.2.4-1 and Table 4.2C.2.4-2 is expected.

The requirements in this clause apply provided that the valid information for the satellite serving the target cell has been provided by the serving cell.

The requirements in this clause apply provided that SSB of neighbour cells are within the time shifted SMTC.

# <Next change>

#### 9.2C.6.1	Intra-frequency cell identification

The UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index (reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured), or the UE has been indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). Otherwise UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_with_index. The UE shall be able to identify a new detectable intra frequency SS block of an already detected cell within Tidentify_intra_without_index.

Tidentify_intra_without_index = TPSS/SSS_sync_intra + T SSB_measurement_period_intra  ms

Tidentify_intra_with_index = TPSS/SSS_sync_ntra + T SSB_measurement_period_intra + TSSB_time_index_intra ms

Where:

TPSS/SSS_sync_intra: it is the time period used in PSS/SSS detection given in table 9.2C.6.2-1.

TSSB_time_index_intra: it is the time period used to acquire the index of the SSB being measured given in table 9.2C.6.2-2.

T SSB_measurement_period_intra: equal to a measurement period of SSB based measurement given in table 9.2C.6.3-1.

Kgap is the scaling factor for a SSB frequency layer to be measured within an associated measurement gap pattern. Kgap = 1 when the UE is not configured with concurrent measurement gaps. When the UE is configured with concurrent measurement gaps and the two measurement gaps are fully overlapping with MGRP=160ms, Kgap = 2. Otherwise, Kgap = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows:

For a window W of duration max(SMTC period,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gap, and starting from the beginning of any SMTC occasion:

-	Ntotal is the total number of SMTC occasions that are covered by instances of the associated measurement gap within the window W, including those overlapped with other measurement gap occasions within the window, and

-	Navailable is the number of SMTC occasions that are covered by instances of the non-dropped associated measurement gap within the window W after accounting for measurement gap collisions by applying the measurement gap collision rule in section 9.1.8.3.

Kgap is only applicable for UE supporting parallelMeasurementGap-r17. When concurrent measurement gaps are configured, requirements in this clause do not apply if Navailable =0, or if one SMTC overlaps more than one MGs associated to the frequency layer.

CSSFintra: it is a carrier specific scaling factor and is determined according to CSSFwithin_gap,i in clause 9.1.5.2 for measurement conducted within measurement gaps.

<unchanged parts skipped>

# <End of change>