3GPP TSG RAN Meeting #106	         RP-242458

Madrid, Spain, December 9-12, 2024

Agenda Item:	9.1.1

Source:	Lenovo

Title:	Ambient IoT scope discussion

Document for:	Discussion

# 1	Introduction

RAN1#119 and RAN2#128 completed the Rel-19 SID on Ambient IoT solutions and the recommendations are captured in 3GPP TR38.769. This contribution discusses the potential scope, way forward and guidance for the work item.

# 2	Discussion

The RAN1#119 and RAN2#128 completed the Rel-19 study item on the Ambient IoT solutions and this contribution provides the potential scope, way forward and guidance for the work item by considering the status of the Ambient IoT solutions progress in SA2 and SA3 working groups.

## 2.1 Design requirements for device type 1, 2a and 2b

The Ambient IoT device type 1 having a target power consumption of ~1 µW should have a better design to achieve more coverage compared to UHF RFID. The design to achieve more coverage for Ambient IoT device type 1 should go beyond the implementation aspects such as higher transmit power, lower tag return loss and backscatter loss. Moreover, the sampling clock accuracy after calibration for device type 1 which was heavily discussed during RAN1#119 should be discussed during the scoping phase and captured as one of the design requirement for device type 1 as many of the design features starting from X-ambles i.e., preamble, midamble, postamble, D2R small frequency shift for device type 1, multiple access i.e., TDMA, FDMA, CDMA for Msg1 etc., depend on the accuracy. Since the target is to have better design feature for device type 1 compared to UHF RFID and there are many references showing that the implementation [2] of 104 PPM clock accuracy after calibration is achievable using clock counting or using the external carrier wave, the design of Ambient IoT type 1 should design features according to 104 clock accuracy after calibration.

Proposal 1: 3GPP Ambient IoT device type 1 normative work should target better design feature compared to UHF RFID to achieve better coverage:

- The design requirement should take into account, the maximum achievable clock accuracy which is primarily the target to design X-amble, D2R multiple access etc.,

- Maximum achievable clock accuracy after clock calibration for Ambient IoT device type 1 should be kept as 104 PPM for design target during normative phase.

The Ambient IoT study item completed the study of harmonized air interface design to support different device types but also states to study differences. Some companies think that the harmonized design should develop an unified solution for all device types without considering the constraints and capabilities of various device types. In the normative phase, the unified solution should go beyond the optimal solution for device type 1 and consider the operation and performance of the device type 2b. The harmonized design in the normative phase should therefore be interpreted as baseline design for Ambient IoT devices and some features can be removed or added depending on the device types.

Proposal 2: Consider developing a baseline feature set that works for all Ambient IoT devices under harmonized design while adding necessary features to achieve optimal performance for device type 2b such as

- Frequency synchronization

- Power saving feature for device 2b i.e, power states, duty cycle based monitoring, multi-access slot etc.,

The following table compares some example design features for device type 1 and device type 2b.

| D2R Features | Device type 1 | Device type 2b | Reason for differentiated design for device type 2b |
| --- | --- | --- | --- |
| Waveform | ON-OFF keying (OOK), BPSK | MSK | MSK provides continuous signal benefitting the active device type. MSK can be optional to be supported by device type 1. |
| Device power states | ON, OFF | ON, Sleep/memory retention state, OFF | Saves discharging for device type 2b |
| Duty cycle-based operation / device monitoring configuration | No | Yes | Device type 2b capable entering sleep/memory retention states |
| Wake up signal/trigger | No | Yes | Device type 2b wake up from sleep/memory retention state |
| Slot based access - TDMA | Single access slot i.e., reader sends R2D command with slot counter in every slot | Multi-access slot i.e., reader sends R2D command for N associated slots | Prevents device type 2b from monitoring every slot for R2D command. |
| D2R multiple access – CDMA support for Msg1 | TDMA, FDMA | TDMA, FDMA, CDMA | At least random access resources can be CDMed enabling reader to detect multiple devices to achieve faster access and less discharging time |

Table 1: Example D2R features for device type 1 and device type 2b

The AS level segmentation is needed to be supported by both device types because maximum TB size transmission depends on the coverage/data rate, energy consumption/device availability etc., Also, since the Ambient IoT does not transmit any measurement result to the reader, the reader may or may not be aware of the channel condition, energy status at the device side for higher TB size transmission. Hence, AS level segmentation is necessary to be supported by RAN2 for all device types.

Proposal 3: Consider supporting AS level segmentation for all device types.

The carrier wave waveforms (waveform 1 and waveform 2) and carrier wave node signaling should be provided by the network in the licensed frequency due to switching off the transmission of carrier wave waveform when inventory procedures are not happening and control interference, power etc.,. The signaling and the interface between the carrier wave node and the network should be studied during the normative phase.

Proposal 4: Consider both carrier wave waveform 1 and waveform 2 during normative phase and mechansim to control the carrier wave node by the network should be specified.

RAN1 studied on the RF energy harvesting and considered two directions.

The following directions, not for down-selection, are studied regarding the potential impact of device unavailability due to energy harvesting: Direction 1: Reader does not provide information to a device regarding when the device may become available/unavailable. Direction 2: Reader can provide information to a device based on which the device may become available/unavailable.

The detailed solutions under these directions were not discussed during the study item phase which leaves several solutions under these directions. It becomes difficult to down select a particular direction at this stage without proper evaluation and power state assumptions for these device types. The number of power states supported by a device type i.e., ON, OFF or ON, OFF, Sleep/memory retention has an influence on certain solutions and unfortunately RAN1 did not agree on the number of power states supported by these device types.

Proposal 5:  Study on supporting RF energy harvesting based on direction 1, direction 2 and applicability of it to different device types & power states.

## 2.2 Discussion on Ambient IoT scope

The Ambient IoT scope discussion should take into consideration the status of the work in SA2 and SA3 working groups. The SA3 working group discusses on the security solution for the Ambient IoT in the last two meetings and the solutions are captured in the TR 33.761 v 0.5.0 are not stable yet. Hence. the study item needs to be extended to finish evaluation of the solutions beyond SA#106. Even if the SA3 extends the study item to complete evaluation of the solutions, the normative phase for Rel-19 becomes difficult. SA2 completed the study item on the Ambient IoT solutions for the DT and DO-DTT use cases but the SA2 Rel19 stage 2 freeze is SA#106, which means any normative work in SA2 for Rel19 Ambient IoT solutions needs to start with an exception request from the beginning of the work item approval process.

Observation 1: SA3 needs to extend the study item to finish evaluating the solutions for Ambient IoT beyond SA#106. While, SA2 Rel19 stage 2 freeze is in SA#106.

The Rel19 functional freeze for RAN1 is RAN#108 which is June 2025 while the RAN2 ASN.1 freeze is in RAN#109 which is September 2025. RAN1 cannot complete the entire design for Ambient IoT solutions described in TR 38.769 (also provided below) within Rel19 and requires at least two releases to finish the work item or entire Rel20 to finish the work item. RAN1 has only 3 meetings to finish normative work for Ambient IoT within Rel19 (if agreed) which is challenging even for the narrow work item scope considering the amount of details needed to finish the needed design even for one device type. Hence, RAN1 requires an exception request to finish the needed design beyond RAN#108.

Below we present three different options to continue with the normative phase of Ambient IoT solutions:

Option-1: Considering that exceptions are needed to do any potential normative phase work in SA2 and SA3 and it will be challenging even for RAN working groups to finish a potential work item containing essential design details even for one device type by Rel19 functional freeze i.e. RAN#108, postpone the normative phase work to Rel20.

Option-2: The normative phase work of Ambient IoT at RAN1 for Rel19 can be extended by a quarter beyond Rel19 timeline i.e., until RAN#109, to finish the essential design details for one device type. The scope of the work item should be narrowly designed to finish the essential design details for one device type. In this way, the normative phase work for the remaining device types in Rel20 is not much affected for RAN1 and RAN2.

Option 3: The normative phase of Ambient IoT solutions goes beyond the Rel19 timeline by many quarters e.g., Q2 2026 by agreeing to a larger scope e.g., device type 1 and device type 2b. Any additional new use case such as outdoor use case should be appropriately planned for Rel20.

Proposal 6: Option-1 is to continue the RAN1 and RAN2 study until the remainder of Rel19 to finish the remaining details and postpone the normative phase work to Rel20.

Proposal 7: Option 2 is to finish the normative phase work of Ambient IoT solutions in RAN1 and RAN2 by RAN#109 using a narrower scope with one device type and normative phase work for the remaining device types in Rel20.

Proposal 8: Option 3 is that the normative phase work of Ambient IoT solution goes many quarters into Rel20 by agreeing to a larger scope and appropriately plan any additional new use case for Rel20, if time permits.

Considering that the Ambient IoT study for Rel19 studied only two use cases out of four SA1 / RAN use cases and there is considerable interest to work on outdoor deployments, 3GPP RAN and SA should work together to provide a long-term timeline for Ambient IoT evolution within 5G. Such endorsed Ambient IoT evolution timeline considering evolution of different device types and deployments i.e., indoor, and outdoor may significantly decrease the skepticism among companies and ease the pressure to agree on all design details for all device types and deployment scenarios within Rel19.

Proposal 9: TSG RAN and SA work on endorsing timeline for Ambient IoT evolution in 3GPP considering device types and deployments i.e. indoor and outdoor for use cases specified in SA1 and RAN.

# 3	Conclusion

This contribution discusses Ambient IoT scope for Rel19.

Proposal 1: 3GPP Ambient IoT device type 1 normative work should target better design feature compared to UHF RFID to achieve better coverage:

- The design requirement should take into account, the maximum achievable clock accuracy which is primarily the target to design X-amble, D2R multiple access etc.,

- Maximum achievable clock accuracy after clock calibration for Ambient IoT device type 1 should be kept as 104 PPM for design target during normative phase.

Proposal 2: Consider developing a baseline feature set that works for all Ambient IoT devices under harmonized design while adding necessary features to achieve optimal performance for device type 2b such as

- Frequency synchronization

- Power saving feature for device 2b i.e, power states, duty cycle based monitoring, multi-access slot etc.,

Proposal 3: Consider supporting AS level segmentation for all device types.

Proposal 4: Consider both carrier wave waveform 1 and waveform 2 during normative phase and mechansim to control the carrier wave node by the network should be specified.

Proposal 5:  Study on supporting RF energy harvesting based on direction 1, direction 2 and applicability of it to different device types & power states.

Proposal 6: Option-1 is to continue the RAN1 and RAN2 study until the remainder of Rel19 to finish the remaining details and postpone the normative phase work to Rel20.

Proposal 7: Option 2 is to finish the normative phase work of Ambient IoT solutions in RAN1 and RAN2 by RAN#109 using a narrower scope with one device type and normative phase work for the remaining device types in Rel20, if time permits.

Proposal 8: Option 3 is that the normative phase work of Ambient IoT solution goes many quarters into Rel20 by agreeing to a larger scope and appropriately plan any additional new use case for Rel20.

Proposal 9: TSG RAN and SA work on endorsing timeline for Ambient IoT evolution in 3GPP considering device types and deployments i.e. indoor and outdoor for use cases specified in SA1 and RAN.

# References

TR 38.848 V18.0.0, “Study on Ambient IoT (Internet of Things) in RAN”, September 2023.

Chairman notes - Draft Report of 3GPP TSG RAN WG1 #119 v0.2.0

# Annexure

The RAN1#119 revised the coverage target for the device types for different deployment scenario and topology. The evaluation results for the coverage are captured in TR 38.769.

For Rel-19 Ambient IoT, the target value(s) for applicable maximum distance are refined as follows, - For device 1 in D1T1: - 15m - For device 2a in D1T1: - 25m - For device 2b in D1T1: - 50m - For device 1 in D2T2: - 10m - For device 2a in D2T2: - 15m - For device 2b in D2T2: - 40m

RAN1 recommends that in a normative phase, the design should start from the descriptions in this TR. (1) The following techniques are recommended in the first normative phase: - PRDCH and PDRCH, as respectively the single physical channels for R2D and D2R - Modulations of OOK-1 and OOK-4 in R2D - Multiplexing in time domain for R2D - Convolutional FEC in D2R and no FEC in R2D - R2D and D2R signal(s) (2) At least the following aspects may be considered for inclusion or non-inclusion in a given first or subsequent Release: - Device 1 and/or Device 2a (with or without large frequency shift) and/or Device 2b (with RF-ED and/or IF-ED and/or ZIF receiver architecture) - D1T1 and/or D2T2 - Scenario A1 and/or A2 and/or B and/or C - Line code(s) Manchester/PIE for R2D, and Manchester / Miller / no line code for D2R - Multiple access by TDMA and/or FDMA and/or CDMA for D2R; FDMA for device 2b (IF-ED/ZIF receiver) in R2D - OOK and/or BPSK and/or MSK modulation for D2R - Carrier-wave waveform 1 and/or 2, and transmission case(s) - Device (un)availability direction 1 and/or 2 - Proximity determination solution 1 and/or 2