3GPP TSG RAN Meeting #106	  RP-242476

Madrid, Spain, December 9-12, 2024

Agenda item:	9.2.2 Study on solutions for Ambient IoT (Internet of Things) in NR

Source:	Nokia

Title:	Rel-19 study on Ambient IoT – way forward

Document for:	Discussion

# 1	Introduction

In RAN#103 the Study on solutions for Ambient IoT (Internet of Things) in NR [1] was approved.

In this paper, we propose a way forward.

# 2	Discussion

## 2.1	Motivation & proposal

With reference to [1] the general scope of Ambient IoT is:

General Scope:

The definitions provided in TR 38.848 are taken into this SI, and the following are the exclusive general scope:

- The overall objective shall be to study a harmonized air interface design with minimized differences (where necessary) for Ambient IoT to enable the following devices:

- ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither DL nor UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally.

- ≤ a few hundred µW peak power consumption1, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both DL and/or UL amplification in the device. The device’s UL transmission may be generated internally by the device, or be backscattered on a carrier wave provided externally.

- X  is to be decided in WGs.

- Coverage design target: Maximum distance of 10-50 m with device indoors as per TR 38.848: “…a range that WGs can sub-select within”.

- For Topologies 1 & 2 (UE as intermediate node under NW control) per TR 38.848, with no RRC states, no mobility (i.e. at least no cell selection/re-selection -like function), no HARQ, no ARQ.

NOTE 1: It is to be understood that “≤ a few hundred µW” means WGs are not tasked with setting a particular value, and that it will be for WG discussions to determine if a presented design with corresponding power consumption satisfies the “≤ a few hundred µW” requirement.

- Deployment Scenarios with the following characteristics, referenced to the tables in Clause 4.2.2 of TR 38.848:

- Deployment scenario 1 with Topology 1

  - Basestation and coexistence characteristics: Micro-cell, co-site

- Deployment scenario 2 with Topology 2 and UE as intermediate node, under network control

  - Basestation and coexistence characteristics: Macro-cell, co-site

  - The location of intermediate node is indoor

- FR1 licensed spectrum in FDD.

- Spectrum deployment in-band to NR, in guard-band to LTE/NR, in standalone band(s).

- Traffic types DO-DTT, DT, with focus on rUC1 (indoor inventory) and rUC4 (indoor command).

- From RAN#104, the study will assess whether the harmonized air interface design (per bullet ‘A’ above) can address the DO-A (Device-originated autonomous) use case, only to identify which part(s) of the harmonized air interface design (per bullet ‘A’ above) is/are not sufficient for the DO-A use case.

## 2.2	Status on Ambient IoT Study in SA & RAN

The Ambient IoT study [1] was concluded in the 2024 November RAN working groups. The status of both SA & RAN working groups is important to ensure a holistic end-to-end concept for Ambient IoT.

Status in RAN WG:

The RAN WG has completed the TR [3]. There are still pending items not addressed in the study including:

- CW2D control not included in scope of the SI: CW ON while R2D is ON will block R2D reception.

- R2D device selectivity schemes: Ambient IoT devices with no steep channel filtering (e.g. RF-ED RX) they may exhibit frequent false wake ups on any RF signal within a very large frequency range.

- Ambient IoT sensitivity/coverage analysis: Analysis of impact of different Ambient IoT device sensitivity for RF harvesting, device wake-up and R2D reception is incomplete. Relates to supported/assumed device states: Devices always ON or ON/ SLEEP/ OFF.

- Ambient IoT R2D, CW and D2R reader selection and re-selection procedure /assistance: The proximity detection definitions: Only D2R reader-based detection in TR 38.769 will not identify if R2D, CW2D or D2R is the limiting link so of little use for re-selection.

Furthermore, a close inspection of TR38.769 (although not officially endorsed by RAN) reveals that core features to support Ambient IoT communications have been identified and described, i.e., PRDCH and PDRCH. However, only options and directions to design them in a possible WI phase or further study phase are included in the description. No detailed design principle is present. Our understanding in this regard is that heterogeneous technical preferences exist among companies participating in the Ambient IoT discussion, hence rather long discussions on many aspects can be expected, in the case of WI phase. With only 6 months left until Rel. 19 completion, it is fair to say that this does not necessarily warrant a WI phase which could play out as expected. Extending the RAN study on Ambient IoT throughout Rel. 19 timeframe and target a WID for Rel. 20 is certainly a more sensible course of action in this context.

Status in SA2:

SA2 has been running a study for Ambient IoT during the whole of 2024. At end of SA2#166, the SA2 study on Ambient IoT is 90% complete. It is expected that one additional quarter is needed to complete this study, with discussions in January ad hoc but mainly February F2F meeting, TR was updated in [2]. In the SA2 TR conclusions (23.700-13), there are still around 30 ENs remaining unresolved, indicating that the study is still quite far from completion.

A work item proposal was discussed in SA2, without agreement.

SA2 is normally freezing release 19 stage 2 this quarter. It is unlikely that SA2 will proceed with normative work in release 19. In this context it does not make sense to have RAN do normative work in release 19 without stage 2 specifications done or ready.

Status in SA3:

SA3 has been studying Ambient IoT since the approval of the study item at SA#103 (March 2024). The dependencies to SA2 relates to the architecture for topology 1 and 2 wherein the security procedures shall be applied.

SA3 is not at the conclusion phase and depends on other groups which are also behind schedule, therefore pulling normative security work into release 19 with the given schedule, will lead to security vulnerabilities for Ambient IoT or the ned to allow deployment without security introducing vulnerabilities in the operated networks.

Proposal:

Considering the status in SA and RAN WGs, it is proposed:

- Extend the RAN study on Ambient IoT throughout Rel. 19 timeframe and target a WID for Rel. 20.

- Inform SA about such decision to allow for a holistic treatment of the feature, i.e. study in Rel-19 and normative work in Rel-20

# 3	Conclusions

Proposal:

Considering the status in SA and RAN WGs, it is proposed:

- Extend the RAN study on Ambient IoT throughout Rel. 19 timeframe and target a WID for Rel. 20.

- Inform SA about such decision to allow for a holistic treatment of the feature, i.e. study in Rel. 19 and normative work in Rel. 20

# References

RP-234058: New SID: Study on solutions for Ambient IoT (Internet of Things) in NR

3GPP TR 23.700-13 Study on Architecture support of Ambient power-enabled Internet of Things (Release 19)

3GPP TR 38.769 Study on solutions for ambient IoT (Internet of Things) (Release 19)